| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0580 |
| --- | --- |

| Title: | AHG10: Crosscheck of TI's Configurable and CU-group level parallel merge/skip (JCTVC-H0082 combination case) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, JAPAN | Tel: Email: | +81 43 299 8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution provides cross-check results of combination case in “AHG10: Configurable and CU-group level parallel merge/skip” (JCTVC-H0082). JCTVC-H0082 and the combination case with JCTVC-H0092 method1 and JCTVC-H0240 enable the usage of merge mode at parallel motion estimation (PME) case. When 16x16 PME case, it is reported that average BD-rate loss of the combination case is 0.4 % to 1.1 %, while that of encoder only restriction, which is provided from proponent, is 1.9 % to 4.2 %. When 32x32 PME case, it is reported average BD-rate loss of combination case is 1.2 % to 2.8 %, while that of encoder only restriction, which is provided from proponent, is 4.4 % to 8.4%.

The coding results perfectly match with those provided by proponents and the source code has been checked to verify its consistency with the proposal description.

# Validation of the combination case

JCTVC-H0084 [1] addressed the parallel motion estimation issue that if motion estimations of different level of PUs are processed parallel, merge/skip modes cannot be used because merge mode candidates cannot be derived at the internal PUs.

The combination case of JCTVC-H0082 with JCTVC-H0092 [2] and JCTVC-H0240 [3] is summarized as follow.

- introduce log2_parallel_merge_level_minus2 in PPS to define MER (motion estimation region) size

  - 0: 4x4 PME

  - 1: 8x8 PME

  - 2: 16x16 PME  // tested in this cross-check

  - 3: 32x32 PME  // tested in this cross-check

  - 4: 64x64 PME

- All spatial candidates is independent in MER

  - If a neighboring PU and the current PU belong to the same MER, the neighboring PU is not referred (unavailable)

- All temporal candidates is independent in MER

  - If a neighboring PU and the current PU belong to the same MER, the neighboring PU is not referred (unavailable) for the temporal MVP reference index derivation process. (original JCTVC-H0082 method)

  - Fixing the temporal MVP reference index to zero to remove PU dependency for TMVP refIdx derivation (JCTVC-H0092 method1).

- Limiting all the PUs inside an 8x8 CU to have a same merge/skip MVP list to reduce the number of MCL to be derived (JCTVC-H0240). This constraint is enabled when  log2_parallel_merge_level_minus2 > 0.

With the method, at each internal PU in MER, the merge candidate process is processed only using outside PU of the MER, in other words, all merge candidate of PUs in a MER can be motion estimated simultaneously.

# Experimental result

Table 1: Summary of PME on combination with JCTVC-H0092 method1 and JCTVC-H0240

|  |  | RA-HE | LB-HE | RA-HE10 | RA-LC | LB-LC |
| --- | --- | --- | --- | --- | --- | --- |
| 16x16 PME | proposed (tested by cross-checker) | 0.6 % | 0.7 % | 0.4 % | 0.9 % | 1.1 % |
|  | HM5 (provided by proponent) | 2.5 % | 3.1 % | 1.9 % | 3.1 % | 4.2 % |
| 32x32 PME | proposed (tested by cross-checker) | 1.6 % | 2.0 % | 1.2 % | 2.2 % | 2.8 % |
|  | HM5 (provided by proponent) | 5.3 % | 6.6 % | 4.4 % | 6.3 % | 8.4 % |

Table 2: Simulation results of 16x16 PME on combination with JCTVC-H0092 method1 and JCTVC-H0240

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.5% | 0.5% | 0.4% | 0.8% | 0.9% | 0.9% | 0.4% | 0.4% | 0.5% |
| Class B | 0.4% | 0.1% | 0.1% | 0.6% | 0.6% | 0.5% | 0.4% | 0.1% | 0.2% |
| Class C | 0.6% | 0.8% | 1.0% | 1.0% | 1.2% | 1.2% |  |  |  |
| Class D | 0.7% | 0.7% | 0.8% | 1.1% | 1.3% | 1.2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.6% | 0.5% | 0.6% | 0.9% | 0.9% | 1.0% | 0.4% | 0.2% | 0.3% |
|  | 0.6% | 0.5% | 0.6% | 0.9% | 0.9% | 0.9% | 0.4% | 0.2% | 0.3% |
| Class F | 0.4% | 0.4% | 0.5% | 0.7% | 0.8% | 0.8% |  |  |  |
| Enc Time[%] | 100% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.5% | 0.4% | 0.6% | 0.8% | 0.5% | 1.1% |  |  |  |
| Class C | 0.9% | 0.6% | 0.9% | 1.3% | 1.2% | 1.4% |  |  |  |
| Class D | 1.1% | 0.9% | 1.4% | 1.5% | 2.6% | 2.1% |  |  |  |
| Class E | 0.4% | 0.4% | 0.5% | 0.7% | 0.8% | 0.9% |  |  |  |
| Overall | 0.7% | 0.6% | 0.9% | 1.1% | 1.3% | 1.4% |  |  |  |
|  | 0.7% | 0.6% | 0.8% | 1.1% | 1.3% | 1.4% |  |  |  |
| Class F | 0.5% | 0.6% | 0.4% | 0.9% | 0.1% | 0.4% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |  |  |  |

Table 2: Simulation results of 32x32 PME on combination with JCTVC-H0092 method1 and JCTVC-H0240

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 1.5% | 1.4% | 1.5% | 2.0% | 2.1% | 2.2% | 0.9% | 1.0% | 1.0% |
| Class B | 1.5% | 1.0% | 0.9% | 2.1% | 1.9% | 1.8% | 1.5% | 1.0% | 1.0% |
| Class C | 1.8% | 2.1% | 2.2% | 2.4% | 2.7% | 2.9% |  |  |  |
| Class D | 1.7% | 1.6% | 1.9% | 2.1% | 2.4% | 2.5% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 1.6% | 1.5% | 1.6% | 2.2% | 2.3% | 2.3% | 1.2% | 1.0% | 1.0% |
|  | 1.6% | 1.5% | 1.6% | 2.1% | 2.3% | 2.3% | 1.2% | 1.0% | 1.0% |
| Class F | 1.0% | 1.1% | 1.2% | 1.4% | 1.7% | 1.7% |  |  |  |
| Enc Time[%] | 100% |  |  | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 1.9% | 1.2% | 1.2% | 2.9% | 2.0% | 2.1% |  |  |  |
| Class C | 2.2% | 1.9% | 2.2% | 3.0% | 3.1% | 3.2% |  |  |  |
| Class D | 2.4% | 2.1% | 2.7% | 3.1% | 4.6% | 4.0% |  |  |  |
| Class E | 1.2% | 1.1% | 0.9% | 2.2% | 1.5% | 2.4% |  |  |  |
| Overall | 2.0% | 1.6% | 1.8% | 2.8% | 2.8% | 2.9% |  |  |  |
|  | 2.0% | 1.6% | 1.7% | 2.8% | 2.8% | 2.9% |  |  |  |
| Class F | 1.6% | 1.7% | 1.5% | 2.1% | 1.4% | 2.0% |  |  |  |
| Enc Time[%] | 101% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |  |  |  |

# Conclusion

The coding results and software on JCTVC-H0082 with JCTVC-H0092 method1 and JCTVC-H0240 was confirmed. The method enables the usage of merge modes when motion estimations of different level of PUs are processed parallel, which improves coding performance in parallel motion estimation case.

# References

[1] M. Zhou, “AHG10: Configurable and CU-group level parallel merge/skip” , JCTVC-H0082, JCT-VC 8th Meeting, San Jose, February 2012

[2] Y. Jeon, B. Jeon, V. Seregin, X Wang, J. Chen, M. Karczewicz, “Non-CE9: Removing PU dependency in TMVP reference index derivation, “JCTVC-H0092, JCT-VC 8th Meeting, San Jose, February 2012

[3] H. Y. Kim, K. Y. Kim, S. M. Kim, G. H. Park (KHU), S.-C. Lim, J. Lee, S. Cho, J. S. Choi, “Non-CE9: Throughput improvement for merge/skip mode,” JCTVC-H0240, JCT-VC 8th Meeting, San Jose, February 2012