| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0584 |
| --- | --- |

| Title: | CE5: Cross-verification of subtest CE5.c of BBC (JCTVC-H0064) by Nokia |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jewon Kang Antti Hallapuro Jani Lainema | Tel: Email: | +358 50 4860857 jewonkan@usc.edu antti.hallapuro@nokia.com Jani.Lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

CE 5.c by BBC addresses the mode dependent significance scans for different transform skip modes (TSM), which are implemented on the top of CE 5.a [1]. This report verifies the cross-check results. The experimental results are the same as one provided by the proponent. The proposed scheme provides the overall BD-rate savings about -0.4%, -0.6%, -0.9%, -1.5%, -0.1%, respectively in RA HE, LB HE, RA LC, LB LC, and RA HE10 over HM 5.

# Introduction

This contribution is to report the results of JCTVC-H0064 by BBC. In JCTVC-H0064, when one transform is skipped on a row direction or a vertical direction, a vertical or horizontal scans are respectively used for a transform coefficient coding. The contribution is to evaluate the coding efficiency achieved by the additional scanning patterns as compared to the HM5 entropy coding design which uses a diagonal scanning pattern.

# Software Implementation

There was no problem in compiling or running the software. In implementation, the horizontal and vertical scanning patterns are applied for a 4x4 and 8x8 TU sizes, but the same diagonal scanning is used for 16x16 and 32x32 TU sizes.

# Experimental results

The experimental conditions are the same as ones used in CE5.a [1]. We confirm that the results about BD-rate savings are the same with what proponent reported in the contribution. As to the decoding time, the numbers are justified within a reliable level, but the encoding time is not reported in this document due to the hybrid conditions in servers.

Table 1: Summary results of JCTVC-H0064 as compared to HM 5.

# Reference

- M. Mrak , J. Sole , J. Xu , and A. Saxena , “Description of core experiment 5 (CE5): Transform Skipping”, JCTVC-G1205, Geneva, Nov 2011.