| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0586 M23471 |
| --- | --- |

| Title: | Crosscheck of Cisco proposal JTCVC-H0066 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gordon CLARE (Orange Labs), Félix HENRY (Orange Labs) | Tel: Email: | +33 2 99 12 45 58 gordon.clare@orange-ftgroup.com felix.henry@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck of a substest of contribution JCTVC-H0066.  JCTVC-H0066 is a proposal by Cisco aiming at providing ALF syntax at the LCU level A specific subtest within this proposal consists in adding a statistics-reset per LCU line in order to make it compatible with wavefront parallel processing. Verified coding efficiency results are strictly identical to the ones measured by the proponents [+0.5%(AIHE), +0.9%(RAHE), +0.5(LDBHE), +0.4%(LDPHE), +1.3%(RAHE10)].  Furthermore, the source code has been studied and matches the description of JCTVC-H0066.

# Compression performance verification

The source code provided by the proponents of JCTVC-H0066 has been compiled and run under the following test conditions:

- JCTVC-G1200 test conditions [1]

- High Efficiency AIHE, RAHE, LBHE, LPHE, class A,B,C,D,E,F.

- High Efficiency 10 bit RAHE10 only, class A, B, C, D, F.

The software used for the specific substest produced the performance recorded in Table 1.

Table 1: Rate-distortion performance measured by the crosschecker.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | 1,0% | 0,4% | 0,7% |
| Class B | 0,5% | 0,9% | 0,7% |
| Class C | 0,2% | 1,8% | 1,8% |
| Class D | 0,3% | 1,7% | 2,0% |
| Class E | 0,8% | 4,3% | 4,8% |
| Overall | 0,5% | 1,8% | 1,9% |
|  | 0,5% | 1,8% | 1,9% |
| Class F | 0,1% | 0,9% | 0,8% |
| Enc Time[%] | 85% |  |  |
| Dec Time[%] | 93% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A (8bit) | 1,7% | -0,4% | -0,5% |
| Class B | 1,0% | 0,2% | -0,5% |
| Class C | 0,6% | 0,1% | 0,4% |
| Class D | 0,9% | 0,4% | 0,6% |
| Class E |  |  |  |
| Overall | 0,9% | 0,2% | 0,0% |
|  | 0,9% | 0,2% | 0,1% |
| Class F | 0,3% | 0,5% | 0,3% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 93% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0,7% | -0,2% | -0,8% |
| Class C | 0,2% | -0,1% | -0,1% |
| Class D | 0,8% | 0,6% | 1,6% |
| Class E | 0,3% | 0,4% | -2,3% |
| Overall | 0,5% | 0,1% | -0,3% |
|  | 0,5% | 0,1% | -0,3% |
| Class F | 0,2% | -0,1% | -0,4% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 95% |  |  |

|  | Low delay P HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0,9% | -0,9% | -1,6% |
| Class C | 0,2% | -0,5% | -0,8% |
| Class D | 0,3% | 0,5% | 1,1% |
| Class E | 0,5% | 0,7% | -2,5% |
| Class F | 0,2% | -0,1% | -0,4% |
| Overall | 0,4% | -0,1% | -0,8% |
|  | 0,4% | -0,1% | -0,8% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 94% |  |  |

|  | Random Access HE-10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 2,1% | 6,2% | 6,4% |
| Class B | 1,2% | 2,1% | 1,2% |
| Class C | 0,8% | 0,6% | 0,8% |
| Class D | 1,1% | 1,1% | 1,5% |
| Class E |  |  |  |
| Class F | 0,7% | 0,9% | 0,9% |
| Overall | 1,3% | 2,5% | 2,4% |
|  | 1,3% | 2,4% | 2,4% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 91% |  |  |

# Source code examination

The source code has been studied and matches the description of the algorithm provided in JCTVC-H0066.

# Bibliography

[1] JCTVC-G1200, “Common test conditions and software reference configurations” F. Bossen, 7th JCTVC meeting, Geneva, November 2011