| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0588 |
| --- | --- |

| Title: | AHG13: Proposed interlace format indication for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Shun-ichi Sekiguchi Hiroharu Sakate Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

Abstract

This contribution proposes syntax to support interlace format indication in HEVC standard. A flag to indicate whether the input format is interlace format or not is signaled in SPS, and information to indicate whether each picture belongs to top or bottom field is signaled as SEI message. The proposed method does not need any changes on normative process specific for interlaced source.

# Introduction

It has been proposed to support interlace format in HEVC since 6th JCT-VC meeting in Torino[1][2][3]. At the last JCT-VC meeting in Geneva, AHG13 to develop candidate text for an SEI message signaling an interlaced format indicator was established. It was suggested to have an enable flag at the VUI level with a top/bottom flag SEI on each picture for a starting point for this work. We propose to introduce a very simple solution to  signal interlace format indicator and field parity information, which does not require any changes to the current normative process in a slice.

# Proposed changes to support interlace signaling

To support interlace format in HEVC, we propose to put a constraint such that a coded sequence shall not be able to be encoded using adaptive frame/field picture structure. Instead, a coded sequence shall be able to contain coded pictures consisting of frames only (progressive or interlace) or fields only. This can simplify handling of any timing-related normative process such as POC representation or reference picture list management, etc. With this constraint, the problem to be considered to support interlace can be limited to how to signal whether a coded picture represents a frame (more specifically, including 3:2 pull-down case) or a field.

We propose to support a one bit interlace format indication flag in SPS or VUI level header and SEI message to indicate the parity of each field. The proposed CD text is shown below.

interlace_format_indication_flag equal to 1 specifies that all the pictures in the sequence are in filed format. interlace_format_indication_flag equal to 0 specifies that all the picture in the sequence are in frame format.

D.1.1 Field parity indication SEI message syntax

| field_parity_indication( payloadSize ) { | Descriptor |
| --- | --- |
| top_field_flag | u(1) |
| reserved_zero_7bits /* equal to 0 */ | u(7) |
| } |  |

D.2.1 Field parity indication SEI message semantics

top_field_flag equal to 1 specifies that the current picture is a top field. Top_field_flag equal to 0 specifies that the current pictureis a bottom field.

# Conclusion

We recognize that it is quite important to complete the work of HEVC standardization within currently agreed timeline. In this sense, it is not feasible to discuss normative coding tool enhancements specifically for interlace (legacy) sources. Thus, we believe that the most realistic approach should be to support field-picture representation only with minimum required changes such as the proposed SEI message.

We propose the described method to be supported in CD.

References:

[1] S.Sekiguchi, et.al., “Proposal on the support of interlace format in HEVC”, JCTVC-F194, July 2011.

[2] S.Sekiguchi, et.al., “Joint proposal of HEVC requirement on the support of interlace format”, ISO/IEC JTC1/SC29/WG11/M21411, July 2011.

[3] K.Sugimoto, et.al., “High level syntax to support interlace format”, ISO/IEC JTC1/SC29/WG11/JCTVC-G450, November 2011.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).