| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0589_r1 |
| --- | --- |

| Title: | CE7: Cross-check for Tests 1 and 2 (JCTVC-H0126) by BBC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Authors: | Marta Mrak, David Flynn BBC R&D, Centre House, 56 Wood Lane W12 7SB London, UK | Email: | marta.mrak@bbc.co.uk |
| Source: | British Broadcasting Corporation |  |  |

_____________________________

# Abstract

This document reports the evaluation of Tools 3 from CE7. The tool enables a boundary-dependent transform applied on inter prediction residue with 8x8 and 4x4 secondary transform. The results confirm the proponent’s BD-rates. The running times are close to the proponents’ and within the range of variability associated with the cluster used for the experiments.

# Cross-check report

This test evaluates secondary transforms for inter residuals. In Test 1 at size 8 and larger for the inter residue, a secondary transform of size 8x8 is used after the DCT. In Test 2 at size 8 and larger for the inter residue, a secondary transform of size 4x4 is used after the DCT.

Tested method is evaluated with respect to HM-5.0. Two sets of evaluations are performed - for Test 1 (results in Table a) and for Test 2 (results in Table 2) from JCTVC-H0126. All mandatory inter configurations are tested and compared to HM-5.0.

Table 1. BD Rate gains when proposed transform scheme is applied on Inter prediction Residue with 8x8 secondary transform

|  | Random Access HE-10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  | V |  |  |  |
| Class A | -0.2% |  | -0.2% |  | 0.1% |  |  |  |
| Class B | -0.2% |  | 0.1% |  | 0.1% |  |  |  |
| Overall | -0.2% |  | 0.0% |  | 0.1% |  |  |  |
| Enc Time[%] | 101% |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |  |  |
|  | Random Access HE |  |  |  |  | Random Access LC |  |  |
|  | Y | U |  | V |  | Y | U | V |
| Class A (8bit) | -0.4% | -0.2% |  | -0.2% |  | -0.6% | 0.1% | 0.0% |
| Class B | -0.3% | 0.0% |  | 0.0% |  | -0.4% | 0.3% | 0.2% |
| Class C | -0.3% | -0.1% |  | 0.0% |  | -0.3% | 0.1% | 0.2% |
| Class D | -0.2% | -0.2% |  | -0.2% |  | -0.3% | 0.3% | 0.2% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.1% |  | -0.1% |  | -0.4% | 0.2% | 0.2% |
|  | -0.3% | -0.1% |  | -0.1% |  | -0.4% | 0.2% | 0.1% |
| Class F | -0.1% | 0.1% |  | 0.0% |  | -0.1% | 0.1% | 0.0% |
| Enc Time[%] | 102% |  |  |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  |  |  | 101% |  |  |
|  | Low delay B HE |  |  |  |  | Low delay B LC |  |  |
|  | Y | U |  | V |  | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.4% | -0.1% |  | 0.1% |  | -0.6% | 0.4% | 0.8% |
| Class C | -0.4% | -0.2% |  | -0.1% |  | -0.5% | 0.4% | 0.3% |
| Class D | -0.3% | 0.2% |  | 0.0% |  | -0.5% | 0.8% | 0.8% |
| Class E | -0.2% | 0.4% |  | -0.3% |  | -0.4% | 0.1% | 0.8% |
| Overall | -0.4% | 0.1% |  | -0.1% |  | -0.5% | 0.4% | 0.7% |
|  | -0.4% | 0.1% |  | -0.1% |  | -0.5% | 0.5% | 0.5% |
| Class F | -0.3% | 0.0% |  | -0.2% |  | -0.7% | -1.0% | -0.3% |
| Enc Time[%] | 101% |  |  |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  |  |  | 101% |  |  |

Table 2. BD Rate gains when proposed transform scheme is applied on Inter prediction Residue with 4x4 secondary transform

|  | Random Access HE-10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  | V |  |  |  |
| Class A | -0.2% |  | 0.0% |  | 0.0% |  |  |  |
| Class B | -0.2% |  | 0.1% |  | 0.0% |  |  |  |
| Overall | -0.2% |  | 0.0% |  | 0.0% |  |  |  |
| Enc Time[%] | 101% |  |  |  |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  |  |  |  |
|  | Random Access HE |  |  |  |  | Random Access LC |  |  |
|  | Y | U |  | V |  | Y | U | V |
| Class A (8bit) | -0.4% | -0.2% |  | -0.2% |  | -0.5% | 0.1% | 0.0% |
| Class B | -0.2% | 0.0% |  | 0.0% |  | -0.3% | 0.2% | 0.2% |
| Class C | -0.2% | -0.1% |  | 0.0% |  | -0.3% | 0.1% | 0.2% |
| Class D | -0.3% | -0.2% |  | -0.1% |  | -0.3% | 0.1% | 0.1% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.1% |  | -0.1% |  | -0.3% | 0.1% | 0.1% |
|  | -0.3% | -0.1% |  | -0.1% |  | -0.3% | 0.1% | 0.1% |
| Class F | -0.1% | 0.0% |  | 0.0% |  | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  |  |  | 100% |  |  |
|  | Low delay B HE |  |  |  |  | Low delay B LC |  |  |
|  | Y | U |  | V |  | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.4% | -0.1% |  | 0.2% |  | -0.5% | 0.2% | 0.4% |
| Class C | -0.4% | -0.1% |  | 0.1% |  | -0.5% | 0.3% | 0.2% |
| Class D | -0.3% | 0.3% |  | 0.3% |  | -0.5% | 1.1% | 0.3% |
| Class E | -0.2% | -0.2% |  | 0.0% |  | -0.3% | 1.0% | 0.7% |
| Overall | -0.3% | 0.0% |  | 0.1% |  | -0.4% | 0.6% | 0.4% |
|  | -0.3% | 0.0% |  | 0.1% |  | -0.4% | 0.6% | 0.4% |
| Class F | -0.2% | 0.2% |  | -0.1% |  | -0.6% | -0.5% | -0.4% |
| Enc Time[%] | 101% |  |  |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  |  |  | 101% |  |  |