| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0593 |
| --- | --- |

| Title: | Cross-verification of JCTVC-H0348 on simplification on tiles and slices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vadim Seregin Muhammed Coban 5775 Morehouse Drive San Diego, CA 92121, USA | Email: | vseregin@qualcomm.com mcoban@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of JCTVC-H0348 on simplification on tiles and slices. The experiment results are matched with the proponent data.

# Introduction and experimental results

Contribution [1] proposes restriction on one or more slices to be included inside a tile but not cross it, so tile is always larger or equal to a slice as shown in Fig. 1.

Picture

Slice

Tile

Picture

Tile

Slice

Current

Proposed

Fig. 1. Relationship between tiles and slices

Two configuration sets were tested with 2 and 4 columns dependent tiles, and first slice is always encoded as non-entropy slice in tiles.

Additionally, it is proposed to modify syntax, since with slice restriction tile_boundary_independence_flag is not needed, loop_filter_across_tile_flag can be substituted by loop_filter_across_slice_flag, and markers for entry points of tiles in the bitstream are not necessary because they are known by parsing slice header and matching slice address and tile entry address. Max number of merge candidates syntax is excluded from the entropy slice header syntax.

The source code based on HM5.0-dev-misc(r1763) with applied patch #286 was released by the proponent and it was verified that described method for slice restriction was implemented. The source code was executed on Linux cluster under common test conditions [2] and experimental results were compared against HM5.0-dev-misc(r1763) with patch #286.

Experimental results, summarised in the next tables, are matched with the data provided by the proponent, however encoding and decoding time is not accurate due to non-uniform cluster.

Table 1. CABAC initialization by table (column 2)

Table 2. Using Entropy slice (column 2)

Table 3. CABAC initialization by table (column 4)

Table 4. Using Entropy slice (column 4)

# References

- T. Lee, B. Choi and J. Park “Simplification on tiles and slices,” JCTVC-H0348, 8th JCT-VC Meeting, San Jose, US, February 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” JCTVC-G1200, 7th JCT-VC Meeting, Geneva, CH, November 2011.