| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0595 |
| --- | --- |

| Title: | Cross-check of Canon’s harmonisation of 8x8 TU residual scan (JCTVC-H0145) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Joel Sole 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-2429 joels@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution presents the results of the cross-check performed by Qualcomm on Canon’s proposed replacement of the 8x8 TU scan with 4x4 sub-block scans in JCTVC-H0145. The cross-check performance results match those provided by the proponent. The implemented algorithms also agree with what is described in Canon’s proposal.

# Introduction

Software implementing JCTVC-H0145 [1] on top of HM5.0 basis was provided by Canon. Code changes are small and localized in files TComRom.cpp, TEncSbac.cpp and TDecSbac.cpp.

Changes in TComRom.cpp modify the scan definition for 8x8 TU, which is divided in 4x4 sub-blocks. Diagonal scan is done within a sub-block and across sub-blocks for 8x8. There are two variants to tackle MDCS. The difference is the scanning process across sub-blocks: in one case a diagonal scan is always used while in the other, the horizontal / vertical scan is used.

Modifications in TEncSbac.cpp and TDecSbac.cpp are to process the 8x8 TU with the 2-level significance map of the 16x16 and 32x32 TUs.

Complexity of the algorithm is similar to that of the anchor. Harmonization of the processing of all TUs in sub-blocks of 4x4 is achieved.

# Results

Coding results using common conditions have been obtained and match those of the proponent. Results are provided in the accompanying excel files.

- References

- C. Rosewarne, M. Maeda, “Non-CE11: Harmonisation of 8x8 TU residual scan”, JCTVC-H0145, 8th JCT-VC Meeting, San Jose, USA, February 2012.