| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0596 |
| --- | --- |

| Title: | Non-CE6: Cross-verification of Qualcomm's intra mode coding simplification (JCTVC-H0563) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shigeru Fukushima Toru Kumakura JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6597 fukushima.shigeru@ jvckenwood.com kumakura.toru@ jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of non-CE6 intra mode coding simplification (JCTVC-H0563) by Qualcomm. It is confirmed that the BD-rate results match those provided by the proponent.

# Introduction

This contribution reports cross-verification results of intra mode coding simplification (JCTVC-H0563) by Qualcomm [1].

# Algorithm Description

The number of MPMs is increased to 3. The MPM index is coded with truncated unary binarization, therefore the maximum number of bins for the MPM index is 2. The context for first bin is added (bypass decoding is applied for second bin).

The derivation of additional prior candidate is the same as described in [2]. The assignment of the additional prior candidate is different from as described in [2]. The additional prior candidate is assigned to index 0 of most probable mode.

The mode18 is added for 4x4 blocks so that the total number of remaining mode is just 24. For 8x8 / 16x16 / 32x32 / 64x64 blocks, as the total number of remaining mode is just 25, the escape code can be removed.

# Experiments

## Software

The software provided by Qualcomm is implemented into HM5.0.

## Test conditions

The common test conditions [3] are used.

## Experimental environment

The experimental environment used is presented in Table1.

Table 1 Experimental environment

| CPU | Intel Core i7 870 2.93GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table2 presents the simulation results for intra only.

Table 2 Simulation results for intra only

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.3% | -0.1% | 0.0% | -0.3% | -0.1% | -0.1% |
| Class B | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | 0.0% |
| Class C | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% |
| Class D | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class E | -0.3% | -0.1% | -0.1% | -0.3% | 0.0% | -0.1% |
| Overall | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% |
|  | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% |
| Class F | -0.2% | -0.2% | -0.2% | -0.2% | 0.1% | -0.3% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusions

It is confirmed that the BD-rate results match those provided by the proponent. Since this proposal can achieve fixed length binarization for intra remainder mode coding, it is recommended to review this proposal.

# References

[1] “Non-CE6: Intra mode coding simplification”,

JCTVC-H0563, San José, February 2012

[2] “CE6: Subtest 6c (6.2.1) – Intra mode coding simplification”,

JCTVC-H0075, San Jose, February 2012

[3] “Common test conditions and software reference configurations”,

JCTVC-G1200, Geneva, November 2011

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).