| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0600 |
| --- | --- |

| Title: | AHG7: Level definition to limit memory bandwidth of MC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo, Yoshitaka Morigami and Teruhiko Suzuki Sony City Osaki, 2-10-1 Osaki Shinagawa-ku, Tokyo Japan | Tel: Email: | +81-50-3750-2744 kenji.kondo@jp.sony.com Yoshitaka.Morigami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to limit maximum memory bandwidth of motion compensation (MC). The desirable restriction is discussed and simple solutions are introduced. One of the solutions is to restrict small PU block, another solution is to limit number of motion vectors in a LCU. This method is applied in H.264/AVC standard [1]. The maximum number of MVs of anchor streams is reported. For small block restriction, the impact of coding efficiency is shown.

# Introduction

The memory bandwidth especially maximum of MC process is big issue for implementation. It was mentioned in [2]. We showed that small block and bi-prediction consume memory bandwidth in the [2].

## Desirable restriction

For level definition of MC, we think that several conditions are required as follow:

- Possible to constrain the max memory bandwidth

  - In relevant small region

    - A restriction for PU level is best to limit the bandwidth but it will increase the coding efficiency loss.

    - A restriction for picture level is difficult to design hardware both encoder and decoder, but it may decrease the coding efficiency loss.

    - We think that the guarantee for each LCU or CU is good trade-off.

- Easy to check the conformance

  - To motivate us to protect this level definition.

- Good flexibility for new level

  - In the future, new level may be defined by something reason.

- Harmless coding performance

## Restriction for small block

The restriction for PU is simpler to limit the memory bandwidth. If we restrict the PU have large memory bandwidth, the restriction case is shown in Table 1. For example, restriction A3 was chosen to restrict bi-prediction for 4x4, 4x8, and 8x4. Where, the reason that Bi-4x8 is restricted ahead than Bi-8x4 comes from memory access alignment that has rectangular map (e.g. 4x2).

Table 1: Restriction case

| Restriction | Restricted modes |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A0 |  |  |  |  |  |  |  |
| A1 | Bi-4x4 |  |  |  |  |  |  |
| A2 | Bi-4x4 | Bi-4x8 |  |  |  |  |  |
| A3 | Bi-4x4 | Bi-4x8 | Bi-8x4 |  |  |  |  |
| A4 | Bi-4x4 | Bi-4x8 | Bi-8x4 | Uni-4x4 |  |  |  |
| A5 | Bi-4x4 | Bi-4x8 | Bi-8x4 | Uni-4x4 | Bi-8x8 |  |  |
| A6 | Bi-4x4 | Bi-4x8 | Bi-8x4 | Uni-4x4 | Bi-8x8 | Uni-4x8 |  |
| A7 | Bi-4x4 | Bi-4x8 | Bi-8x4 | Uni-4x4 | Bi-8x8 | Uni-4x8 | Uni-8x4 |

## Maximum number of motion vectors

The one of solutions to reduce the bandwidth is to limit number of motion vectors. In H.264/AVC standard, the max number of motion vectors per two consecutive macro blocks (MaxMvsPer2Mb) is defined. By this level definition, it was prohibited that small block such as 4x4 block of bi-prediction is not always continually used. Actually, this limitation is meaningful for implementation. For hardware, engineers can design the memory bus including throughput and latency to satisfy maximum memory bandwidth.

In H.264/AVC standard, the MaxMvsPer2Mb is 32 and 16 for restriction B3 and over 3.1. If we extend the level definition to HEVC, it may be shown in Table 2. Where the restriction B1 of this table is corresponding to restriction B3 of H.264/AVC and the restriction B2 is corresponding to level 3.1 and above of H.264/AVC. The size of MB is 16x16 and two MBs can have 32 motion vectors for restriction B3 of H.264/AVC, so 16 MVs can be calculated per one MB.

Table 2 Max number of motion vectors per one LCU

| LCU size | Restriction B1 | Restriction B2 | Restriction B3 |
| --- | --- | --- | --- |
| 16x16 | 16 | 8 | 4 |
| 32x32 | 64 | 32 | 16 |
| 64x64 | 256 | 128 | 64 |

The size of largest coding unit (LCU) is given by log2_min_coding_block_size_minus3 and log2_diff_max_min_coding_block_size in sequence parameter set (sps).

For restriction B3, we think that more restriction is required than restriction B2 for large image application such as 4K2K or high frame rate.

# Experiment results

## Small block restriction

For large image as class A, B and E, we checked the impact of coding efficiency when small blocks are restricted as Table 1. It’s shown in Figure 1. Where, we tested also class A in low delay case to know the BD% and the class A includes four test sequences. As we can see, Lv.0 case has some gain because the Lv. 0 allow to use the inter 4x4. The case Lv. 5 as orange bar has large loss than Lv. 4. Probably, the reason is that a skip (/merge) mode with candidates of bi-prediction was prohibited. The solutions are proposed to avoid the problem in [3, 4].

Figure 1: BD% for restriction case

## Number of motion vectors

We investigated the max number of motion vectors in LCU using anchor streams. The results are shown in Figure 2, Figure 3 and Table 3. As we can see, the number of MVs increases as QP decreases, and the maximum of MVs in all streams is 203 at “BQSquare”, QP22 and RA-LC. For low-delay P condition, the number is smaller about half because the condition does not allow using bi-prediction.

Figure 2 Max number of motion vectors for class C, D and E

Figure 3: Max number of motion vectors for class A and B

Table 3: Maximum number of MVs

| Cs, test sequence | QP22 | QP27 | QP32 | QP37 | Max |
| --- | --- | --- | --- | --- | --- |
| LB-HE |  |  |  |  |  |
| A.NebutaFestival | 121 | 101 | 91 | 78 | 121 |
| A.PeopleOnStreet | 144 | 115 | 111 | 100 | 144 |
| A.SteamLocomotiveTrain | 126 | 108 | 74 | 83 | 126 |
| A.Traffic | 134 | 123 | 108 | 89 | 134 |
| B.BasketballDrive | 159 | 129 | 121 | 98 | 159 |
| B.BQTerrace | 164 | 143 | 130 | 117 | 164 |
| B.Cactus | 172 | 164 | 140 | 108 | 172 |
| B.Kimono1 | 131 | 122 | 98 | 80 | 131 |
| B.ParkScene | 153 | 134 | 117 | 116 | 153 |
| C.BasketballDrill | 158 | 136 | 113 | 104 | 158 |
| C.BQMall | 152 | 144 | 133 | 98 | 152 |
| C.PartyScene | 183 | 174 | 143 | 135 | 183 |
| C.RaceHorses | 153 | 115 | 107 | 84 | 153 |
| D.BasketballPass | 152 | 143 | 116 | 108 | 152 |
| D.BlowingBubbles | 165 | 155 | 126 | 98 | 165 |
| D.BQSquare | 177 | 178 | 146 | 110 | 178 |
| D.RaceHorses | 147 | 125 | 110 | 89 | 147 |
| E.vidyo1 | 126 | 98 | 86 | 72 | 126 |
| E.vidyo3 | 145 | 133 | 117 | 114 | 145 |
| E.vidyo4 | 126 | 104 | 91 | 70 | 126 |
| LB-LC |  |  |  |  |  |
| A.NebutaFestival | 142 | 122 | 119 | 84 | 142 |
| A.PeopleOnStreet | 160 | 138 | 125 | 109 | 160 |
| A.SteamLocomotiveTrain | 124 | 100 | 102 | 87 | 124 |
| A.Traffic | 166 | 142 | 116 | 107 | 166 |
| B.BasketballDrive | 167 | 139 | 124 | 109 | 167 |
| B.BQTerrace | 181 | 157 | 134 | 123 | 181 |
| B.Cactus | 185 | 178 | 156 | 116 | 185 |
| B.Kimono1 | 144 | 118 | 92 | 79 | 144 |
| B.ParkScene | 167 | 141 | 132 | 128 | 167 |
| C.BasketballDrill | 170 | 136 | 118 | 107 | 170 |
| C.BQMall | 164 | 156 | 129 | 112 | 164 |
| C.PartyScene | 183 | 173 | 159 | 135 | 183 |
| C.RaceHorses | 164 | 142 | 126 | 93 | 164 |
| D.BasketballPass | 155 | 144 | 130 | 121 | 155 |
| D.BlowingBubbles | 182 | 162 | 144 | 115 | 182 |
| D.BQSquare | 188 | 165 | 128 | 108 | 188 |
| D.RaceHorses | 148 | 136 | 128 | 102 | 148 |
| E.vidyo1 | 142 | 139 | 115 | 84 | 142 |
| E.vidyo3 | 160 | 172 | 153 | 111 | 172 |
| E.vidyo4 | 151 | 130 | 97 | 92 | 151 |
| LP-HE |  |  |  |  |  |
| B.BasketballDrive | 103 | 86 | 78 | 64 | 103 |
| B.BQTerrace | 111 | 100 | 88 | 74 | 111 |
| B.Cactus | 106 | 100 | 85 | 69 | 106 |
| B.Kimono1 | 82 | 64 | 55 | 46 | 82 |
| B.ParkScene | 101 | 97 | 81 | 73 | 101 |
| C.BasketballDrill | 102 | 95 | 69 | 59 | 102 |
| C.BQMall | 100 | 88 | 77 | 79 | 100 |
| C.PartyScene | 113 | 113 | 104 | 86 | 113 |
| C.RaceHorses | 99 | 83 | 80 | 68 | 99 |
| D.BasketballPass | 102 | 90 | 78 | 64 | 102 |
| D.BlowingBubbles | 104 | 96 | 85 | 61 | 104 |
| D.BQSquare | 100 | 101 | 91 | 75 | 101 |
| D.RaceHorses | 94 | 89 | 74 | 65 | 94 |
| E.vidyo1 | 76 | 62 | 57 | 46 | 76 |
| E.vidyo3 | 81 | 82 | 69 | 66 | 82 |
| E.vidyo4 | 82 | 71 | 58 | 50 | 82 |
| LP-LC |  |  |  |  |  |
| B.BasketballDrive | 107 | 98 | 93 | 69 | 107 |
| B.BQTerrace | 114 | 106 | 97 | 79 | 114 |
| B.Cactus | 109 | 106 | 96 | 78 | 109 |
| B.Kimono1 | 94 | 79 | 60 | 53 | 94 |
| B.ParkScene | 104 | 100 | 89 | 77 | 104 |
| C.BasketballDrill | 108 | 96 | 84 | 67 | 108 |
| C.BQMall | 102 | 107 | 86 | 76 | 107 |
| C.PartyScene | 114 | 112 | 108 | 97 | 114 |
| C.RaceHorses | 102 | 96 | 79 | 69 | 102 |
| D.BasketballPass | 109 | 94 | 82 | 78 | 109 |
| D.BlowingBubbles | 109 | 103 | 104 | 77 | 109 |
| D.BQSquare | 107 | 104 | 93 | 75 | 107 |
| D.RaceHorses | 104 | 95 | 81 | 66 | 104 |
| E.vidyo1 | 92 | 79 | 64 | 52 | 92 |
| E.vidyo3 | 94 | 104 | 87 | 87 | 104 |
| E.vidyo4 | 96 | 77 | 61 | 52 | 96 |
| RA-HE |  |  |  |  |  |
| A.NebutaFestival | 127 | 108 | 95 | 74 | 127 |
| A.PeopleOnStreet | 131 | 124 | 99 | 104 | 131 |
| A.SteamLocomotiveTrain | 109 | 83 | 80 | 70 | 109 |
| A.Traffic | 140 | 116 | 128 | 128 | 140 |
| B.BasketballDrive | 145 | 133 | 108 | 85 | 145 |
| B.BQTerrace | 165 | 157 | 128 | 104 | 165 |
| B.Cactus | 172 | 148 | 153 | 141 | 172 |
| B.Kimono1 | 140 | 115 | 90 | 74 | 140 |
| B.ParkScene | 147 | 148 | 128 | 104 | 148 |
| C.BasketballDrill | 171 | 142 | 104 | 75 | 171 |
| C.BQMall | 154 | 122 | 110 | 101 | 154 |
| C.PartyScene | 193 | 168 | 144 | 124 | 193 |
| C.RaceHorses | 145 | 106 | 86 | 71 | 145 |
| D.BasketballPass | 158 | 131 | 109 | 87 | 158 |
| D.BlowingBubbles | 180 | 140 | 120 | 101 | 180 |
| D.BQSquare | 178 | 162 | 129 | 119 | 178 |
| D.RaceHorses | 144 | 118 | 97 | 72 | 144 |
| RA-HE10 |  |  |  |  |  |
| A.NebutaFestival | 127 | 103 | 86 | 66 | 127 |
| A.PeopleOnStreet | 136 | 116 | 110 | 98 | 136 |
| A.SteamLocomotiveTrain | 95 | 86 | 80 | 80 | 95 |
| A.Traffic | 134 | 105 | 115 | 98 | 134 |
| B.BasketballDrive | 158 | 129 | 100 | 105 | 158 |
| B.BQTerrace | 168 | 146 | 121 | 128 | 168 |
| B.Cactus | 163 | 146 | 155 | 134 | 163 |
| B.Kimono1 | 140 | 107 | 92 | 76 | 140 |
| B.ParkScene | 149 | 147 | 127 | 112 | 149 |
| RA-LC |  |  |  |  |  |
| A.NebutaFestival | 154 | 150 | 118 | 85 | 154 |
| A.PeopleOnStreet | 144 | 133 | 111 | 105 | 144 |
| A.SteamLocomotiveTrain | 127 | 106 | 92 | 85 | 127 |
| A.Traffic | 159 | 126 | 128 | 104 | 159 |
| B.BasketballDrive | 162 | 145 | 125 | 98 | 162 |
| B.BQTerrace | 172 | 156 | 132 | 128 | 172 |
| B.Cactus | 177 | 153 | 153 | 148 | 177 |
| B.Kimono1 | 133 | 115 | 92 | 70 | 133 |
| B.ParkScene | 172 | 155 | 130 | 128 | 172 |
| C.BasketballDrill | 177 | 141 | 123 | 89 | 177 |
| C.BQMall | 163 | 144 | 129 | 120 | 163 |
| C.PartyScene | 201 | 171 | 156 | 137 | 201 |
| C.RaceHorses | 150 | 121 | 113 | 94 | 150 |
| D.BasketballPass | 163 | 135 | 109 | 93 | 163 |
| D.BlowingBubbles | 171 | 155 | 131 | 110 | 171 |
| D.BQSquare | 203 | 167 | 152 | 127 | 203 |
| D.RaceHorses | 156 | 126 | 102 | 80 | 156 |
| Max | 203 | 178 | 159 | 148 | 203 |

# Conclusions

This contribution introduced a desirable restriction and the ideas to limit memory bandwidth. For small block restriction, the impact of coding efficiency was reported. And for anchor streams, the maximum number of motion vectors per one LCU was reported. The maximum number of MVs was 203 in condition RA-LC, QP22, and test sequence “BQSquare”. It’s proposed that ideas mentioned in this contribution are further studied in AHG for memory bandwidth of MC.

# References

[1] ITU-T Recommendation H.264 and ISO/IEC 14496-10 (MPEG-4 part 10) Advanced Video Coding (AVC)

[2] K. Kondo, “Non-CE3: Report on a restriction for small block”, JCTVC-G392, JCT-VC 7th meeting, Geneva, 2011

[3] T. Ikai, “AHG7: Controllable memory bandwidth reduction with bi-pred to uni-pred conversion”, JCTVC-H0096, JCTVC 8th meeting, San Jose, 2011

[4] K. Kondo, et al., “AHG7: Modification of merge candidate derivation to reduce MC memory bandwidth”, JCTVC-H0221, JCT-VC 8th meeting, San Jose, 2011

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).