| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0601 |
| --- | --- |

| Title: | Non-CE4: Crosscheck report of Compatible QP prediction with RC and AQ (JCTVC-H0204) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Masato Shima Canon Inc. 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan | Tel: Email: | +81-3-3758-2111 shima.masato@canon.co.jp |
| Source: | Canon Inc. |  |  |

_____________________________

# Abstract

This contribution contains the cross check result of JCTVC-H0204, Compatible QP prediction with RC and AQ proposed by JVC Kenwood.  JCTVC-H0204 contains two proposals, but this contribution crosschecks the results presented as proposal 2 in JCTVC-H0204.

# Cross Check Procedure

The source code corresponds to JCTVC-H0204 was provided by JVC Kenwood, and it was compiled with GCC 3.4.6.  All the simulation was done on a Linux cluster.  The results of CE4 Subtest1.1, which predicts QP from previously coded CU, were used as the reference data.  JCTVC-H0204 contains two proposals, but this contribution crosschecks the results presented as proposal 2 in JCTVC-H0204.

For the proposal 2 results, the tested data were generated by modifying the provided source code as follows:

#define JVC_QP_PREDICTION                    1

#if JVC_QP_PREDICTION

#define G1028_QP_PREDICTION_WITH_INTRA    0

#define G1028_QP_PREDICTION_WITHIN_LCU    1

#endif

The test condition defined in JCTVC-G1204 subtest 1 was used for the crosscheck.

# Results

## Simulation Result of proposal 2 with MaxCUDepthDQP=3 (8x8)

Cross check results for “proposal2” with MaxCUDepthDQP=3 (8x8) are summarized in Table 1, where proposed “proposal2” is compared with CE4 subtest1 anchor (subtest1.1).

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.2% | -0.2% | -0.2% | -4.2% | -0.1% | -0.1% | -0.1% | -3.9% |
| Class B | -0.3% | -0.3% | -0.3% | -6.7% | -0.2% | -0.2% | -0.2% | -6.7% |
| Class C | -0.4% | -0.3% | -0.4% | -8.9% | -0.3% | -0.3% | -0.3% | -9.1% |
| Class D | -0.4% | -0.4% | -0.3% | -9.6% | -0.3% | -0.3% | -0.3% | -9.7% |
| Class E | -0.3% | -0.3% | -0.3% | -6.0% | -0.3% | -0.3% | -0.4% | -6.7% |
| Overall | -0.3% | -0.3% | -0.3% | -7.5% | -0.3% | -0.3% | -0.3% | -7.6% |
|  | -0.3% | -0.3% | -0.3% |  | -0.3% | -0.3% | -0.3% |  |
| Class F | -0.1% | -0.1% | -0.1% | -4.3% | -0.1% | -0.2% | -0.1% | -4.0% |
| Enc Time[%] | 99% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.2% | -0.1% | -0.1% | -3.8% | -0.4% | -0.2% | -0.3% | -3.7% |
| Class B | -0.2% | -0.2% | -0.2% | -4.9% | -0.3% | 0.0% | 0.0% | -4.6% |
| Class C | -0.3% | -0.1% | -0.3% | -6.8% | -0.3% | -0.4% | -0.2% | -6.8% |
| Class D | -0.3% | -0.4% | -0.3% | -7.0% | -0.3% | -0.5% | -0.4% | -6.9% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.2% | -0.2% | -5.8% | -0.3% | -0.3% | -0.2% | -5.7% |
|  | -0.3% | -0.2% | -0.2% |  | -0.3% | -0.3% | -0.2% |  |
| Class F | -0.1% | -0.1% | -0.1% | -3.6% | -0.2% | -0.2% | -0.2% | -3.5% |
| Enc Time[%] | 101% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | -0.2% | -4.0% | -0.3% | -0.3% | -0.4% | -3.5% |
| Class C | -0.3% | -0.3% | -0.1% | -5.9% | -0.3% | -0.2% | -0.1% | -5.5% |
| Class D | -0.3% | -0.5% | -0.2% | -6.0% | -0.3% | -0.4% | 0.0% | -5.8% |
| Class E | -0.1% | -0.4% | -0.6% | -3.3% | -0.2% | -1.4% | 0.1% | -4.0% |
| Overall | -0.2% | -0.4% | -0.2% | -4.8% | -0.3% | -0.5% | -0.1% | -4.7% |
|  | -0.2% | -0.4% | -0.3% |  | -0.3% | -0.5% | 0.0% |  |
| Class F | -0.1% | -0.2% | -0.1% | -2.5% | -0.1% | 1.0% | 0.1% | -2.4% |
| Enc Time[%] | 98% |  |  |  | 98% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 99% |  |  |  |

|  | Random Access HE-10 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. |
| Class A (8bit) | -0.1% | -0.1% | -0.5% | -3.2% |
| Class B | -0.2% | -0.3% | -0.1% | -4.9% |
| Class C |  |  |  |  |
| Class D |  |  |  |  |
| Class E |  |  |  |  |
| Overall | -0.2% | -0.2% | -0.3% | -4.2% |
|  | -0.2% | -0.2% | -0.3% |  |
| Class F |  |  |  |  |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

Table 1: BD-rate results for proposal2 vs. Subtest1.1 anchor with MaxCUDepthDQP=3 (8x8)

## Simulation Result of proposal 2 with MaxCUDepthDQP=2 (16x16)

Cross check results for “proposal2” with MaxCUDepthDQP=2 (16x16) are summarized in Table 2, where proposed “proposal2” is compared with CE4 subtest1 anchor (subtest1.1).

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.1% | -0.1% | -0.1% | -3.0% | 0.0% | 0.0% | -0.1% | -2.7% |
| Class B | -0.1% | -0.1% | -0.1% | -5.4% | -0.1% | -0.1% | -0.1% | -5.2% |
| Class C | -0.2% | -0.1% | -0.1% | -7.7% | -0.1% | -0.1% | -0.1% | -7.7% |
| Class D | -0.1% | -0.2% | -0.1% | -7.7% | -0.1% | -0.1% | -0.1% | -7.7% |
| Class E | -0.2% | -0.2% | -0.2% | -5.3% | -0.2% | -0.1% | -0.1% | -5.4% |
| Overall | -0.1% | -0.1% | -0.1% | -6.1% | -0.1% | -0.1% | -0.1% | -6.1% |
|  | -0.1% | -0.1% | -0.1% |  | -0.1% | -0.1% | -0.1% |  |
| Class F | 0.0% | 0.1% | 0.0% | -3.3% | 0.0% | -0.1% | 0.0% | -3.4% |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 102% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.1% | -0.2% | 0.0% | -3.2% | -0.2% | -0.3% | -0.1% | -3.3% |
| Class B | -0.1% | 0.0% | 0.1% | -4.1% | -0.1% | -0.1% | 0.0% | -4.1% |
| Class C | -0.2% | 0.0% | 0.0% | -5.8% | -0.2% | -0.2% | 0.0% | -5.9% |
| Class D | -0.2% | -0.1% | -0.3% | -5.8% | -0.2% | 0.0% | -0.1% | -6.0% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.2% | -0.1% | 0.0% | -4.9% | -0.2% | -0.1% | -0.1% | -5.0% |
|  | -0.2% | -0.1% | -0.1% |  | -0.2% | -0.1% | -0.1% |  |
| Class F | -0.1% | -0.2% | -0.1% | -2.8% | -0.2% | -0.2% | -0.2% | -2.9% |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 102% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1% | -0.2% | -0.2% | -3.7% | -0.2% | -0.3% | -0.7% | -3.5% |
| Class C | -0.2% | -0.2% | -0.1% | -5.3% | -0.2% | 0.2% | 0.0% | -5.3% |
| Class D | -0.1% | -0.2% | 0.2% | -5.5% | -0.2% | 1.0% | 0.2% | -5.5% |
| Class E | -0.3% | 0.0% | -0.5% | -3.6% | -0.2% | 1.0% | 0.2% | -3.7% |
| Overall | -0.2% | -0.2% | -0.1% | -4.5% | -0.2% | 0.4% | -0.1% | -4.5% |
|  | -0.2% | -0.2% | -0.1% |  | -0.2% | 0.4% | -0.1% |  |
| Class F | -0.2% | 0.1% | -0.3% | -2.0% | 0.0% | -0.2% | -0.4% | -1.9% |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |

|  | Random Access HE-10 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. |
| Class A (8bit) | -0.1% | 0.1% | 0.1% | -2.9% |
| Class B | -0.1% | -0.1% | 0.0% | -4.2% |
| Class C |  |  |  |  |
| Class D |  |  |  |  |
| Class E |  |  |  |  |
| Overall | -0.1% | 0.0% | 0.0% | -3.6% |
|  | -0.1% | 0.0% | 0.0% |  |
| Class F |  |  |  |  |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

Table 2: BD-rate results for proposal2 vs. Subtest1.1 anchor with MaxCUDepthDQP=2 (16x16)

The BD-rate results obtained in our simulation match the results provided by JVC Kenwood.  Also, there is no substantial difference on Enc/Dec time between our results and results provided by JVC Kenwood.  Accordingly, the bitrate, PSNR, and Enc/Dec time results provided by JVC Kenwood were verified.

# Conclusion

It has been verified that the bitrate and PSNR results provided by JVC Kenwood on JCTVC-H0204 proposal 2 are accurate.