| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0602 |
| --- | --- |

| Title: | Non-CE4: Crosscheck report of High-level Syntaxes for the Scaling List Matrices Parameters and Parametric coding (JCTVC-H0460) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Masato Shima Canon Inc. 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan | Tel: Email: | +81-3-3758-2111 shima.masato@canon.co.jp |
| Source: | Canon Inc. |  |  |

_____________________________

# Abstract

This contribution contains the crosscheck result of JCTVC-H0460, proposal of High-level Syntaxes for the Scaling List Matrices Parameters and Parametric coding proposed by Sony.

# Cross Check Procedure

The source code corresponds to JCTVC-H0460 was provided by Sony, and it was compiled with GCC 3.4.6.  All the simulation was done on a Linux cluster.  According to the test description in JCTVC-G1204 (CE4 description), the following performance was measured:

- 6 sets symmetry and asymmetry quantization matrices (12 in total) coding performance

  - Results of compression performance of matrix itself with error statistics

The test condition defined in JCTVC-G1204 subtest 2 was used for the crosscheck.

# Results

## Quantization Matrices Coding Performance

Using the provided software and configuration file for JCTVC-H0460, testing was conducted for all 12 quantization matrices (symmetry1 to symmetry6 and asymmetry1 to asymmetry6).  The measured results are as follows:

| (H0460) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Header Bit | 4 | 4 | 4 | 4 | 4 | 4 |
| Matrix [0][0] Bit | 84 | 68 | 100 | 94 | 84 | 100 |
| Matrix [0][1] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [0][2] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [0][3] Bit | 87 | 60 | 92 | 94 | 87 | 92 |
| Matrix [0][4] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [0][5] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][0] Bit | 262 | 208 | 334 | 271 | 208 | 262 |
| Matrix [1][1] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][2] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][3] Bit | 188 | 126 | 236 | 250 | 126 | 188 |
| Matrix [1][4] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][5] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Total Bit | 641 | 482 | 782 | 729 | 525 | 662 |
| MaxError | 30 | 30 | 85 | 47 | 62 | 96 |
| AvgError | 4.11 | 5.23 | 18.23 | 6.32 | 17.35 | 34.11 |

Table 1: Symmetry quantization matrices coding results for proposed JCTVC-H0460

| (H0460) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Header Bit | 4 | 4 | 4 | 4 | 4 | 4 |
| Matrix [0][0] Bit | 126 | 98 | 152 | 160 | 126 | 152 |
| Matrix [0][1] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [0][2] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [0][3] Bit | 138 | 88 | 150 | 198 | 138 | 150 |
| Matrix [0][4] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [0][5] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][0] Bit | 430 | 324 | 548 | 640 | 324 | 430 |
| Matrix [1][1] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][2] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][3] Bit | 374 | 184 | 460 | 562 | 184 | 374 |
| Matrix [1][4] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][5] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Total Bit | 1088 | 714 | 1330 | 1580 | 792 | 1126 |
| MaxError | 28 | 21 | 103 | 129 | 55 | 143 |
| AvgError | 4.09 | 3.28 | 21.81 | 18.01 | 15.52 | 31.48 |

Table 2: Asymmetry quantization matrices coding results for proposed JCTVC-H0460

The presented results above matched the results provided by the proponent of JCTVC-H0460.

# Conclusion

It has been verified that the results of “High-level Syntaxes for the Scaling List Matrices Parameters and Parametric coding” provided as JCTVC-H0460 are accurate.