| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H604 |
| --- | --- |

| Title: | Cross-check of JCTVC-H0549 on sub-LCU parallel decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | T. Davies 14 Waterside Drive Langley, SL3 6EZ | Email: | thdavies@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

This document is a cross-check of Qualcomm’s proposal JCTVC-H0549 [1] on sub-LCU parallel decoding. The software provided by the proponent has been reviewed and simulated under common conditions, and the results found to match the proponent’s.

# Introduction

Parallel decoding is possible using various high-level mechanisms specified in the Working Draft (WD) [2], such as tiles, slices, wavefront parallel processing and entropy slices. Once entropy decoding is complete, it is also possible to decode LCU rows in parallel. This requires a delay of 2 LCUs per row in order to meet dependences (intra prediction and MV candidates). The purpose of proposal JCTVC-H0549 is to provide more fine-grained parallelism than this. It is proposed to allow the top half and the bottom half of an LCU row to be processed in parallel after parsing and motion vector prediction. This requires restricting intra prediction within an LCU.

Since intra prediction is applied per-TU, and TUs are limited to 32x32, it is possible to restrict intra-prediction pixel availability per quarter of an LCU. The proposal modifies availability for the bottom-right quarter of an LCU only, which can no longer depend upon the LCU to the above right. If decoders work on 32x32 units, it is claimed that the delay between decoders can be reduced to 1 LCU.

# Software

The software has been reviewed. The change is minimal and agrees with the proposal description, although parallel decoding structures are not implemented.

# Results

The software provided by the proponent was simulated under common conditions [3]. The simulation environment was a homogeneous Intel cluster running Debian Linux. The collated results agreed with those provided by the proponent.

The summary results are shown below. Full results are included in this package. Simulation times are not reliable, due to logging being present on the anchor.

|  | All Intra HE |  |  | All Intra LC |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A (8bit) | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class B | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% |  |  |  |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class F | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
|  | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 85% |  |  | 92% |  |  |  |  |  |
| Dec Time[%] | 93% |  |  | 92% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.0% | -0.1% | 0.1% | 0.1% | -0.1% | 0.0% | 0.2% | 0.7% |
| Class B | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
| Class D | 0.0% | -0.1% | 0.1% | -0.1% | 0.3% | -0.2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.3% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | 0.3% |
| Enc Time[%] | 87% |  |  | 94% |  |  | 94% |  |  |
| Dec Time[%] | 92% |  |  | 94% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | -0.3% | 0.1% |  |  |  |
| Class C | 0.0% | -0.1% | 0.1% | 0.0% | 0.2% | 0.0% |  |  |  |
| Class D | 0.0% | -0.6% | 0.2% | 0.0% | 0.7% | 0.0% |  |  |  |
| Class E | 0.1% | 0.5% | -0.7% | 0.1% | 0.2% | -0.6% |  |  |  |
| Class F | -0.1% | 0.0% | -0.7% | -0.3% | -0.2% | -0.5% |  |  |  |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | -0.1% |  |  |  |
|  | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | -0.1% |  |  |  |
| Enc Time[%] | 91% |  |  | 94% |  |  |  |  |  |
| Dec Time[%] | 95% |  |  | 96% |  |  |  |  |  |

# References

[1]	JCTVC-H0549, “Enabling Sub-LCU level parallel decoding”, S. Kumar, H. -C. Chuang, S. Xiao, X. Wang, W. -J. Chien, M. Karczewicz (Qualcomm), JCT-VC 8th Meeting: San José, CA, USA, 1–10 February, 2012

[2] JCTVC-G1103, “High Efficiency Video Coding (HEVC) text specification Working Draft 5”, B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand (Editors), JCT-VC Meeting, Geneva, CH, 21-30 November, 2011

[3]	JCTVC-G1200, “Common HM test conditions and software reference configurations”, F. Bossen, 7th JCT-VC Meeting, Geneva, CH, 21-30 November, 2011