| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0608 |
| --- | --- |

| Title: | CE3: Cross-check for half-pel Chroma interpolation filter modification (JCTVC-H0494 ) by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Elena Alshina, Alexander Alshin, JeongHoon Park Samsung #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | Elena Alshina +82-10-3026-1305 {elena_a.alshina, alexander_b.alshin, jeonghoon}@samsung.com |
| Source: | Samsung Electronics Co. Ltd.. |  |  |

_____________________________

# Abstract

Test results provided by Motorola in CE3 are verified here. Proponent tested two variant of half-pel Chroma interpolation filter. First provides 0.2% Chroma BR-rate gain. Second variant provides in average 0.1% Chroma BD-rate gain in mandatory tests, but 0.1% loss if we take into account all test video and all test cases.

# Introduction

Core experiment on half-pel Chroma interpolation filter was established based on [1]. Proponent provided 2 variants of half-pel Chroma interpolation filter: variant 1 matches with [1], variant 0 was not mentioned in original contribution [1].. But proponent modified only half-pel Chroma interpolation filter according to description in [2].

Coefficients for tested variants in comparison with current HM5.0 Chroma interpolation filter are listed in Table 1. They both are obtained using DCTIF formula [3] with phase shift =0.5, Mmin=-1, Mmax=2, Size =4 and different smoothing window sizes N.

Let’s assume that we have values in integer positions pl, -MminlMmax and we need to calculate p in fractional position . Filter size is Size = Mmax- Mmin+1. The center of the template is

.

(1)

Here is smoothing-window size, .are boundary of reference pixels template. Window size controls smoothing properties of windowing function.

For real application float-point filter (1) coefficients are scaled by S and rounded. During rounding normalization (filter coefficients sum is equal to S) should be considered

Because of different strength of smoothing frequency responses for HM5.0 and two filters in verified tests are also different. Figure 1 shows frequency responses for both proposed variant in comparison with HM5.0 filter. HM5.0 filter has no amplification of any frequency signal (filter gain doesn’t exceed 1). For both proposed variants “0” and “1” due to interpolation signal will be amplified on certain frequency diapason. It provides sharpening effect but may amplify noise as well.

Table 1. Half -pel 4-taps Chroma filters.

|  | Filter coefficients | N |
| --- | --- | --- |
| HM5.0 | { -4, 36, 36, -4 } | 5.7 |
| Variant 0 | { -5, 37, 37, -5 } | 7.0 |
| Variant 1 | { -7, 39, 39, -7 } |  |

Fig. 1. Frequency responses of various half-pel Chroma interpolation filters.

# Simulation results

Proponent provided s/w before CE3 dead-line and we tested it according to [4]. Summary of our verification tests is presented in Table 2. Both variants provide less than 0.1% Luma BD-rate change. So Chroma BD-rate is reported in Table 2. More details about test data can be found in appendix (Tables 4,5) and excel spreadsheets attached to this contribution.

Due to sharpening effect “variant 1” provides good gain if SAO is enabled, but due to noise amplification with variant shows drop in LC test case. The maximum drop is 16%.

“Variant 0” improves objective Chroma gain in all test cases except LP-LC where small drop comes due to noise amplification.

Table 2. Summary of verified test (average Chroma BD-rate gain is reported).

|  | V0 | V1 | V1(if SAO is ON) |
| --- | --- | --- | --- |
| RA-HE | -0.2% | -0.6% |  |
| LB-HE | -0.3% | -0.8% |  |
| LP-HE | -0.3% | -0.6% |  |
| RA-LC | -0.3% | -0.7% | -0.8% |
| LB-LC | 0.0% | 1.5% | -1.1% |
| LP-LC | 0.1% | 2.8% | -1.0% |
| RA-HE10 | -0.1% | -0.4% |  |
| LB-HE10 | -0.3% | -0.4% |  |
| LP-HE10 | -0.2% | -0.4% |  |
| All Mandatory | -0.2% | -0.1% |  |
| All | -0.2% | 0.1% |  |
| Enc. Time. | 100% | 100% |  |
| Dec. Time | 100% | 100% |  |

The most write column reports performance of V1 Chroma filter if SAO is enabled. This indicates that with and without SAO V1 half-pel Chroma filter provides very different performance: from 2.8% drop to 1.1% Chroma BD-rate gain. Which might be not desirable for application where SAO might be forced by encoder to be switched off.

We found Luma BD-rate variations in average for both verified tests.

# Subjective quality test

We have compared visual quality of reconstructed video of HM5.0 and both proposed variants of half-pel Chroma interpolation filter. Following test cases were compared:

- Vidyo 1, QP=37, LP-LC

- BQTerrace, QP=37, LP-LC

- ChinaSpeed, QP=37, LP-HE

Still images for the last encoded frame are shown in Table 3.

It hard to find difference in visual quality: in some cases HM5.0 filter provides the best quality, in other cases suggested modification is better.

Table 3. Fragment of still images (last frames) for three tested filters.

| Variant 1 | Variant 0 | Anchor |
| --- | --- | --- |

# Conclusions

Samsung confirms performance reported by Motorola in CE3 test on half-pel Chroma interpolation filter.

# References

- J. Lou, K. Minoo, L. Wang, A. Luthra (Motorola Mobility), “On Chroma interpolation filter,” Document of Joint Collaborative Team on Video Coding, JCTVC-G698, November 2011.

- T. Chujoh, E. Alshina, “Description of Core Experiment 3 (CE3): Chroma Interpolation Filter,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1203, November 2011

- E. Alshina ,A. Alshin “CE3: DCT derived interpolation filter test by Samsung.,” Document of Joint Collaborative Team on Video Coding, JCTVC-F247, July, 2011

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, November 2011.

Appendix.

Table 4. Performance test results for variant 0.

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% | 0.0% | -0.3% | -0.3% | 0.0% | -0.1% | -0.2% |
| Class B | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.3% | 0.0% | -0.2% | -0.2% |
| Class C | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | -0.4% | -0.5% | 0.0% | -0.3% | -0.6% | 0.0% | -0.1% | -0.3% |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.1% | 0.0% | 0.0% |
| Overall | 0.0% | -0.2% | -0.2% | 0.0% | -0.3% | -0.3% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.2% | -0.2% | 0.0% | -0.3% | -0.3% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | -0.2% | 0.0% | -0.3% | 0.4% | 0.0% | -0.4% | -0.4% |
| Class C | -0.1% | -0.3% | -0.2% | 0.0% | -0.1% | -0.3% | 0.0% | -0.4% | -0.3% |
| Class D | 0.0% | -0.8% | -0.6% | -0.1% | -0.1% | -0.2% | 0.0% | -0.6% | -1.0% |
| Class E | 0.0% | 0.2% | -0.1% | 0.0% | 0.5% | 0.9% | 0.0% | 0.0% | 0.6% |
| Class F | -0.3% | -0.3% | -0.4% | -0.1% | 0.0% | -0.4% | -0.1% | 0.0% | -0.4% |
| Overall | -0.1% | -0.3% | -0.3% | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | -0.3% |
|  | -0.1% | -0.4% | -0.3% | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | -0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -0.4% | -0.4% | 0.0% | 0.4% | 0.0% | 0.0% | -0.3% | -0.6% |
| Class C | 0.0% | -0.3% | -0.5% | 0.0% | 0.1% | 0.2% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | -0.6% | -0.7% | -0.1% | -0.4% | 0.0% | 0.0% | -0.7% | -0.7% |
| Class E | 0.0% | 0.3% | 0.0% | 0.0% | 0.4% | 0.3% | 0.0% | -0.2% | 0.5% |
| Class F | 0.0% | 0.1% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% |
| Overall | 0.0% | -0.2% | -0.3% | 0.0% | 0.1% | 0.1% | 0.0% | -0.3% | -0.2% |
|  | 0.0% | -0.2% | -0.3% | 0.0% | 0.1% | 0.1% | 0.0% | -0.3% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  | 100% |  |  |

Table 5. Performance test results for variant 1.

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.6% | -0.6% | 0.0% | -1.0% | -0.8% | 0.0% | -0.4% | -0.4% |
| Class B | 0.0% | -0.7% | -0.7% | 0.0% | -0.8% | -0.7% | 0.0% | -0.4% | -0.5% |
| Class C | 0.0% | -0.4% | -0.5% | 0.0% | -0.6% | -0.7% | 0.0% | -0.4% | -0.3% |
| Class D | 0.0% | -0.7% | -1.0% | 0.0% | -0.9% | -1.1% | 0.0% | -0.4% | -0.5% |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | -0.1% | -0.2% | -0.2% | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% |
| Overall | 0.0% | -0.5% | -0.6% | 0.0% | -0.7% | -0.7% | 0.0% | -0.4% | -0.4% |
|  | 0.0% | -0.5% | -0.6% | 0.0% | -0.7% | -0.7% | 0.0% | -0.4% | -0.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -1.1% | -0.9% | 0.0% | 2.1% | 2.4% | 0.0% | -0.8% | -0.5% |
| Class C | 0.0% | -0.9% | -0.9% | 0.0% | 0.7% | 0.7% | 0.0% | -0.6% | -0.5% |
| Class D | 0.0% | -1.5% | -1.5% | 0.0% | 1.8% | 2.7% | -0.1% | -0.5% | -1.5% |
| Class E | 0.1% | 0.5% | 0.2% | 0.0% | 3.2% | 2.9% | 0.1% | 0.5% | 1.2% |
| Class F | -0.1% | -0.4% | -0.5% | -0.1% | -1.1% | -0.5% | -0.1% | -0.1% | -0.3% |
| Overall | 0.0% | -0.8% | -0.8% | 0.0% | 1.3% | 1.6% | 0.0% | -0.4% | -0.4% |
|  | 0.0% | -0.8% | -0.8% | 0.0% | 1.3% | 1.5% | 0.0% | -0.3% | -0.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -0.9% | -1.1% | 0.0% | 3.6% | 3.4% | 0.0% | -0.8% | -0.7% |
| Class C | 0.0% | -0.6% | -0.9% | 0.0% | 1.7% | 2.3% | 0.0% | -0.5% | -0.4% |
| Class D | 0.0% | -0.6% | -1.8% | 0.0% | 3.1% | 5.0% | 0.0% | -0.7% | -1.1% |
| Class E | 0.0% | -0.2% | 0.7% | 0.1% | 4.8% | 3.4% | 0.0% | 0.8% | 0.8% |
| Class F | 0.0% | 0.3% | -0.4% | 0.1% | 0.7% | 0.1% | -0.1% | 0.1% | 0.1% |
| Overall | 0.0% | -0.4% | -0.8% | 0.0% | 2.7% | 2.8% | 0.0% | -0.3% | -0.3% |
|  | 0.0% | -0.4% | -0.8% | 0.0% | 2.6% | 2.7% | 0.0% | -0.4% | -0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |