| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0610 |
| --- | --- |

| Title: | Non-CE3: Cross-check for phase adjustment in interpolation filter (JCTVC-H0180) by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Elena Alshina, Alexander Alshin, JeongHoon Park Samsung #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | Elena Alshina +82-10-3026-1305 {elena_a.alshina, alexander_b.alshin, jeonghoon}@samsung.com |
| Source: | Samsung Electronics Co. Ltd.. |  |  |

_____________________________

# Abstract

Test results provided by Toshiba are verified here. Proponent proposed to modify interpolation filter coefficients for Luma and Chroma in order to remove phase shift mismatch appeared due to rounding filter coefficients. Proposed modification doesn’t lead to performance improvement: in average 0.3% (mandatory tests)/ 0.1% (all tests) Luma BD-rate drop. Statistically modified filter reduces hit-ratio of fractional MVs.

# Introduction

Interpolation filter coefficients in HM were calculated using DCTIF formula [1]. Let’s assume that we have values in integer positions pl, -MminlMmax and we need to calculate p in fractional position . Filter size is Size = Mmax- Mmin+1. The center of the template is

.

(1)

Here is smoothing-window size, .are boundary of reference pixels template. Window size controls smoothing properties of windowing function.

For real application float-point filter (1) coefficients are scaled by S and rounded. During rounding normalization (filter coefficients sum is equal to S) should be considered. Different variant of rounding are possible. In [2] alternative rounding for filter coefficients is tested for 1/4-pel Luma and 3/8-pel Chroma filters. Table 1 summarizes modification for filter coefficients. Figures 1 and 2 show frequency responses for modified filters compare to HM5.0.

Because of different smoothing frequency responses are also different. Figure 1 shows frequency responses for both proposed variant in comparison with HM5.0 filter. HM5.0 filter has no amplification of any frequency signal (filter gain doesn’t exceed 1). For both proposed variants “0” and “1” due to interpolation signal will be amplified on certain frequency diapason. It provides sharpening effect but may amplify noise as well.

Table 1. Modified coefficients of Interpolation filter.

| HM5.0 | In verified test |
| --- | --- |
| { -1, 4, -10, 58, 17, -5, 1, 0 } | { -1, 4, -10, 57, 18, -5, 1, 0 } |
| { -6, 46, 28, -4 }, | { -6, 48, 26, -4 }, |

| Fig. 1. Frequency responses in amplitude (a) and phase (b) domains for 1/4 7 taps Luma filter. |  |
| --- | --- |

| Fig. 2. Frequency responses in amplitude and phase domain for 3/84 taps Chroma filter. |  |
| --- | --- |

From these graphs we conclude that for both Luma 1/4-pel filter and 3/8 Chroma filter proponent resolves mismatch between theoretical and actual phase shift. From view point of error accumulation or de-noising effect both HM5.0 and filters in verified tests should have almost equal effect (Chroma filter may have higher error accumulation compare to HM5.0, but difference is very small).

# Simulation results

Proponent provided s/w for cross-check in time enough for analysis. We tested it according to [3]. Summary of our verification tests is presented in Table 2.

Table 2. Summary of verified test (average Luma BD-rate gain is reported).

| Test case | Luma BD-rate | Chroma BD-rate |
| --- | --- | --- |
| RA-HE | 0.1% | -0.1% |
| LB-HE | 0.2% | 0.0% |
| LP-HE | 0.2% | 0.0% |
| RA-LC | 0.3% | 0.0% |
| LB-LC | 0.5% | 0.0% |
| LP-LC | -1.2% | -0.1% |
| RA-HE10 | 0.2% | 0.1% |
| LB-HE10 | 0.2% | -0.1% |
| LP-HE10 | 0.2% | 0.0% |
| All Mandatory | 0.3% | 0.1% |
| All | 0.1% | 0.0% |
| Enc. Time. | 100% | 100% |
| Dec. Time | 100% | 100% |

More details can be found in Table 3 (appendix) and excel spread-sheet attached to this report. Additionally to PSNR and bit-rate we report statistical number of arithmetical operations per 1 pixel motion compensation and memory access measurement according to [4].

# Conclusions

Samsung confirms performance reported by Toshiba in non-CE3 contribution on phase adjustment for interpolation filter:

0.3% (in average for mandatory tests) / 0.1% (in averaged for all tests) Luma BD-rate.

# References

- E. Alshina ,A. Alshin “CE3: DCT derived interpolation filter test by Samsung.,” Document of Joint Collaborative Team on Video Coding, JCTVC-F247, July, 2011

- T. Chujoh, T. Yamakage “Non-CE3: Phase adjustment of interpolation filter,” Document of Joint Collaborative Team on Video Coding, JCTVC-H0180, February, 2012

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, November 2011.

- T. Chujoh, E.Alshina “CE3: Motion Compensation,” Document of Joint Collaborative Team on Video Coding, JCTVC-F903, July, 2011

Appendix.

Table 3. Performance test results.

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | -0.3% | -0.1% | -0.5% | -0.3% | 0.0% | -0.1% | -0.1% |
| Class B | 0.1% | -0.1% | -0.1% | 0.2% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% |
| Class C | 0.2% | 0.0% | 0.0% | 0.4% | 0.1% | 0.0% | 0.3% | 0.1% | 0.0% |
| Class D | 0.4% | 0.0% | 0.1% | 0.9% | 0.3% | 0.3% | 0.4% | 0.3% | 0.5% |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Overall | 0.1% | -0.1% | -0.1% | 0.3% | 0.0% | 0.0% | 0.2% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |
| Mults | -2% |  |  | -2% |  |  | -2% |  |  |
| Adds | -2% |  |  | -2% |  |  | -2% |  |  |
| MemAccess | -1% |  |  | -1% |  |  | -1% |  |  |
| Avg. MemBand2D | 0% |  |  | -1% |  |  | 0% |  |  |
| Avg. MemBand1D | 0% |  |  | -1% |  |  | 0% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.1% | 0.0% | 0.4% | 0.3% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class C | 0.2% | -0.1% | 0.0% | 0.5% | -0.1% | 0.1% | 0.2% | 0.0% | 0.0% |
| Class D | 0.5% | 0.2% | 0.6% | 1.0% | 0.8% | 0.4% | 0.3% | -0.6% | 0.1% |
| Class E | 0.2% | 0.0% | 0.0% | 0.4% | 0.5% | 0.5% | 0.1% | -0.4% | 0.2% |
| Class F | -0.1% | -0.3% | -0.6% | 0.1% | -1.1% | -0.7% | 0.1% | -0.2% | -0.2% |
| Overall | 0.2% | -0.1% | 0.1% | 0.5% | 0.0% | 0.0% | 0.2% | -0.2% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 101% |  |  |
| Mults | -2% |  |  | -2% |  |  | -2% |  |  |
| Adds | -2% |  |  | -2% |  |  | -2% |  |  |
| MemAccess | -1% |  |  | -1% |  |  | -1% |  |  |
| Avg. MemBand2D | -1% |  |  | -1% |  |  | -1% |  |  |
| Avg. MemBand1D | -1% |  |  | -1% |  |  | 0% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.1% | 0.2% | -0.1% | -2.0% | 0.3% | -0.2% | 0.1% | 0.1% | 0.1% |
| Class C | 0.4% | 0.0% | -0.2% | -0.8% | -0.4% | 0.1% | 0.3% | -0.1% | 0.2% |
| Class D | 0.5% | 0.3% | 0.0% | -0.3% | -0.3% | 1.4% | 0.5% | -0.1% | 0.5% |
| Class E | 0.1% | 0.3% | -0.3% | -3.4% | -0.6% | -0.5% | 0.1% | -0.2% | -0.5% |
| Class F | 0.0% | -0.2% | 0.0% | 0.1% | -0.8% | -0.1% | -0.1% | 0.0% | -0.1% |
| Overall | 0.2% | 0.1% | -0.1% | -1.2% | -0.3% | 0.2% | 0.2% | -0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  | 101% |  |  |
| Mults | -2% |  |  | -1% |  |  | -2% |  |  |
| Adds | -2% |  |  | -1% |  |  | -2% |  |  |
| MemAccess | -1% |  |  | -1% |  |  | -1% |  |  |
| Avg. MemBand2D | 0% |  |  | 0% |  |  | -1% |  |  |
| Avg. MemBand1D | 0% |  |  | 0% |  |  | 0% |  |  |