| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0613 |
| --- | --- |

| Title: | Non-CE6: Cross-verification of Samsung’s DC mode as a default mode (JCTVC-H0242) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shigeru Fukushima Toru Kumakura JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6597 fukushima.shigeru@jvckenwood.com kumakura.toru@jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of DC mode as a default mode (JCTVC-H0242) by Samsung. It is confirmed that the BD-rate results match those provided by the proponent.

# Introduction

This contribution reports cross-verification results of DC mode as a default mode (JCTVC-H0242) by Samsung [1].

# Algorithm Description

The variables intraPredModeN are set equal to DC mode instead of  planar mode if neighboring block is not available or not coded as intra or in above current LCU. The time complexity for DC mode is less than for planar mode because planar mode has pixel-by-pixel process. Therefore the average time complexity can be reduced.

The derivation process of the variables candIntraPredModeN are not changed, thus candIntraPredModeN are set equal to planar mode if intraPredModeN are greater than or equal to intraPredModeNum.

Only setting following macro for HM 5.0, above algorithm is able to be implemented.

#define PLANAR_IS_DEFAULT	0

# Experiments

## Test conditions

The common test conditions [2] are used.

## Experimental environment

The experimental environment used is presented in Table1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table2, Table3, and Table4 present the simulation results for intra only, random access, and low delay B respectively.

Table 2 Simulation results for intra only

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class E | 0.1% | -0.4% | -0.3% | 0.1% | -0.3% | -0.3% |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class F | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3 Simulation results for random access

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | -0.2% | 0.1% | 0.0% | -0.4% | 0.1% | 0.0% | 0.4% |
| Class B | 0.0% | -0.2% | -0.1% | 0.1% | 0.0% | -0.2% | 0.1% | -0.1% | -0.1% |
| Class C | 0.0% | -0.2% | 0.0% | 0.1% | 0.0% | 0.0% |  |  |  |
| Class D | 0.1% | -0.2% | 0.1% | 0.0% | 0.4% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.2% | -0.1% | 0.0% | 0.1% | -0.1% | 0.1% | -0.1% | 0.1% |
|  | 0.0% | -0.2% | 0.0% | 0.0% | 0.1% | -0.2% | 0.1% | -0.1% | 0.2% |
| Class F | -0.1% | -0.1% | -0.2% | 0.0% | -0.2% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 100% |  |  |

Table 4 Simulation results for low delay B

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.1% | 0.0% | -0.3% | -0.4% | -0.1% |
| Class C | 0.0% | -0.1% | 0.1% | -0.1% | 0.0% | -0.2% |
| Class D | 0.0% | -0.4% | -0.2% | -0.1% | 0.9% | 0.2% |
| Class E | 0.1% | 0.3% | -0.4% | -0.2% | 0.0% | -0.2% |
| Overall | 0.0% | -0.1% | -0.1% | -0.2% | 0.1% | 0.0% |
|  | 0.0% | -0.1% | -0.1% | -0.2% | 0.1% | 0.0% |
| Class F | -0.1% | 0.0% | -0.6% | 0.0% | -0.5% | -0.3% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Conclusions

It is confirmed that the BD-rate results match those provided by the proponent.

# References

[1] “DC mode as a default intra mode”,

JCTVC-H0242, San José, February 2012

[2] “Common test conditions and software reference configurations”,

JCTVC-G1200, Geneva, November 2011

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).