| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, USA, 1-10 February, 2012 | Document: JCTVC-H0615 |
| --- | --- |

| Title: | Cross-check of JCTVC-H0526 on multi-level significance maps for small TU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yinji Piao Elena Alshina JeongHoon Park | Email: | yj1003.piao@samsung.com elena_a.alshina@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of multi-level significance map for 8x8 block described in JCTVC-H0526[1]. Same concept of significance group flag in large transform unit [2] is extended to 8x8 block. The software implementation is consistent with the algorithm description and the crosscheck results match the results from proponent.

# Code-level check

The proposal was implemented on top of HM5.0. RIM_MLS_STU is used for the proposed modification. Look-up table with size of 4x4 is introduced for sub-block scan order in case of horizontal, vertical, and diagonal scans. Contexts of significance group flag in large transform are shared in 8x8 TU. Additionally, bin counting in decode side is also available in the same source code which counts bin of significance group flag, significance flag and total bins for significance map coding.

# Test Results

The tests were carried out under the test conditions specified in the JCTVC-G1200[3]. S/w provided by proponent was compiled on 64-bit Linux and executed on Linux cluster. The summary results are presented in following tables, and more details can be found in accompanying excel sheet.

Table I. Test results in HM5.0 under common test condition

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.2% | -0.1% | -0.2% | 0.1% | -0.1% | -0.3% |  |  |  | 0.2% | -0.3% | -0.1% |
| Class B | 0.3% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | -0.1% | -0.1% |
| Class C | 0.3% | 0.0% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
| Class D | 0.2% | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% | 0.0% | -0.2% | 0.1% |  |  |  |
| Class E | 0.6% | -0.1% | -0.1% |  |  |  | 0.1% | -0.2% | -0.9% |  |  |  |
| Overall | 0.3% | 0.0% | 0.0% | 0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.2% | -0.2% | -0.1% |
| Enc Time | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time | 99% |  |  | 99% |  |  | 99% |  |  | 100% |  |  |
|  | All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  |  |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |  |  |  |
| Class A | 0.2% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class B | 0.2% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | -0.4% | 0.3% |  |  |  |
| Class C | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | -0.2% |  |  |  |
| Class D | 0.1% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | -0.1% | 0.4% | -0.6% |  |  |  |
| Class E | 0.4% | 0.1% | 0.1% |  |  |  | 0.1% | -1.6% | 0.2% |  |  |  |
| Overall | 0.2% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.3% | -0.1% |  |  |  |
| Enc Time | 100% |  |  | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time | 98% |  |  | 98% |  |  | 98% |  |  |  |  |  |

# Conclusions

Test results from JCTVC-H0526 are verified in this report. The simulation results matched the data provided by the proponents.

# References

- JCT-VC, “Multi-level Significance Maps for Small Transform Units”, JCTVC-H0526, 8th JCT-VC Meeting, San José, USA, 1-10 Feb., 2012

- JCT-VC, “Multi-level significance maps for large transform units”, JCTVC-G644, 7th JCT-VC Meeting, Geneva, CH, 21-30 Nov, 2011

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-G1200, 7th JCT-VC Meeting, Geneva, CH, 21-30 Nov, 2011