| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, USA, 1-10 February, 2012 | Document: JCTVC-H0616 |
| --- | --- |

| Title: | CE11: Cross-check of subtest 3.1.2 (JCTVC-H0282) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yinji Piao JeongHoon Park | Email: | yj1003.piao@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports cross-verification result of significance flag coding described in JCTVC-H0282 [1] with updated initialization parameter. The software implementation is consistent with the algorithm description and the crosscheck results match the results from proponent.

# Code-level check

The proposal was implemented on top of HM5.0. G448_RED_SIG_CXTS was used for the proposed modification. Contexts of high frequency position in 4x4 and 8x8 TU were grouped into one context both in luma and chroma of 4x4 and 8x8 blocks. The result reported in following section is implemented with new initialization values provided by the proponent.

# Test Results

The tests were carried out under the test conditions specified in the JCTVC-G1200[2]. S/w provided by proponent was compiled on 64-bit Linux and executed on Linux cluster. The summary results are presented in following tables, and more details can be found in accompanying excel sheet.

Table I. Test results in HM5.0 under common test condition

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  | 0.1% | 0.2% | 0.4% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | 0.3% | 0.0% | 0.1% | 0.0% |
| Class C | 0.1% | 0.0% | 0.1% | 0.0% | -0.1% | 0.1% | 0.1% | -0.1% | 0.2% |  |  |  |
| Class D | 0.1% | 0.1% | 0.1% | 0.0% | -0.2% | 0.2% | 0.1% | 0.0% | 0.4% |  |  |  |
| Class E | 0.0% | 0.1% | 0.1% |  |  |  | 0.0% | -0.3% | -0.7% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.2% |
| Enc Time | 100% |  |  | 100% |  |  | 99% |  |  | 100% |  |  |
| Dec Time | 100% |  |  | 99% |  |  | 98% |  |  | 100% |  |  |
|  | All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  |  |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |  |  |  |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | -0.1% | 0.5% |  |  |  |
| Class C | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.2% |  |  |  |
| Class D | 0.1% | 0.0% | 0.0% | 0.0% | 0.5% | 0.1% | 0.0% | 0.6% | 0.4% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% |  |  |  | -0.1% | -1.5% | -0.1% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | -0.1% | 0.3% |  |  |  |
| Enc Time | 99% |  |  | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time | 100% |  |  | 100% |  |  | 100% |  |  |  |  |  |

Table II. Test results in HM5.0 with LowQP

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.0% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.3% | 0.2% | 0.1% | 0.2% | 0.3% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.1% | 0.3% | 0.4% | 0.1% | 0.5% | 0.5% |  |  |  |
| Class E | 0.0% | 0.1% | 0.1% |  |  |  | 0.1% | 0.2% | 0.9% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.2% | 0.1% | 0.2% | 0.4% | 0.0% | 0.1% | 0.0% |
| Enc Time | 99% |  |  | 98% |  |  | 97% |  |  | 101% |  |  |
| Dec Time | 96% |  |  | 98% |  |  | 96% |  |  | 101% |  |  |
|  | All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  |  |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |  |  |  |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.3% | 0.1% | 0.2% | 0.2% |  |  |  |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.4% | 0.4% | 0.2% | 0.6% | 0.6% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.1% | 0.6% | 0.6% | 0.1% | 1.0% | 0.9% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% |  |  |  | 0.0% | 0.6% | 0.7% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.1% | 0.3% | 0.3% | 0.1% | 0.6% | 0.6% |  |  |  |
| Enc Time | 97% |  |  | 97% |  |  | 100% |  |  |  |  |  |
| Dec Time | 98% |  |  | 97% |  |  | 98% |  |  |  |  |  |

# Conclusions

Test results of JCTVC-H0282 with updated initialization parameter are verified in this report. The simulation results matched the data provided by the proponents.

# References

- JCT-VC, “CE11.1.2: Simplified method for coding 4x4 & 8x8 significance maps”, JCTVC-H0282, 8th JCT-VC Meeting, San José, USA, 1-10 Feb., 2012

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-G1200, 7th JCT-VC Meeting, Geneva, CH, 21-30 Nov, 2011