| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0618 WG11 Number: m |
| --- | --- |

| Title: | Cross-verification of Ericsson's proposal on deblocking filter simplifications (JCTVC-H0473) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Seungwook Park, Byeongmoon Jeon 221, Yangjae-Dong, Seocho-Gu, Seoul, Korea | Tel: Email: | +81-10-3922-5896 seungwook.park@ lge.com bm.jeon@lge.com |
| Source: | LG |  |  |

_____________________________

# Abstract

This documents reports the crosscheck results of JCTVC-H0473 on BS and deblock filtering simplifications submitted by Ericsson. It is reported that the results match those provided by Ericsson exactly and there is no running time increase compared with HM4.0.

# Introduction

In JCTVC-H0473 [1], BS and deblock filtering simplification is proposed by Ericsson. In the current HM, the Bs is determined for the 8-pel boundary as a maximum of two Bs values of the two 4-pel parts of the boundary even though filtering process is performed in 4-pel boundary. So this proposal suggests removing this maximum operation and using separate Bs value for each of the two 4-pel parts instead. And the same value of “d” in the strong filtering decision, as in the weak filtering decision is proposed also.

According to the investigation on the source code, the proposed method is well implemented so that it is well readable. In this crosscheck, we compare the results we got with Ericsson’s software and the results provided by Ericsson. They are exactly the same.

# Experimental results

Our experimental results match those provided by Ericsson exactly (refer to Table 1).

Table 1.  Test results

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | -0.2% | -0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  | 99% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.2% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.3% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | -0.5% | 0.0% | 0.0% | 0.5% | 0.0% |
| Class E | 0.0% | 0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| Overall | 0.0% | -0.2% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | -0.2% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class F | -0.1% | -0.2% | -0.6% | -0.1% | -0.1% | -0.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.3% | 0.0% | 0.1% | -0.4% |
| Class C | 0.0% | 0.0% | -0.2% | 0.0% | -0.1% | 0.2% |
| Class D | 0.0% | -0.3% | -0.5% | 0.0% | -0.2% | 0.7% |
| Class E | 0.0% | 0.2% | 0.1% | 0.0% | -0.2% | -0.6% |
| Class F | 0.0% | 0.2% | 0.0% | 0.1% | -0.2% | 0.5% |
| Overall | 0.0% | 0.0% | -0.2% | 0.0% | -0.1% | 0.1% |
|  | 0.0% | 0.0% | -0.2% | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 92% |  |  | 95% |  |  |

# Conclusions

The source codes are well implemented following the proposed method and built properly under any OS platform. The results provided by Ericsson are matched exactly with this test. So we confirmed that JCTVC-H0473 has no problem in source code and the results.

# References

- A. Norkin “CE10.3: Deblocking filter simplifications: Bs computation and strong filtering decision”, JCTVC-H0473, San José, Feb. 2012.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200, JCT-VC Meeting, Geneva, November. 2011.