| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1-10 February, 2012 | Document: JCTVC-H0619 WG11 Number: m23518 |
| --- | --- |

| Title: | Non-CE11: Cross-check report for JCTVC-H0285 on Block-based significance map context selection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Hisao Sasai Kengo Terada Takahiro Nishi 1006 Kadoma, Kadoma, Osaka, Japan | Tel: Email: | +81 6 6900 9689 sasai.hisao@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is a report of cross-check result for MediaTek's proposal in JCTVC-H0285. The experimental results perfectly match with the one provided by proponent in the rate-distortion performance. It is also confirmed that the software is matched with the algorithm description provided by proponent.

# Source code check

In the proposal[1], the following two modifications are introduced:

A) Block-based significance map context selection

B) Inferred significant_coeffgroup_flag for region-1 subblocks

The following MACRO setting on provided codes is used for cross-check.

#define DC_BLOCK_CTX_SET
This macro is to change the region boundary from a diagonal line to sub-blocks.

#define DC_BLOCK_INFERRED_SG_FLAG

This macro is to unify the significant_coeffgroup_flag coding with A).

# Simulation Results

## Experimental Condition

The software to be tested was provided by proponent and its performance relative to HMv5 software was checked under the common test condition described in JCTVC-G1200 [2]. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3.

## Results

The following two tests are conducted as described in JCTVC-H0285.

TEST1: for A)

#define DC_BLOCK_CTX_SET		1

#define DC_BLOCK_INFERRED_SG_FLAG	0

TEST2: for A) and B)

#define DC_BLOCK_CTX_SET		1

#define DC_BLOCK_INFERRED_SG_FLAG	1

BD BR and execution time for TEST1 on common test condition is shown in Table 1.

Table 1: TEST 1 Experimental results on the common test condition

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class F | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | -0.1% | 0.0% | 0.2% | 0.8% |
| Class B | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
| Class D | 0.0% | -0.2% | 0.0% | 0.0% | 0.4% | -0.3% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.1% | 0.1% | 0.3% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | -0.1% | 0.2% | 0.1% | 0.4% |
| Class F | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | -0.1% | 0.0% | -0.2% | 0.4% |  |  |  |
| Class C | 0.0% | 0.1% | 0.2% | 0.0% | 0.3% | 0.1% |  |  |  |
| Class D | 0.0% | -0.3% | 0.2% | 0.0% | 0.7% | -0.3% |  |  |  |
| Class E | 0.0% | 0.5% | -0.9% | 0.0% | -0.9% | 0.2% |  |  |  |
| Overall | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
|  | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.2% |  |  |  |
| Class F | -0.2% | 0.0% | -0.3% | 0.1% | -0.7% | -0.3% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

BD BR and execution time for TEST2 on common test condition is shown in Table 2.

Table 2: TEST 2 Experimental results on the common test condition

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.3% |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | -0.1% | 0.1% | 0.5% | 0.9% |
| Class B | 0.1% | 0.1% | 0.3% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.1% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | 0.0% |  |  |  |
| Class D | 0.0% | -0.2% | 0.2% | 0.0% | 0.5% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.3% | 0.0% | 0.1% | 0.2% | 0.5% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.3% | -0.1% | 0.2% | 0.2% | 0.5% |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | 0.4% | 0.1% | -0.1% | 0.2% |  |  |  |
| Class C | 0.1% | -0.1% | -0.1% | 0.0% | 0.1% | 0.2% |  |  |  |
| Class D | 0.0% | -0.2% | -0.4% | 0.0% | 0.8% | 0.4% |  |  |  |
| Class E | 0.1% | 0.7% | -0.2% | 0.0% | -0.6% | -0.8% |  |  |  |
| Overall | 0.1% | 0.1% | -0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
|  | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class F | 0.0% | 0.1% | -0.4% | 0.1% | -0.6% | -0.2% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

# Conclusion

The results provided by the proponent for " Block-based significance map context selection " were confirmed. Their first method is also proposed by JCTVC-H0290[3] and JCTVC-H0432[4]. It is recommended to discuss with other related contributions.

# References

- T-D. Chuang et al., “Non-CE11: Block-based significance map context selection”, JCTVC-H0285, San Jose, February 2012.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200, Geneva, November 2011.

- K. Terada et al., “Non-CE11:Simplification of context selection for significant_coeff_flag”, JCTVC-H0290, San Jose, February 2012.

- T. Ji et al., “Sub-Block Based Significance Map Region Classification”, JCTVC-H0432, San Jose, February 2012.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).