| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0622 WG11 Number: m23521 |
| --- | --- |

| Title: | CE8.f: Crosscheck report on objective evaluation of visual testing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ohji Nakagami Masaru Ikeda Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 Ohji.Nakagami@jp.sony.com Masaru.Ikeda@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports objective cross-check results of CE8.f visual test (JCTVC-G1208 [1]). It is reported that the bitrate of bitstreams generated in the CE8.f test condition are verified. Also, the BD-bit rate results of post-filter ALF processing, which source code is provided by TI, are stated. It is reported that both cross-check results match the results provided by JCTVC-HXXXX.

# Introduction

In CE8.f, visual test is held to understand of ALF subjective gain (JCTVC-G1208 [1]). This contribution reports objective cross-check results of the bitrate of the generated bitstream in the CE8.f condition. In addition, it is reported that the BD-bit rate results of postfilter ALF provided by TI.

# Result

The bitrate of the CE8.f condition is reportedly shown in Table 1 and 2. The BD-bit rate results of postfilter ALF provided by TI are reportedly shown in Table3.

Table 1 Verification results of bitrate for CE8.f visual test

| Sequence | Class | QP | Coding config | Bitrate |  | Percentage Difference (%) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ALF ON | ALF Off |  |
| BasketballDrive | B | 32 | LB | 3150.01 | 3164.24 | 0.5% |
|  |  | 37 | LB | 1608.99 | 1611.41 | 0.2% |
| Cactus | B | 32 | LP | 2665.96 | 2657.03 | -0.3% |
|  |  | 37 | LP | 1311.12 | 1303.20 | -0.6% |
| Kimono | B | 32 | LP | 1177.35 | 1183.68 | 0.5% |
|  |  | 37 | LP | 573.75 | 578.08 | 0.8% |
| BasketballDrill | C | 32 | RA | 825.75 | 817.28 | -1.0% |
|  |  | 37 | RA | 436.87 | 433.08 | -0.9% |
| RaceHorses | C | 32 | LP | 1020.78 | 1019.07 | -0.2% |
|  |  | 37 | LP | 462.77 | 460.81 | -0.4% |
| Vidyo1 | E | 32 | LB | 359.1312 | 355.1344 | -1.1% |
|  |  | 37 | LB | 189.9432 | 188.3352 | -0.8% |
| Riverbed | 1080p | 32 | LB | 6046.5856 | 6033.3408 | -0.2% |
|  |  | 37 | LB | 2930.2464 | 2926.9696 | -0.1% |
| SpinCalendar | 720p | 32 | LB | 807.2099 | 814.6403 | 0.9% |
|  |  | 37 | LB | 388.8419 | 384.247 | -1.2% |

Table 2 Verification results of bitrate for CE8.f visual test

| Sequence | Class | QP | Coding config | Bitrate |  | Percentage Difference (%) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ALF ON | ALF-Post |  |
| BasketballDrive | B | 32 | LB | 3150.01 | 3205.19 | 1.8% |
|  |  | 37 | LB | 1608.99 | 1638.01 | 1.8% |
| Cactus | B | 32 | LP | 2665.96 | 2696.69 | 1.2% |
|  |  | 37 | LP | 1311.12 | 1327.66 | 1.3% |
| Kimono | B | 32 | LP | 1177.35 | 1195.64 | 1.6% |
|  |  | 37 | LP | 573.75 | 584.76 | 1.9% |
| BasketballDrill | C | 32 | RA | 825.75 | 834.91 | 1.1% |
|  |  | 37 | RA | 436.87 | 444.49 | 1.7% |
| RaceHorses | C | 32 | LP | 1020.78 | 1029.34 | 0.8% |
|  |  | 37 | LP | 462.77 | 467.54 | 1.0% |
| Vidyo1 | E | 32 | LB | 359.1312 | 373.3160 | 3.9% |
|  |  | 37 | LB | 189.9432 | 198.5416 | 4.5% |
| Riverbed | 1080p | 32 | LB | 6046.59 | 6047.82 | 0.0% |
|  |  | 37 | LB | 2930.25 | 2936.18 | 0.2% |
| SpinCalendar | 720p | 32 | LB | 807.21 | 852.03 | 5.6% |
|  |  | 37 | LB | 388.84 | 410.65 | 5.6% |

Table 3 Verification results of postALF on common test condition

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.4% | -0.5% | -0.6% |
| Class B | 1.0% | -0.3% | -0.6% |
| Class C | 1.2% | -0.6% | -0.2% |
| Class D | 1.7% | 0.5% | 0.5% |
| Class E |  |  |  |
| Class F | 1.1% | 0.8% | 0.6% |
| Overall | 1.2% | 0.0% | 0.0% |
|  | 1.2% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 110% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 1.2% | -0.1% | -0.3% |
| Class C | 2.1% | 0.0% | 0.3% |
| Class D | 2.2% | 1.0% | 2.0% |
| Class E | 4.3% | 2.1% | -1.6% |
| Class F | 3.3% | 1.8% | 1.7% |
| Overall | 2.5% | 0.8% | 0.5% |
|  | 2.5% | 0.8% | 0.5% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 115% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 2.6% | 0.8% | 0.5% |
| Class C | 2.1% | 0.1% | -0.1% |
| Class D | 1.3% | 1.1% | 1.3% |
| Class E | 5.4% | 2.2% | -1.2% |
| Class F | 3.2% | 2.2% | 2.1% |
| Overall | 2.8% | 1.2% | 0.6% |
|  | 2.8% | 1.3% | 0.6% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 119% |  |  |

# Conclusions

In this contribution, the cross-check results on the bitstream used in CE8.f visual test regarding ALF filter are reported. It is confirmed that the results match the ones provided by JCTVC-HXXXX.

# References

- JCT-VC, “Description of Core Experiment 8 (CE8): Non-deblocking loop filtering”, JCTVC-G1208, 7th. JCT-VC Meeting, Geneva, CH, November, 2011

- “”, JCTVC-HXXXX, San José, February, 2012.