| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0623 |
| --- | --- |

| Title: | AHG15: Crosscheck of JCTVC-H0269 on independent control of Max CU and LCU size for small LCU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yongjoon Jeon, Byeongmoon Jeon #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: | +82-10-9043-9419 yongjoon.jeon@lge.com bm.jeon@lge.com |
| Source: | LG |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results for JCTVC-H0269 on independent control of MaxCU and LCU size for small LCU. This crosscheck verifies the case when LCU size is equal to 64x64 and Max_CU_size is equal to 32 only and the case when Max_CU_size is equal to 16 is not verified. The verification work was performed using the software provided by the proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. It has been confirmed that objective R-D performance produced in this work perfectly matches the results from the proponent.

# Verification

This cross-verification work was conducted using modified HM5.0 software provided by the proponent. The code was successfully compiled and run on the following platform : Intel Xeon5570@2.93GHz 64 bits Linux machines.

# Analysis

In the current HM, the maximum CU size is identical to LCU size. In the proposed method of JCTVC-H0269 [1], the maximum CU size (Max_CU_size) and LCU size can be set differently : Max_CU_size is smaller than or equal to LCU size. To allow this, the following syntax element (log2_diff_lcu_max_coding_block_size) is proposed and signalled in the SPS.

Table 1. Proposed syntax in JCTVC-H0269

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| log2_diff_lcu_max_coding_block_size | ue(v) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |

With the proposed syntax, Max_CU_size and LCU size are derived as shown in Table 2. Table 2 and 3 shows the difference between the proposed method of JCTVC-H0269 and the current HM.

Table 2. Derivation of log2(Max_CU_size) and log2(LCU_size) in JCTVC-H0269

log2(Max_CU_size) = (log2_min_coding_block_size_minus3 + log2_diff_max_min_coding_block_size +3) log2(LCU_size) = log2(Max_CU_size) + log2_diff_lcu_max_coding_block_size

Table 3. Derivation of log2(Max_CU_size) and log2(LCU_size) in HM5.0

log2(LCU_size) = log2(Max_CU_size) =(log2_min_coding_block_size_minus3 + log2_diff_max_min_coding_block_size +3)

When the value of log2_diff_lcu_max_coding_block_size is equal to 0, the proposed method works the same as the current HM except signalling log2_diff_lcu_max_coding_block_size in the SPS. Fig. 1 (a) (b) illustrates this case. Fig. 1 (c) and (d) illustrates the case when the value of the syntax is greater than 0. In this example, the value is equal to 1. As can be seen from Fig. 1 (d), the only difference to the current HM according to the proposed method is the processing unit order.

Figure 1. (a)LCU size 64x64 in HM5.0 (c) LCU size 32x32 in HM 5.0 vs (b) LCU size and Max CU size are equal to 64x64 in proposed one (d) LCU size and Max CU size different in proposed one

For simulation, the common test condition and reference configuration of [2] are followed except the following changes (only the yellow marked ones are the changes to the configuration of [2]).

Reference

MaxCUWidth : 32 # Maximum coding unit width in pixel MaxCUHeight : 32 # Maximum coding unit height in pixel MaxPartitionDepth : 3 # Maximum coding unit depth

Prop.

MaxCUWidth : 64 # Maximum coding unit width in pixel MaxCUHeight : 64 # Maximum coding unit height in pixel MaxPartitionDepth : 4 # Maximum coding unit depth MinPartitionDepth : 1 # Minimum coding unit depth

Table 4 shows the results when LCU size is 64x64 and Max_CU_size is 32.

Table4. LCU size is 64x64 and Max_CU_size = 32 vs  LCU size =32 in HM5.0

All the results perfectly match the results from the proponent.

# Conclusion

This contribution reports cross-verification results for JCTVC-H0269 on independent control of MaxCU and LCU size for small LCU. It has been confirmed that all the results perfectly match the results from the proponent. Similar tendency on encoding and decoding time was also observed. JCTVC-F0269 focuses on the case of small LCU size and the proposed method provides coding efficiency benefit in this case.

# References

[1] J. Choi, J. Kim, K. Lee, J. Park, C. Kim, “AHG15 : Independent control of Max CU and LCU size for small LCU”, JCT-VC Document, JCTVC-H0269, 8th Meeting: San José, CA, USA, 1–10 February, 2012.

[2]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-G1200, 7th Meeting: Geneva, CH, 21-30 November, 2011.