| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0629 |
| --- | --- |

| Title: | Non-CE9: Cross-check of the throughput improvement for merge/skip mode (JCTVC-H0240) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Patrice ONNO Guillaume LAROCHE Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | patrice.onno@crf.canon.fr guillaume.laroche@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This document is a crosscheck report of the contribution JCTVC-H0240 about the throughput improvement for merge/skip mode. Three experiments were cross-checked in that report corresponding to one solution with two alternatives: “SOL”, “VAR1”, “VAR2”. The results provided here perfectly match those provided by ETRI and Kyung Hee University in document JCTVC-H0240.

# Introduction

In the current HM5.0 design, there is sequential dependency between PUs since the spatial position for TMVP reference index derivation could be overlapped with another PU. This sequential dependency between PUs prohibits parallel merge estimation PU0 and PU1, and hence makes it very difficult to meet high-throughput.

In JCTVC-H0240, the proponents propose a CU-based approach for MCL (Merge Candidate List) construction, where all the PUs inside a CU share a single MCL, which is the 2Nx2N PU’s. JCTVC-H0240 also presents two differently restricted versions of the proposed solution: Variant 1 only removes inter-PU dependency in TMVP reference index derivation by sharing a TMVP refIdx position of 2Nx2N PU with all the other PU partitions. Variant 2 applies the proposed solution only for 8x8 CUs, which is the worst case.

- Code verification

ETRI has provided a modified source code based on HM5.0. The source code was compiled successfully by using Linux 2.6.18-194.el5, gcc version 4.1.2 20080704 (Red Hat 4.1.2-48) 64 bits.

# Results

Experiments were conducted under the five conditions (RA-HE, RA-LC, RA-HE10, LB-HE, LB-LC) as specified in common test conditions [2].

## Results for the “proposed solution”

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.3% | 0.3% | 0.3% | 0.3% | 0.2% | 0.2% | 0.3% | 0.3% | 0.2% |
| Class B | 0.2% | 0.1% | 0.2% | 0.1% | 0.2% | 0.1% | 0.2% | 0.2% | 0.2% |
| Class C | 0.3% | 0.2% | 0.3% | 0.3% | 0.2% | 0.3% |  |  |  |
| Class D | 0.3% | 0.4% | 0.2% | 0.3% | 0.3% | 0.3% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.3% | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.3% | 0.2% |
|  | 0.3% | 0.2% | 0.3% | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% |
| Class F | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.4% | 0.4% | 0.5% | 0.3% | 0.1% | 0.2% |  |  |  |
| Class C | 0.5% | 0.1% | 0.5% | 0.4% | 0.6% | 0.3% |  |  |  |
| Class D | 0.5% | 0.2% | 0.1% | 0.4% | 0.6% | 0.4% |  |  |  |
| Class E | 1.0% | 0.5% | 0.5% | 0.7% | 0.5% | 1.2% |  |  |  |
| Overall | 0.5% | 0.3% | 0.4% | 0.4% | 0.4% | 0.5% |  |  |  |
|  | 0.5% | 0.4% | 0.4% | 0.4% | 0.5% | 0.5% |  |  |  |
| Class F | 0.3% | 0.2% | 0.0% | 0.2% | -0.7% | -0.3% |  |  |  |
| Enc Time[%] | 100% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |  |  |  |

## Results for the alternative solution “Variant 1”

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | -0.2% | 0.1% | 0.0% | 0.2% | 0.0% |  |  |  |
| Class D | 0.0% | 0.1% | 0.2% | 0.0% | 0.3% | 0.0% |  |  |  |
| Class E | 0.1% | -0.2% | -0.3% | 0.0% | -0.3% | -0.1% |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% |  |  |  |
| Class F | -0.1% | -0.2% | -0.5% | -0.2% | -0.5% | -0.7% |  |  |  |
| Enc Time[%] | 100% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |  |  |  |

## Results for the alternative solution “Variant 2”

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.0% | 0.0% | 0.2% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% |  |  |  |
| Class D | 0.2% | 0.0% | 0.1% | 0.2% | 0.2% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.0% |
|  | 0.1% | 0.0% | 0.0% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 99% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% |  |  |  |
| Class C | 0.2% | 0.1% | 0.3% | 0.3% | 0.5% | 0.3% |  |  |  |
| Class D | 0.2% | 0.0% | -0.1% | 0.3% | 0.6% | 0.2% |  |  |  |
| Class E | 0.2% | -0.4% | 0.0% | 0.3% | 0.0% | 0.1% |  |  |  |
| Overall | 0.2% | 0.0% | 0.1% | 0.2% | 0.3% | 0.1% |  |  |  |
|  | 0.2% | 0.0% | 0.0% | 0.2% | 0.3% | 0.2% |  |  |  |
| Class F | -0.1% | 0.0% | -0.4% | 0.1% | -0.3% | -0.3% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  |  |  |  |

# Conclusion

The results of experiments “proposed solution”, “Variant 1” and “Variant 2” presented by ETRI and Kyung Hee University in JCTVC-H0240[1] have been confirmed by this cross-check.

# References

- Hui Yong Kim, Kyung Yong Kim, Sang Min Kim, Gwang Hoon Park, Seunghyun Cho, Sung-Chang Lim, Jinho Lee, and Jin Soo Choi, “ Non-CE9: Throughput improvement for merge/skip mode”, Document of Joint Collaborative Team on Video Coding, JCTVC-H0240, Jan. 2012

- F. Bossen, "Common HM test conditions and software reference configurations", Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, Nov. 2011.