| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, 1-10 February, 2012 | Document: JCTVC-H0633 |
| --- | --- |

| Title: | Cross-verification of Simplified coeff_abs_level_minus3 coding from Motorola Mobility (JCTVC-H0498) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-845-7532 liweig@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution verified the BD-rate results of JCTVC-H0498 which is proposed by Motorola Mobility.

# Introduction

In JCTVC-H0633, a method has been proposed to change the truncation point of Golomb-Rice code for the coding of coeff_abs_level_minus3. With the proposed change, the truncated Rice codewords can be limited to 8 bits. In this method, the number of Rice code parameters is kept to be 4 (4Parameters). This method is further combined with the method in JCTVC-H0497 (5Parameters).

We have reviewed the codes and confirmed the implementation matches the description in JCTVC-H0498.

# Experimental results

The codebase is HM 5.0 for normal QP test and HM5.1rc2 for low QP (2, 7, 12, 17) test. The coding efficiency is evaluated under common conditions as specified in [1]. The coding efficiency is measured by using 4 point BD-Rate. The encoding time and decoding time are obtained from cluster.

Test results of 5Parameters under normal QP  Test results of 4Parameters under normal QP

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 97% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 105% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.2% | 0.0% | 0.1% | -0.1% | 0.1% | 0.0% | 0.6% |
| Class B | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| Class D | 0.0% | -0.1% | 0.2% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 98% |  |  | 98% |  |  | 93% |  |  |
| Dec Time[%] | 102% |  |  | 98% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | 0.2% |  |  |  |
| Class C | 0.0% | -0.1% | 0.1% | 0.1% | 0.1% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | -0.2% | -0.1% | 0.6% | 0.4% |  |  |  |
| Class E | 0.1% | -0.9% | -0.2% | -0.1% | 0.2% | 0.2% |  |  |  |
| Class F | -0.2% | -0.1% | -0.5% | 0.1% | 0.0% | -0.2% |  |  |  |
| Overall | 0.0% | -0.2% | -0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
|  | 0.0% | -0.2% | -0.2% | 0.0% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 97% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |  |  |  |

Test results of 5Parameters under normal QPI  Test results of 4Parameters under low QP

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class C | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% |  |  |  |
| Class D | 0.3% | 0.2% | 0.2% | 0.3% | 0.2% | 0.2% |  |  |  |
| Class E | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Overall | 0.2% | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% |  |  |  |
|  | 0.2% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% |  |  |  |
| Class F | 0.5% | 0.5% | 0.5% | 0.5% | 0.4% | 0.4% |  |  |  |
| Enc Time[%] | 122% |  |  | 112% |  |  |  |  |  |
| Dec Time[%] | #NUM! |  |  | 97% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.3% | 0.3% | 0.2% | 0.4% | 0.4% | 0.4% |  |  |  |
| Enc Time[%] | 118% |  |  | 107% |  |  | 105% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% |  |  |  |
| Enc Time[%] | 117% |  |  | 107% |  |  |  |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |  |  |  |

Test results of 5Parameters under normal QP  Test results of 5Parameters under normal QP

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 97% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | -0.2% | 0.1% |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| Class D | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | -0.1% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 95% |  |  | 94% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.4% | 0.0% | -0.1% | 0.0% |  |  |  |
| Class C | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class D | -0.1% | -0.2% | -0.3% | 0.0% | 0.4% | 0.3% |  |  |  |
| Class E | 0.1% | -0.5% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| Class F | -0.2% | 0.1% | -0.3% | 0.2% | 0.2% | -0.3% |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
|  | 0.0% | -0.2% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 96% |  |  | 97% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |  |  |  |

Test results of 5Parameters under normal QPV  Test results of 5Parameters under low QP

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| Class C | -0.3% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% |  |  |  |
| Class D | -0.5% | -0.4% | -0.4% | -0.5% | -0.3% | -0.3% |  |  |  |
| Class E | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| Overall | -0.2% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% |  |  |  |
|  | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |  |  |  |
| Class F | -1.8% | -1.7% | -1.7% | -1.8% | -1.4% | -1.4% |  |  |  |
| Enc Time[%] | 126% |  |  | 109% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | -1.1% | -0.9% | -0.9% | -1.1% | -0.8% | -0.8% |  |  |  |
| Enc Time[%] | 114% |  |  | 112% |  |  | 114% |  |  |
| Dec Time[%] | #NUM! |  |  | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | -0.4% | -0.4% | -0.4% | -0.5% | -0.3% | -0.4% |  |  |  |
| Enc Time[%] | 112% |  |  | 108% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

# Conclusions

This contribution reports the cross-verification result of JCTVC-0498.The BD-rate exactly matches the data reported by the proponent.

# References

- F.Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200, JCT-VC Meeting, Geneva, Switzerland, Nov. 2011.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).