| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0635 |
| --- | --- |

| Title: | Cross verification of unified design on parallel merge/skip with reduced candidates (JCTVC-H0247) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chanyul Kim | Email: | dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Cross-check on 4 tests of JCTVC-H0247

In this document, a cross-checking report on 4 tests of JCTVC-H0247 [1] is provided. Each test is conducted to verify effect according to parallel merge level. Parallel merge level is set to 1, 2, 3 and for each test.

Software was provided by the proponent. It is confirmed that the source code has modified according to the description and the results under common test condition [2] are same with the results of the contribution as summarized in Table 1, Table 2, Table 3 and Table 4.

Loss of parallel merge level 1 is 0.2% for class A to E in average without encoding and decoding time change. Gain of class F is 0.1% in average. Loss of parallel merge level 2 is 0.4% for class A to E in average without encoding and decoding time change. Gain of class F is 0.4% in average. Loss of parallel merge level 3 is x.x% for class A to E in average without encoding and decoding time change. Gain of class F is x.x% in average. [Results of this test will be added]. Loss of parallel merge level 4 is 2.1% for class A to E in average without encoding and decoding time change. Gain of class F is 1.7% in average.

It is recommended to discuss this contribution together with similar approaches in this field since there are several similar approached..

Table 1. Summary result for log2_parallel_merge_level_minus2 = 0

Table 2. Summary result for log2_parallel_merge_level_minus2 = 1

Table 3. Summary result for log2_parallel_merge_level_minus2 = 3

Table 4. Summary result for log2_parallel_merge_level_minus2 = 4

# References

- H. Y. Kim, K. Y. Kim, Y. Jeon, B. Jeon, M. Zhou, X. Wen and O. C. Au, “AHG10: Unified design on parallel merge/skip with reduced candidates”, Document of Joint Collaborative Team on Video Coding, JCTVC-H0247, San Jose, Feb. 2012

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, Geneva, Nov. 2011.