| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0637r3 |
| --- | --- |

| Title: | Profiles and Levels in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ajay Luthra 6420 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 858 404 3470 aluthra@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

This contribution suggests looking not only at the past but also to the future while specifying the Profiles and the Levels in the HEVC standard. In the past, Profiles and Levels were specified with the point-of-views that were dominated by the facts that large numbers of applications were using one way communication (e.g. Broadcast TV) and that large number of systems had fixed and similar range of processing powers. The ever increasing curve of processing power at acceptable cost was largely ignored. This led to defining fixed Profiles and Levels. That is so 20th century concept and it is suggested to revisit that. While fixed Profiles and Levels should be specified, a mechanism should also be developed such that the clients and the servers can negotiate the Profiles and Levels related attributes. That mechanism could be based on a client sending an xml document describing its capabilities at a particular time.

Furthermore, considering that there will be multiple video coding standards implemented in most of the systems, the concentric structure (also known as Onion structure) of the definitions of Profiles should not be force fitted.  Also, it is suggested to revisit the naming convention for the Profiles and only one profile, called Profile A, should be defined at this stage with focus on high efficiency.

# Introduction

The HEVC (High Efficiency Video Coding) standard consists of various coding tools. HEVC is expected to continue to add more coding tools corresponding to higher bit depth, scalability etc. To promote interoperability Profiles and Levels are defined in the standard corresponding to the needs of various sets of applications. However, in the past it was done with point-of-views that were dominated by the facts that large numbers of applications were using one way communication (e.g. Broadcast TV). In addition, the ever increasing curve of the processing power at acceptable cost was ignored. Furthermore, it was also not taken into consideration that there will be multiple video coding standards. Looking forward, there is a need to take a new look at the Profiles and Levels approaches so that they are consistent with the past thinking as well as the future.

# Profiles

## Naming convention

In the past profiles have been given the names like Baseline, Main, High etc. Those names, with time, create wrong impression. The word like “Main” gives the impression that was the main set of applications the standard was focusing on. That with time proves to be wrong, e.g. High Profile in AVC is not any less important than Main profile, or Baseline Profile is not any less important than Main and was not the focus of the standard. Furthermore, the word like “High” creates more confusion as it suggests that that profile is highly more complex. The word like Simple in the name is equally confusing and becomes troublesome with time, e.g. in MPEG-4 Part 2 the name Advanced Simple Profile had to be created to provide a more efficient version of the Simple profile and Advanced Simple Profile name has been quite confusing in the market place. So, it is suggested to instead use the letters, like Profile A, B, C … or some other neutral names that do not wrongly suggest the attributes and importance of the profiles

## Moving from “Onion” structure to “Garlic” structure

Profiles in a video coding standard were first created in the MPEG-2/H.262 standard. That time the view was that there will only be one video coding standard so it was important to create a Profile structure that consisted of concentric profiles (commonly also called as “onion” structure). Thinking was that it would promote cross application-set interoperability. However, for cross application-set interoperability one needs system layer interoperability as well. In addition, as the existence of multiple coding standards in the same devices and systems has proven that if cross application-set interoperability is needed, multiple non-concentric profiles based system can be efficiently and cost effectively designed. I.e. if some systems need two different profiles then they will implement both the profiles. So, distorting the definitions of Profiles to fit the onion structure model is not worth the trouble. AVC standard started to move in that direction and in HEVC standard we should completely avoid artificially forcing the onion (concentric) structure model and develop the structure that allows different profiles to co-reside without one being forced to be a subset of other (hence, the name Garlic structure).

## Low delay and Random Access

The low delay and random accesses are fundamentally encoder based decisions and we should not define two different profiles, one for low delay and other for random access.

## High efficiency and low efficiency (sometimes called low complexity)

At this stage we should define only one profile corresponding to high efficiency. For the systems wanting lower complexity, hence lower efficiency, the AVC standard can be used and the low complexity related profile should be specified only when its needs and parameters are well understood.

## Error resilience

It is clear that, unlike past, there will be digital TV distribution on the channels that are not practically error free. And, there will also be digital TV distribution on the channels that are practically error free. Two different profiles should be specified for those significantly different environments. However, it is suggested to avoid defining profiles for distribution over the channels with large errors until the characteristics of those channels and the tools needed for distribution over those channels are well understood.

## Profile ‘A’ for HEVC

As mentioned above, define only one profile (suggested name: Profile A) at this stage corresponding to the highest coding efficiency. Revisit and see what is the complexity associated with supporting higher precision/bit depth and consider including that in this profile.

# Levels

## Defining only the fixed Levels is so 20th century

Similar to the thinking behind the specification of the Profiles, the Levels have been defined with primarily one-way systems as well very small set of systems with similar processing power in mind. In attempt to take a baby step towards the needs of the systems with wide range of processing power, AVC defined multiple of levels. That has made life difficult for both – the systems with fixed processing power and the systems with processing power that increases with time as well as may vary as a function of what other things are being processed. In HEVC, we should split up the two worlds. For the systems with known and fixed processing power and/or the ones with only one way communication, let us define discrete Levels but reduce the total number of levels. At the same time, let us start developing mechanism that allows more flexibility in selecting parameters associated with Levels and allows a client and a server to negotiate those values.

In the fixed Levels, to align with the Levels in AVC, it will be important to keep the number 3 corresponding to SDTV/VGA and the number 4 corresponding to HDTV. This will help in avoiding confusion between the levels in AVC and HEVC.

### Fixed Levels

### Suggested levels are:

Level 3: Support 720x480 at 30 frames per sec, 720x576 at 25 frames per sec, 640x480 at 30 frames per sec, 960x540 (QHD) at 30 frames per sec

Level 4: Support 1280x720 at 60 frames per sec, 1920x1080 at 30 frame per sec

Level 5: Support 1920x1080 at 60 frames per sec

Level 6: Support 4kx2k at 60 frames per sec

### Flexible Levels

It is suggested to work with the Systems experts to define and specify Level related negotiations that can happen between servers and clients based on, for example, xml documents describing clients’ capabilities (which could also be time varying). That would allow better flexibility to adjust resolutions, bit rates, delays etc., based on the two way communication, to have more efficient systems.

HEVC should standardize the names of the various parameters associated with the Levels whose values can be modified.

# Flexible Profiles

## Defining only the fixed Profiles is so 20th century

It is suggested to work with the Systems experts to define and specify coding tools related negotiations that can happen between servers and clients. So, for example, a client may be able to negotiate the use of less complex tool to be able to receive higher resolution video.

## XML based flexible Profiles (and Levels)

As an example, today servers and clients can convey the Profiles and Levels as discussed in RFC6381 (http://tools.ietf.org/html/rfc6381 ; co-authored by R. Gellens, Qualcomm Inc.; D. Singer, Apple Inc. and P. Frojdh, Ericsson AB, August 2011). A client and server can negotiate the codec types and associated profiles (e.g. AVC Main Profile or others) by exchanging the xml documents.

In these types of mechanisms, additional information and/or flags can be inserted to convey the capabilities that do not fall in the Profiles and/or Levels specified in the standard. For example, if an AVC Constrained Baseline Profile compliant receiver is capable of also supporting B-pictures then it can convey that information in the XML format and request the bit stream that is or can be compressed using that (Bi-prediction) coding tool. Today, there is no such profile (Constrained Baseline plus B-pictures) in the AVC standard and this limits the users from providing a more efficient compression that is better suited to a client’s capabilities. This mechanism can also be used to convey that a client (or server) does not support all the tools specified in a profile. For example, a client may support all the tools of the Main Profile of AVC standard but not the CABAC. There is no such profile as “Main minus CABAC”.

At a high level, there can be two methods for conveying the tools that are supported by a client (or a server) that do not fall with-in the fixed Profiles specified in the standard:

- By communicating the complete list of the coding tools, or

- By communicating only additional or less tools supported, in comparison to the profiles defined in the standard.

This concept can also be extended to SEI messages. For example, if a client supports a feature associated with certain SEIs, it may convey that by requesting corresponding SEIs. Similarly, if a client wants to convey explicitly to not use certain SEI messages, that can also be negotiated. For example, if a client does not want to receive interlaced scanned video, it can inform the server to not send picture structure SEI associated with interlace video. This way server can potentially take action to convert interlaced video to progressive video before sending it to that client.

This concept is also easily extensible to the information related to Levels also.

## What is needed in HEVC standard

HEVC should standardize the names of the coding tools that can be added or subtracted from the Profiles specified in the standard. If there are certain attributes associated with those coding tools that can also be modified then those should also be specified (for example, in a B-picture the maximum motion range may be allowed to be smaller). This would allow a client to convey the limitations, if any, associated with that attribute of the coding tool as well.

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).