| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H643 |
| --- | --- |

| Title: | Cross-check of weighted prediction precision reduction (JCTVC-H0546) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results on weighted prediction precision proposed in JCTVC-H0546. In this method, the input precision to the weighted prediction module is decreased by 4 bits. The verification task has been done successfully and the results match those provided by proponent. The encoding time and the decoding time of this cross-check are also similar to those provided by proponents.

Experimental results show that BD-bitrate gain of the weighted prediction precision reduction is 0.0%/0.0% and 0.0%/0.0% for black/white-fade sequences on RA-LC and LB-LC cases, respectively, and 0.0%/0.0% and 0.0%/0.1% for black/white-fade sequences on RA-HE and LB-HE cases, respectively.

Although cross-checking results match the proponent’s one, there are some concerns about the consistency with current WD test.

# Cross-checking

The software based on HM version 5.0 and the experimental results were provided on Jan. 26, 2011 by Qualcomm. The software was complied fine and the cross-checking was performed on 64bit environments. The simulation has been conducted under common test conditions [1] for low complexity (LC) conditions and high efficiency (HE) conditions in the case of random access (RA), low delay B (LB) and low delay P (LP) coding structures using black/white-fade sequences provided by [2]. Proponents add only one macro as follows:

#define QC_WP_PREC 4

The value of “4” means 4-bit decrease for input value of weighted sample prediction process. We reviewed the source code and confirmed that this modification of source code in original HM-5.0 software matches those described in JCTVC-H0564 [3], however, the modified WD text in JCTVC-H0546 is not based on the latest one (JCTVC-G1103_d9.doc). Then the equations for weighted prediction are incorrect.

In current design of HM, the high precision motion compensation (MC) technique proposed in JCTVC-F537 [4] was adopted. The intermediate value of MC holds 16-bit memory (the internal calculation is 32-bit precision). At the last stage of MC, the intermediate value is rounded by the source bit depth in the default weighted sample prediction process. The weighted prediction (WP) utilizes also the same concept as current MC process. The input value for WP holds 16-bit memory (no additional memory), same as the value of MC before rounding. The internal calculation of WP is 24-bit for worst case and the predicted value is rounded by the source bit depth in the weighted sample prediction process. Therefore, cross-checkers are wondering that there are any merits of 4 bit decrease for the internal calculation on the weighted sample prediction process or not, because it still needs 32-bit internal calculation.

# Results

Table 1 and Table 2 show BD-bitrate and relative encoding/decoding time of this scheme for black-fade and white-fade sequences respectively. Through the results, it turns out that the loss is negligible and the encoding/decoding time does not change compared with WP anchor. The test results for cross-checking match exactly those provided by proponents.

Table 1. BD-Rate[%] and relative encoding/decoding time[%] (Black-fade sequences)

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.3% | 0.1% | 0.0% | -0.3% | -0.1% | 0.0% | 0.6% | 0.6% |
| Class B | 0.1% | -0.2% | -0.4% | 0.0% | -0.1% | -1.2% | 0.0% | 0.3% | 0.5% |
| Class C | 0.0% | 0.0% | -0.2% | 0.0% | -0.2% | 0.3% |  |  |  |
| Class D | 0.0% | -0.9% | 0.0% | 0.0% | -0.1% | 0.3% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | -0.2% | 0.0% | 0.4% | 0.5% |
|  | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | -0.3% | 0.0% | 0.4% | 0.6% |
| Class F |  |  |  |  |  |  |  |  |  |
| Enc Time[%] | 100.39% |  |  | 100.33% |  |  | 100.37% |  |  |
| Dec Time[%] | 100.76% |  |  | 100.79% |  |  | 100.42% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | -0.8% | 0.1% | 0.1% | -2.9% |  |  |  |
| Class C | 0.0% | 0.0% | -0.3% | -0.1% | -0.2% | -0.1% |  |  |  |
| Class D | -0.1% | -0.6% | -0.9% | -0.1% | -0.9% | -0.1% |  |  |  |
| Class E | -0.1% | 1.0% | -1.0% | 0.2% | -1.3% | -1.0% |  |  |  |
| Overall | 0.0% | 0.0% | -0.7% | 0.0% | -0.5% | -1.2% |  |  |  |
|  | 0.0% | 0.0% | -0.7% | 0.0% | -0.4% | -1.2% |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |
| Enc Time[%] | 100.37% |  |  | 100.43% |  |  |  |  |  |
| Dec Time[%] | 99.81% |  |  | 101.26% |  |  |  |  |  |

Table 2. BD-Rate[%] and relative encoding/decoding time[%] (White-fade sequences)

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.1% | 0.3% | 0.0% | 0.1% | -0.2% | 0.0% | 1.0% | 0.6% |
| Class B | 0.0% | -0.1% | 0.1% | 0.1% | -0.1% | -1.0% | 0.0% | 0.3% | 0.3% |
| Class C | 0.0% | -0.1% | -0.4% | 0.1% | -0.2% | -0.1% |  |  |  |
| Class D | 0.2% | -0.5% | 0.0% | 0.0% | -0.1% | 0.3% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.3% | 0.0% | 0.6% | 0.5% |
|  | 0.0% | -0.2% | 0.0% | 0.0% | -0.2% | -0.2% | 0.0% | 0.6% | 0.5% |
| Class F |  |  |  |  |  |  |  |  |  |
| Enc Time[%] | 100.33% |  |  | 100.40% |  |  | 100.43% |  |  |
| Dec Time[%] | 100.68% |  |  | 100.73% |  |  | 100.84% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | -1.0% | 0.0% | 0.6% | -3.2% |  |  |  |
| Class C | 0.1% | -0.1% | 0.3% | 0.1% | -0.3% | -0.2% |  |  |  |
| Class D | 0.0% | -0.1% | -0.4% | -0.1% | -0.6% | -0.3% |  |  |  |
| Class E | 0.2% | 0.0% | 0.2% | 0.2% | -0.3% | -1.8% |  |  |  |
| Overall | 0.1% | -0.1% | -0.3% | 0.0% | -0.1% | -1.4% |  |  |  |
|  | 0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -1.5% |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |
| Enc Time[%] | 100.41% |  |  | 100.38% |  |  |  |  |  |
| Dec Time[%] | 100.26% |  |  | 100.76% |  |  |  |  |  |

# References

[1]	F. Bossen ," Common test conditions and software reference configurations ", Joint Collaborative Team on Video Coding, JCTVC-FG1200, Geneva, Nov. 2011.

[2]	AHG report, " JCT-VC AHG report: Weighted prediction (AHG 18)," Joint Collaborative Team on Video Coding, JCTVC-F018, Torino, July 2011.

[3]	M. Coban and M. Karczewics, "On weighted prediction precision," Joint Collaborative Team on Video Coding, JCTVC-H0546, San Jose, Feb. 2012.

[4]	F. Bossen , " On the motion compensation process," Joint Collaborative Team on Video Coding, JCTVC-F537, Torino, July 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).