| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0644 |
| --- | --- |

| Title: | Non-CE10: Cross-verification report for difference-QP-based deblocking (JCTVC-H0190) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hirofumi Aoki Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5438 hao@bu.jp.nec.com chono@ct.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

This contribution presents a study and cross-verification results of a modified strength decision method for deblocking filtering under varying QPs from CU to CU, which is presented in JCTVC-H0190. The cross-verification tasks were successfully completed and it was confirmed that the implementation conforms to the description in a revised version of JCTVC-H0190 (JCTVC-H0190r1) and results exactly match those presented by the proponent. It is recommended that the issue raised in JCTVC-H0190 be discussed at the 8th JCT-VC meeting.

# Analysis

JCTVC-H0190 addresses the issue that blocking artifacts are noticeable on CU boundaries where the gap of QPs is large. In JCTVC-H0190, it is proposed to apply the strong filter when the gap is sufficiently large and the QP values are larger than a certain threshold.

The software was provided by the proponent and it was successfully compiled by Microsoft Visual C++ 2008. It was confirmed that the software implementation conforms to the description in a revised version of JCTVC-H0190 (JCTVC-H0190r1). The implementation is simple and straightforward, and there have been no problems found in the software.

It is recommended that this issue be discussed at the 8th JCT-VC meeting.

# Simulation results

The experiment to verify simulation results provided by the proponent was conducted under the CE4 Subtest 1 test conditions described in JCTVC-G1204 [4]. The computing platform used for the verification is shown below. MinCUDQPSize was set to 8x8, which is the mandatory condition in CE4 Subtest1. Evaluation was made in terms of coding efficiency and complexity compared with CE4 test 1.1, which is the anchor in CE4 Subtest 1.

| Computing platform | Usage |
| --- | --- |
| OS: Windows 7 64-bit Professional CPU: Xeon X5680 3.33GHz (12 cores) Memory: 32GiB | Up to 12 processes for encoding and decoding ran at the same time. |

Tables 1 show the summary of results. An attached spreadsheet contains the detailed results. It was confirmed that BD-Rate values are exactly same as those provided by the proponent and that encoding/decoding time increases are negligible.

Table 1: Summary results of test 1 of JCTVC-H0190 against test 1.1 of CE4 (anchor).

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.5% | 0.0% | 0.0% | 0.6% | 0.0% | 0.0% |  |  |  |
| Class B | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |  |  |  |
| Class C | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |  |  |  |
| Class D | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |  |  |  |
| Class E | 0.2% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |  |  |  |
| Overall | 0.2% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |  |  |  |
|  | 0.2% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |  |  |  |
| Class F | 0.4% | 0.0% | 0.0% | 0.5% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.4% | 0.1% | 0.1% | 0.3% | 0.0% | 0.0% | 0.3% | 0.2% | 0.1% |
| Class B | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class C | 0.1% | 0.0% | -0.1% | 0.1% | 0.1% | 0.0% |  |  |  |
| Class D | 0.1% | -0.1% | -0.1% | 0.1% | 0.1% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.2% | 0.1% | 0.1% |
|  | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.2% | 0.1% | 0.0% |
| Class F | 0.3% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.1% | 0.2% | 0.3% | 0.1% | 0.0% | 0.1% |  |  |  |
| Class C | 0.1% | 0.1% | 0.1% | 0.0% | -0.2% | 0.4% |  |  |  |
| Class D | 0.1% | 0.1% | -0.2% | 0.1% | -0.1% | -0.6% |  |  |  |
| Class E | 0.3% | 0.3% | 0.1% | 0.1% | 0.4% | 0.2% |  |  |  |
| Overall | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.0% |  |  |  |
|  | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.0% |  |  |  |
| Class F | 0.3% | 0.1% | 0.2% | 0.2% | 0.4% | 0.7% |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |  |  |  |

# Conclusion

This contribution has presented a study and cross-verification results of a strength decision method for deblocking filtering under varying-QP conditions, which is presented in JCTVC-H0190. The cross-verification tasks were successfully completed and it was confirmed that the implementation conforms to the description in a revised version of JCTVC-H0190 (JCTVC-H0190r1) and results exactly match those presented by the proponent. It is recommended that the issue raised in JCTVC-H0190 be discussed at the 8th JCT-VC meeting.

# References

[1] K. Sato, “Non-CE10: Difference-QP-based Deblocking,” JCT-VC document JCTVC-H0190, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 8th meeting, San Jose, February 2012.

[2] K. Sato, et al., “Description of Core Experiment 4: Quantization,” JCT-VC document JCTVC-G1204, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 7th meeting, Geneva, November 2011.