| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0648 |
| --- | --- |

| Title: | AHG9: Cross-check result of MediaTek's CABAC with constrained outstanding bits (JCTVC-H0271) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

This document reports the cross-check result of JCTVC-H0271 "CABAC with constrained outstanding bits) proposed by MediaTek.

The result is identical to the result reported by the proponent.

# Cross-checked scheme

JCTVC-H0271[1] proposes to modify the arithmetic decoding process in order to remove the problem in the arithmetic encoding process.

The renormalization process in the arithmetic encoding process introduces one variable bitsOutstanding to cope with the case when the value of an output bit is not determined (i.e. outstanding). The variable bitsOutstanding increases when the coding interval is across the middle range (i.e. 512) during the renormalization process, and reset to zero in the other case.

Theoretically, the bounds of the latency (i.e. the number bitsOutstanding) between the input of a bin and the output of the corresponding bit(s) is infinite. This nature makes the design of encoder products difficult since the arithmetic encoding process may output a large number of bits when processing one bin.

JCTVC-H0271 removes the problem by outputting all outstanding bits when it reaches to the threshold value (12 in this experiment). Since it involves the adjustment of the coding interval, the arithmetic decoding process is also changed accordingly.

Specifically, JCTVC-H0271 has two variations. The difference between two methods is in the coding interval adjustment. The first method performs the adjustment by simulating the arithmetic encoding decoding in subclause 9.2.4.2, whereas the second methods performs that in subclause 9.2.4.4 (Bypass encoding) for the purpose of speed-up.

# Results

Table 1 and Table 2 summarize the coding gain in BD-Rate of the proposed scheme (method 1 and method 2). The test condition accords with the condition specified in JCTVC-G1000[2].

Table 1: Summary results of BD-Rate comparing proposal (method 1) to HM5.0

Table 2: Summary results of BD-Rate comparing proposal (method 2) to HM5.0

The result is identical to the result reported from the proponent (MediaTek).

# References

[1] T.-D.Chuang et al., "AHG9: CABAC with constrained outstanding bits", JCTVC-H0271, February 2012, San Jose, USA

[2] "Suggested revised common test conditions", JCTVC-G1000, November 2011, Geneva Switzerland