| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1-10 February, 2012 | Document: JCTVC-H0651 WG11 Number: m |
| --- | --- |

| Title: | CE11: Cross-check report for JCTVC-H0224 on Data Sign Hiding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhan Ma Ankur Saxena Felix C. A. Fernandes 1301 E. Lookout Dr., Richardson TX 75082 | Tel: Email: | +1 972 761 7134 zhan.ma@sta.samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution is a cross-check report for Orange Labs’ proposal JCTVC- H0224. The experimental results perfectly match with the one provided by the proponents in the rate-distortion performance. It is also confirmed that the software is matched with the algorithm description provided by proponents, except two additional variables which were initialized by the cross-checkers to avoid Linux compiler complaints. However, it is observed that the encoding and encoding time measures are slightly different. In H0224 proposal, encoding/decoding time measurements are 103%/99% (AI), 103%/99% (RA) and 103%/100% (LDB) in comparison to 113%/105% (AI), 107%/104% (RA) and 105%/100% (LDB) provided by the cross-checkers.

# Source code check

In the proposal JCTVC-H0224 [1], a method is introduced to embed sign information into the residue. The following MACRO setting on provided codes is used for cross-check.

#define DATA_HIDING 1  // enable data hiding

#define DATA_HIDING_LINUX_COMPILE 1 // avoid linux compiler complains

On top of proponent’s source code, the cross-checkers  initialized two more variables (in function TComTrQuant::xGetCodedLevel() ) to avoid more linux compiler complaints. This may be due to a difference in compiler settings.

#if DATA_HIDING

double dZeroCostSig =0;

double dZeroCost    =0;

#endif

# Simulation Results

## Experimental Conditions

The software to be tested was provided by the proponents, and its performance relative to HM 5.0 software was checked under the common test condition for High Efficiency conditions, described in JCTVC-G1200 [2]. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.1.2.

## Results

BD- Bit Rate and execution times for the cross-check on common test conditions are  shown in Table 1.

Table 1 BD BR and Time measurements

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | -0.5% | -1.0% | -1.0% |
| Class B | -0.6% | -0.7% | -0.6% |
| Class C | -0.6% | -0.9% | -0.8% |
| Class D | -0.7% | -0.9% | -0.9% |
| Class E | -0.3% | -0.9% | -0.7% |
| Overall | -0.6% | -0.8% | -0.8% |
|  | -0.6% | -0.8% | -0.8% |
| Class F | -0.9% | -1.0% | -0.9% |
| Enc Time[%] | 113% |  |  |
| Dec Time[%] | 105% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A (8bit) | -0.4% | -0.6% | -0.9% |
| Class B | -0.5% | -0.5% | -0.5% |
| Class C | -0.5% | -0.4% | -0.4% |
| Class D | -0.5% | -0.7% | -0.1% |
| Class E |  |  |  |
| Overall | -0.5% | -0.5% | -0.4% |
|  | -0.5% | -0.5% | -0.4% |
| Class F | -0.7% | -0.7% | -0.7% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 104% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.7% | -0.4% | 0.0% |
| Class C | -0.7% | -0.7% | -0.4% |
| Class D | -0.7% | -1.0% | -0.5% |
| Class E | -0.2% | -1.3% | -1.0% |
| Overall | -0.6% | -0.8% | -0.4% |
|  | -0.6% | -0.8% | -0.4% |
| Class F | -0.7% | -0.5% | -0.8% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusion

The results provided by the proponents were confirmed with identical R-D performance, and slight different encoding/decoding time measurements.  It is recommended to discuss JCTVC-H0224 with other related contributions.

# References

- Gordon CLARE et al., “CE11: Sign Data Hiding”, JCTVC-H0224, San Jose, February 2012.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200, Geneva, November 2011.