| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0654 |
| --- | --- |

| Title: | Modifications on sequence parameter set RBSP syntax, picture parameter set RBSP syntax and slice header syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Lou Yu Yue Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-2251 yyu@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This document proposes some modifications on sequence parameter set RBSP syntax, picture parameter set RBSP syntax and slice header syntax

# Introduction

In the current HEVC working draft [1], the sequence parameter set RBSP syntax contains a flag max_num_ref_frames, which specifies the maximum number of short-term and long-term reference frames, complementary reference field pairs, and non-paired reference fields that may be used by the decoding process for inter prediction of any picture in the sequence. max_num_ref_frames also determines the size of the sliding window operation. The value of max_num_ref_frames shall be in the range of 0 to MaxDpbFrames, inclusive.

When max_num_ref_frames is 0, there is no reference frame stored. Therefore, the slice type could only be I slice. Some redundacies in the sequence parameter set RBSP syntax, picture parameter set RBSP syntax and the slice header syntax have been found.

# Modifications on sequence parameter set RBSP syntax, picture parameter set RBSP syntax and slice header syntax

The sequence parameter set RBPS syntax can be modified as shown in Table 1, the picture parameter set RBSP syntax as shown in Table 2, and the slice header syntax as shown in Table 3.

Table 1 Modified sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| pcm_enabled_flag | u(1) |
| if ( pcm_enabled_flag ) { |  |
| pcm_bit_depth_luma_minus1 | u(4) |
| pcm_bit_depth_chroma_minus1 | u(4) |
| } |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| max_num_ref_frames | ue(v) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| if ( pcm_enabled_flag ) { |  |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_coding_block_size | ue(v) |
| } |  |
| if( max_num_ref_frames ) { |  |
| max_transform_hierarchy_depth_inter | ue(v) |
| } |  |
| max_transform_hierarchy_depth_intra | ue(v) |
| chroma_pred_from_luma_enabled_flag | u(1) |
| loop_filter_across_slice_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| if ( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| if( max_num_ref_frames ) { |  |
| temporal_id_nesting_flag | u(1) |
| inter_4x4_enabled_flag | u(1) |
| } |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| if ( num_tile_columns_minus1 != 0 ¦ ¦ num_tile_rows_minus1 != 0 ) { |  |
| tile_boundary_independence_flag | u(1) |
| uniform_spacing_flag | u(1) |
| if ( !uniform_spacing_flag ) { |  |
| for ( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[i] | ue(v) |
| for ( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[i] | ue(v) |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 2 Modified picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_synchro | u(v) |
| cabac_istate_reset_flag | u(1) |
| if( entropy_coding_synchro ) |  |
| num_substreams_minus1 | ue(v) |
| if( max_num_ref_frames ) { |  |
| num_short_term_ref_pic_sets | ue(v) |
| for(idx = 0; idx < num_short_term_ref_pic_sets; idx++) |  |
| short_term_ref_pic_set( idx ) |  |
| long_term_ref_pics_present_flag | u(1) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| } |  |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| if( max_num_ref_frames ) { |  |
| constrained_intra_pred_flag | u(1) |
| } |  |
| slice_granularity | u(2) |
| max_cu_qp_delta_depth | ue(v) |
| if( max_num_ref_frames ) { |  |
| weighted_pred_flag | u(1) |
| weighted_bipred_idc | u(2) |
| } |  |
| tile_info_present_flag | u(1) |
| if( tile_info_present_flag = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| if( num_tile_columns_minus1 != 0 ¦ ¦ num_tile_rows_minus1 != 0 ) { |  |
| tile_boundary_independence_flag | u(1) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[i] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[i] | ue(v) |
| } |  |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 3 Modified slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| if( max_num_ref_frames ) { |  |
| slice_type | ue(v) |
| } |  |
| pic_parameter_set_id | ue(v) |
| if( sample_adaptive_offset_enabled_flag ¦¦ adaptive_loop_filter_enabled_flag ) |  |
| aps_id | ue(v) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| else { |  |
| pic_order_cnt_lsb | u(v) |
| if( max_num_ref_frames ) { |  |
| short_term_ref_pic_set_pps_flag | u(1) |
| if( !short_term_ref_pic_set_pps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| if( max_num_ref_frames ) { |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| } |  |
| } |  |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !entropy_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| inherit_dbl_params_from_APS_flag | u(1) |
| if ( !inherit_dbl_params_from_APS_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if ( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag && aps_adaptive_loop_filter_flag ) { |  |
| byte_align( ) |  |
| alf_cu_control_param( ) |  |
| byte_align( ) |  |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| 5_minus_max_num_merge_cand | ue(v) |
| for( i = 0; i < num_substreams_minus1 + 1; i++ ){ |  |
| substream_length_mode | u(2) |
| substream_length[i] | u(v) |
| } |  |
| } |  |

# References

[1] HEVC working draft (d7) JCTVC-G1103, January 25, 2012

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).