| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0655 |
| --- | --- |

| Title: | Non-CE9: Motion information derivation of deformable block |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang Xiaopeng Fan Siwei Ma Debin Zhao Harbin Institute of Technology Harbin, China | Email: | hitnzhang@hit.edu.cn fxp@hit.edu.cn swma@pku.edu.cn dbzhao@hit.edu.cn |
| Source: | Harbin Institute of Technology |  |  |

_____________________________

# Abstract

This contribution proposes an adjacent-block-based prediction model to improve the prediction performance of a deformable block. Based on this model, the motion information of each 4x4 block (the minimum partition (MP) in a CU) involved in a PU can be derived from the motion information of the adjacent MPs to the four corners of current PU. We integrate the adjacent-block-based prediction model into HM4.0 as an additional choice of its merge mode. Simulation results show that compared to HM4.0, our proposed scheme has good performance improvement (BD bit rate saving is up to 12.2%) for sequences which involve deformation motion and has no performance loss for the rest of sequences. Our proposal has almost the same time cost as HM4.0 anchor.

# Introduction

Currently both the new generation video coding standard HEVC and the existing video coding standards including H.264/MPEG-4 AVC employ block matching motion estimation (BMME). However, BMME assumes that all the pixels in a block undergo the same motion, resulting in poor prediction for blocks with non-translation motion such as rotation, zoom, and etc. In this proposal, we approximate the motion of each MP in a deformable block as translational motion, but we use the translational motion of MPs in a deformable block to approximate the deformation motion of the deformable block.

In HM 4.0, the merging candidate list is constructed of the elements as shown in Figure 1 which are given as specified order: A1, B1, B0, A0, B2, Col.

Figure 1. Candidate positions for merge mode in HM 4.0

From Figure 1 we can see that there is at least one candidate position (such as B2 in the above left corner) around each corner of the four corners of the current block. If we utilize the motion information around the four corners to derive the motion information of each MP in a deformable block, we can give a more accurate motion description for the deformable block.

# Proposed Method

To predict the deformable block more effectively, we integrate the adjacent-block-based prediction model into HM4.0 as an additional choice of its merge mode. We let the RDO to decide whether to adopt our proposal or not. If our proposed method is adopted, compared to original merge mode, we do not need to transmit merge index to decoder, but we need to send an additional flag to decoder to inform that encoder uses our proposed complex mode. Otherwise, we not only require to send merge index, but also require to transfer an additional flag to decoder.

## 2.1 Motion information derivation of deformable block

In the proposed method, we employ one control MP for each corner. For above right corner, we first check position B1, if it is unavailable or its prediction mode is intra, we then check B0. Otherwise, B1 is used as control MP for above right corner. Similarly, for left bottom corner, we first check position A1, if it is unavailable or its prediction mode is intra, we then check A0. Like original merge mode, we only can derive motion information of B2 when at least one of the A1, B1, B0, A0 isn't valid. When the motion information of adjacent MP around each corner can be derived and they are not identical in at least one reference list (list 0 or list 1), we propose to consider using our method. We will not use the list in which one of the four control MPs doesn't get its motion information for prediction. Then we can derive motion information of each MP involved in a PU from the motion information of the adjacent MPs to the four corners of current PU.

## 2.2 Derivation of interpolation kernel

Let the number of control MPs be fixed as 4, one around each corner. The MV of the kth control MP of Bm (block with index of m) is specified by dm(MPk) and the MV of the target MP is dm(MPx). Then the motion function over the block is described by

The interpolation kernelφm,k (x) depends on the desired contribution of the kth control MP of Bm to MPx. The interpolation function here we use is bilinear function. We set different interpolation kernels according to the selected control MPs. The way of deriving interpolation kernels for blocks of different sizes is similar. Formula (2) gives the interpolation kernels corresponding to case (d) of Figure 2.

Figure 2. Four different cases of the selected control MPs

In formula (2) and Figure 2, W denotes the width of the block, H denotes the height of the block. And dx represents the distance between the center of target MP and the center of left control MP (MP1 or MP3) in the horizontal direction, dy represents the distance between the center of target MP and the center of above control MP (MP1 or MP2) in the vertical direction. All the distances denote the actual value divided by 4. As shown in Figure 2, a 16x16 block, the value of W is 4.

## 2.3. Derivation of reference indices for target MP

After determining the weighting factor of each control MP, we can derive reference index refIdxLXMPx (with X being replaced by 0 or 1) of the target MP. Reference index of the control MPk (k=1,2,3,4) in list X (with X being replaced by 0 or 1) is refIdxLXMPk.

- If they have the same value, refIdxLXMP1 is assigned to refIdxLXMPx.

- Otherwise, the weights of control MPs whose reference indices are the same are added together.

  - If the number of reference indices with maximum weight is only one, refIdxLXMPx is equal to the corresponding reference index .

  - Otherwise, the minimum reference index with maximum weight is assigned to refIdxLXMPx.

## 2.4. Derivation of motion vector for target MP

Before using the motion vectors of control MPs to interpolate the displacement of target MP, if the MV of a control MP (such as MPi) pointing to a different reference picture, the MV is scaled to the target reference picture refIdxLXMPx as the final MV. The MV of MPi scaling process similar to derivation process for temporal luma motion vector prediction [1] is described as formula (3).

After the MV scaling process, we can utilize the motion vectors of control MPs to interpolate the displacement of any MP in the current block using formula (1) according to the interpolation kernels derived from 2.2 section. We provide the interpolated MV of every MP in a rotary16x16 block and the scaled MV of each control MP as an example in Figure 3. All motion vectors in Figure 3 are given in quarter-luma-sample units.

Figure 3. MV of each MP in a rotary 16x16 block and the scaled MV of each control MP

We can observe that our proposed method provides a more accurate motion description for a rotary block compared to conventional ME for which there is only a motion vector for a block. Our method has a similar role for scaling and other deformation movement. Therefore, applying our method in merge mode can improve its prediction accuracy, further increasing its probability of being selected. Accordingly, the bit rate will be reduced.

# Experimental Results:

Our solution is implemented in HM4.0 and simulated under the common test conditions [2]. Taking into account the effectiveness of implementation, we only apply the adjacent-block-based prediction model to the block using merge mode with CU size larger than 16x16 and the PU splitting mode is 2Nx2N.

In view of our program is mainly applied to solve the problem of predicting the deformation block, in Table 1, we will provide experimental results of the following sequences: Cactus (300 frames, involve local rotation), BlueSky (200 frames, camera rotation, many details), Station (100 frames, long zoom out, many details), Tractor (240 frames, from the local details to the whole scene), Shields(100 frames, significant zoom, many details), Vidyo3 (200 frames, involve head rotation and hand movement), Flowervase (416x240, 300 frames, from the whole scene to the local details); BasketballDrive (300 frames), BQMall (300 frames),  BasketballPass (300 frames) which don't involve significant complex motion.

Table 1 Coding performance comparison between the proposed scheme and HM 4.0

Based on the results in Table 1 we can see that our proposed method has significantly better performance for the first seven sequences which involve deformation motion, especially for Station, the proposed scheme provides 5.3%, 12.2%, 12.1% BD bit rate saving corresponding to RA HE, LB HE, LP HE. The remaining six sequences also have very good performance. Even for sequences which do not contain apparently complex motion such as the latter three sequences in Table 1, our proposal has no performance loss. Our proposal has almost the same time cost as HM4.0 anchor.

Table 2 taking Station for example presents proportion of considering and using our method in HM 4.0 anchor and the proposed method. In Table 2, PC denotes the proportion of the MPs using merge mode, 2Nx2N PU splitting mode and CU size larger than 16x16; PU represents the percentage of the MPs using the proposed method; PI denotes the increment of PC. The value of PI in Table 2 indicates that our proposal increases the utilization of merge mode, which makes our solution has the potential to reduce the bit sent to the decoder. Experimental results have proved this point. The average utilization of our proposal for Station is 33%. PU can achieve 49% for Station in RA HE when the value of QP is 27.

Table 1 Proportion of considering and using our method for Station in HM 4.0 anchor and

the proposed method

# Conclusion

This proposal presents a new adjacent-block-based prediction model for more accurately predicting the deformable block. Different from HM4.0 anchor, we simultaneously utilize these motion information  when deriving the motion information of merge candidates in merge mode. The results obtained by the proposed algorithm show that our proposal has significantly better coding performance than HM4.0 anchor for sequences with complex motion and has no performance loss for the rest of sequences with almost the same time cost, which verifies that our proposal is able to deal with deformable block more effectively. We recommend our proposed method to be adopted into HM.

# Reference

- “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803, JCT-VC 6th Meeting: Torino, July, 2011

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, JCT-VC 6th Meeting: Torino, July, 2011

# Patent rights declaration(s)

Harbin Institute of Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).