| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0657 |
| --- | --- |

| Title: | Non-CE5: A crosscheck report on complexity reduction for transform skipping mode (JCTVC-H0141) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan L. Joshi 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-4511 rajanj@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution provides a summary of a cross-check performed by Qualcomm of InterDigital’s proposal on complexity reduction for transform skip mode (JCTVC-H0141). The BD-rate results matched those reported by the proponents. The average luma BD-rate differences for RA-HE, LB-HE, RA-HE10, RA-LC, and LB-LC configurations were -0.4%, -0.9%, -0.1%, -0.5%, and -1.5% respectively with respect to the HM5.0 anchor. The proposed changes produce virtually identical BD-rate performance when compared with HM5.0_tsm under common test conditions. Slight loss (from 0.0% to 0.2%) is observed for class F sequences.

# Introduction

This contribution provides a summary of a cross-check performed by Qualcomm of InterDigital’s proposal on complexity reduction for transform skip mode (JCTVC-H0141). The code matched the description from the proponents. The vertical-skip and horizontal-skip mode transform bit-shifts were modified so that both the modes had the same bit-widths at various stages. A small reduction in the number of arithmetic operations and lookup tables is achieved for quantization and dequantization operations.

# Results

Results for RA-HE, LB-HE, RA-HE10, RA-LC, and LB-HC configurations were crosschecked. Table 1 shows a summary of the BD-rate results. More detailed coding results can be found in the Excel attachment. No YUV files were written when the decoding times were measured. Encoding as well as decoding was done on a LINUX cluster (64 bit) by specifying only one type of CPU. The decoding times are less reliable because of the variability in the cluster. The proposed changes should not produce any noticeable impact on the decoder run times.

Table 1: BD-rate results for the proposed method against HM5.0 anchor

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.1% | -0.2% | 0.0% |
| Class B | -0.2% | 0.0% | 0.1% | -0.7% | -0.1% | 0.2% | -0.2% | 0.0% | 0.0% |
| Class C | -0.4% | -0.2% | -0.1% | -0.9% | -0.2% | 0.1% |  |  |  |
| Class D | -0.8% | -0.1% | -0.3% | -1.4% | 0.5% | 0.3% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.4% | -0.1% | -0.1% | -0.9% | 0.0% | 0.2% | -0.1% | -0.1% | 0.0% |
|  | -0.4% | -0.1% | -0.1% | -0.9% | 0.0% | 0.1% | -0.1% | -0.1% | 0.0% |
| Class F | -1.9% | -0.9% | -0.9% | -2.7% | -1.5% | -1.6% |  |  |  |
| Enc Time[%] | 126% |  |  | 118% |  |  | 128% |  |  |
| Dec Time[%] | 92% |  |  | 94% |  |  | 94% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.2% | 0.5% | 0.5% | -1.0% | -0.8% | 0.1% |  |  |  |
| Class C | -0.6% | -0.4% | -0.1% | -1.5% | -0.1% | 0.3% |  |  |  |
| Class D | -1.1% | 0.5% | 0.7% | -2.1% | 1.8% | 1.8% |  |  |  |
| Class E | -0.1% | 0.0% | -0.3% | -1.4% | 0.5% | 0.6% |  |  |  |
| Overall | -0.5% | 0.2% | 0.2% | -1.5% | 0.3% | 0.7% |  |  |  |
|  | -0.5% | 0.2% | 0.2% | -1.5% | 0.2% | 0.6% |  |  |  |
| Class F | -4.4% | -2.9% | -3.0% | -6.0% | -4.4% | -3.9% |  |  |  |
| Enc Time[%] | 119% |  |  | 111% |  |  |  |  |  |
| Dec Time[%] | 92% |  |  | 93% |  |  |  |  |  |

# Conclusion

We have verified results reported in JCTVC-H0141. It is proposed to modify the transform bit-shifts for the vertical-skip and horizontal-skip modes so that both the modes have the same bit-widths at various stages. A small reduction in the number of arithmetic operations and lookup tables is achieved by the proposed changes for quantization and dequantization operations.

The average luma BD-rate differences for RA-HE, LB-HE, RA-HE10, RA-LC, and LB-LC configurations -0.4%, -0.9%, -0.1%, -0.5%, and -1.5% respectively, with respect to the HM5.0 anchor. The proposed changes produce virtually identical BD-rate performance when compared with HM5.0_tsm under common test conditions. Slight loss (from 0.0% to 0.2%) is observed for class F sequences.