| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0659 |
| --- | --- |

| Title: | Non-CE11: Crosscheck for Qualcomm’s context assignment for parallel coefficient level coding in JCTVC-H0550 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {peter.chuang, yuwen.huang}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck the parallel context processing for the syntax elements, coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag, in CABAC coefficient level coding proposed by Qualcomm in JCTVC-H0550, where the context modelling dependency between the current coeff_abs_level_greater1_flag and the number of trailing ones in the same subset is removed. Contexts for coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag can be assigned in parallel. The verification task was done successfully, and the results match exactly those provided by Qualcomm.

# Verification

Qualcomm provided the HM source codes of their proposed codec described in JCTVC-H0550 [1]. Two tests are evaluated. The verifications are carried out by setting the following parameters in the provided codes:

1st test: for coeff_abs_level_greater1_flag

#define SIG_LEVEL_GREATER_ONE     1 //turn on for H0550-test A&B
#define SIG_LEVEL_GREATER_TWO     0 //turn on for H0550-test B
#define ALF_FIX_TICKET_250           1 //Fix for the ALF bug affecting HM 5.0 low QP

2nd test: for coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag

#define SIG_LEVEL_GREATER_ONE     1 //turn on for H0550-test A&B
#define SIG_LEVEL_GREATER_TWO     1 //turn on for H0550-test B
#define ALF_FIX_TICKET_250           1 //Fix for the ALF bug affecting HM 5.0 low QP

Seven conditions (HE-AI, HE10-RA, HE-RA, HE-LDB, LC-AI, LC-RA, and LC-LDB) were simulated with normal QPs (22, 27, 32, 37) and low QPs (1, 5, 9, 13). The simulation includes encoding, decoding, and collecting run times of encoding and decoding processes.

# Results

The BD-rates from our experimental results match exactly those provided by Qualcomm. The encoding time and decoding time are the same to those provided by Qualcomm. The detailed results can be found in Excel files called JCTVC-H0659_Test_A_normal_QP.xls, JCTVC-H0659_Test_A_low_QP.xls, JCTVC-H0659_Test_B_normal_QP.xls, and JCTVC-H00659_Test_B_low_QP.xls.

# Conclusion

The results presented by Qualcomm in JCTVC-H0550 are confirmed.

# References

- W.-J. Chien, J. Chen, J. Sole, M. Karczewicz, “Non-CE11: Context assignment for parallel coefficient level coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-H0550, February 2012.