| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0660 |
| --- | --- |

| Title: | Non-CE1: Crosscheck for Qualcomm’s throughput improvement on CABAC coefficients level coding in JCTVC-H0554 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {peter.chuang, yuwen.huang}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck the throughput improvement of CABAC coefficients level coding proposed by Qualcomm in JCTVC-H0554, which reduces context coded bins for coefficients level coding. In JCTVC-H0554, it proposes to code coeff_abs_level_greater1_flag only for a few starting nonzero coefficients and only one coeff_abs_level_greater2_flag in a subset consisting of 16 coefficients, and an early switch to Golomb-Rice code which is coded via bypass bins. The verification task was done successfully, and the results match exactly those provided by Qualcomm.

# Verification

Qualcomm provided the HM source codes of their proposed codec described in JCTVC-H0554 [1]. Two schemes are evaluated. The verifications are carried out by setting the following parameters in the provided codes:

Scheme-1:

#define RESTRICT_C1C2FLAG_NUMBER    	1
#if RESTRICT_C1C2FLAG_NUMBER
#define BUG_FIX_1                   		1
#define BUG_FIX_2                   		1
#define C1FLAG_THRESHOLD_INTRA      	2
#define C1FLAG_THRESHOLD_INTER      	2
#define C2FLAG_NUMBER               	       1
#define RESTRICT_C1C2FLAG_NUMBER_RDOQ	1
#define GOLOMB_RICE_UPDATE          	1
#endif //RESTRICT_C1C2FLAG_NUMBER
#define ALF_FIX_TICKET_250          		1

Scheme-2:

#define RESTRICT_C1C2FLAG_NUMBER    	1
#if RESTRICT_C1C2FLAG_NUMBER
#define BUG_FIX_1                   		1
#define C1FLAG_THRESHOLD_INTRA      	8
#define C1FLAG_THRESHOLD_INTER      	8
#define C2FLAG_NUMBER               	       1
#define RESTRICT_C1C2FLAG_NUMBER_RDOQ	1
#define GOLOMB_RICE_UPDATE          	1
#endif //RESTRICT_C1C2FLAG_NUMBER
#define ALF_FIX_TICKET_250          		1

Seven conditions (HE-AI, HE10-RA, HE-RA, HE-LDB, LC-AI, LC-RA, and LC-LDB) were simulated with normal QPs (22, 27, 32, 37) and low QPs (1, 5, 9, 13). The simulation includes encoding, decoding, and collecting run times of encoding and decoding processes.

# Results

The BD-rates from our experimental results match exactly those provided by Qualcomm. The encoding time and decoding time are the same to those provided by Qualcomm. The detailed results can be found in Excel files named JCTVC-H0660_scheme_1_normal_QP.xls, JCTVC-H0660_scheme_1_low_QP.xls, JCTVC-H0660_scheme_2_normal_QP.xls, and JCTVC-H0660_scheme_2_low_QP.xls.

# Conclusion

The results presented by Qualcomm in JCTVC-H0554 are confirmed.

# References

- J. Chen, W.-J. Chien, R. Joshi, J. Sole, M. Karczewicz, “Non-CE1: Throughput improvement on CABAC coefficients level coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-H0554, February 2012.