| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0661 |
| --- | --- |

| Title: | Non-CE8: Crosscheck for Panasonic's line memory reduction fix at independent slice and tile boundaries in JCTVC-H0372 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {chingyeh.chen, yuwen.huang}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck the line memory reduction fix at independent slice and tile boundaries proposed by Panasonic in JCTVC-H0372, which disables the virtual boundary process in adaptive loop filter when the LCU boundary is an independent slice or tile boundary. Two tests cases, independent slices and independent tiles, are verified, and the experimental data are confirmed.

# Verification

Panasonic provided the HM source codes of their proposed codec described in JCTVC-H0372 [1]. The verification is carried out by setting the following parameters in the provided codes:

#define LINE_MEM_AT_SLICE_TILE_BOUNDARIES 1

#define NUM_SGU_IN_SLICE_H 1

Four conditions (HE-AI, HE-RA, HE-LDB, HE-RA-10bit) without class F were simulated in two testing cases, independent slices and independent tiles. The simulation includes encoding and decoding results.

# Results

The BD-rates from our experimental results match exactly those provided by Panasonic. The detailed results are listed in JCTVC-H0661_tile.xls and JCTVC-H0661_slice.xls.

# Conclusion

The results presented by Panasonic in JCTVC-H0372 are confirmed.

# References

- S. Esenlik, M. Narroschke, T. Wedi, “Non-CE8: Line memory reduction fix at independent slice and tile boundaries,” Document of Joint Collaborative Team on Video Coding, JCTVC-H0372, February 2012.