| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0663 |
| --- | --- |

| Title: | Non-CE11 : Cross check of Qualcomm’s proposal JCTVC-H0427 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jungsun Kim, Chulkeun Kim, Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Tel: Email: | +82-10-2201-9774 jungsun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document reports the verification results of the proposal JCTVC-H0427 on modification of significance map context [1] submitted by Qualcomm. The proponents provide their source codes with the results. The crosschecker checked source codes and provide cross check results to the proponents. It succeeded and produced as same result as those the proponent provided. The spreadsheet contains the detailed data is accompanied.

# Methods Proposed

In the contribution of the proponent, they propose to reduce the dependencies between coefficients to increase the degree of parallelism for significance map context. The contribution is composed of :

- Extending the holes for parallel context calculation

- Grouping coefficients for parallel context calculation

- Support modification for parallel context calculation

The cross checker checked method 3 briefly described below.

## Support modification for parallel context calculation

The proponents propose to change the neighbours whose significance is used to calculate significance map context increment. Fig 1 shows the neighbours used in HM5.0 and Fig 2 shows the neighbours used in the proposed method.

Fig. 1 (a) coefficients on blue positions use 4 yellow pixels as neighbours

(b) coefficients on green position use 5 yellow pixels as neighbours

Fig. 2  (a) coefficients on blue positions use 4 yellow pixels as neighbours for 3 bin parallelization

(b) coefficients on green position use 5 yellow pixels as neighbours for 3 bin parallelization

(c) coefficients on blue positions use 4 yellow pixels as neighbours for 4 bin parallelization

(d) coefficients on green position use 5 yellow pixels as neighbours for 4 bin parallelization

# Experimental Results

## Simulation environment

The encoding environment for cross-checking the method with 3 bin parallelization is as following:

OS: Window XP 64 bit

CPU: i7 875K 2.93Hz

Memory : : DDR3 16G

And the encoding environment for cross-checking the method with 4 bin parallelization is as following:

OS: Linux version 2.6.18-274.el5 (gcc version 4.1.2 20080704 (Red Hat 4.1.2-50))

CPU: Intel Xeon Quad Core E31230 (SANDY BRIDGE3.2GHz)

Memory : 12GB DDR3 UDIMM Memory

The decoding environment for cross-checking is as following:

OS: Win XP 64 bit

CPU: i7 875K 2.93Hz

Memory: DDR3 16G

## Simulation conditions and results

The simulation follows the test conditions of [2]. There was no mismatch between reconstructed sequences at the encoder side and decoded sequences at the decoder. The results are same as the proponent provided. The spreadsheet contains the detailed data of the simulation is accompanied.

### Simulation Results for the case of 3 bin parallelization

|  | All Intra HE |  |  | All Intra LC |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A (8bit) | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% |  |  |  |
| Class B | 0.1% | 0.3% | 0.3% | 0.1% | 0.2% | 0.3% |  |  |  |
| Class C | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class D | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class E | 0.2% | 0.4% | 0.4% | 0.1% | 0.5% | 0.4% |  |  |  |
| Overall | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% |  |  |  |
|  | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% |  |  |  |
| Class F | 0.1% | 0.2% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.2% | -0.1% | 0.0% | 0.2% | 0.1% | 0.2% | 0.3% | 0.8% |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.3% |
| Class C | 0.1% | -0.1% | 0.2% | 0.1% | 0.3% | 0.1% |  |  |  |
| Class D | 0.1% | -0.3% | 0.1% | 0.0% | 0.3% | 0.3% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | 0.1% | 0.1% | 0.3% | 0.1% | 0.2% | 0.2% | 0.5% |
|  | 0.1% | -0.1% | 0.1% | 0.1% | 0.3% | 0.1% | 0.2% | 0.1% | 0.4% |
| Class F | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.2% |  |  |  |
| Class C | 0.1% | 0.1% | 0.3% | 0.0% | 0.0% | -0.1% |  |  |  |
| Class D | 0.1% | -0.4% | 0.0% | 0.0% | 0.2% | -0.3% |  |  |  |
| Class E | 0.1% | 1.2% | -1.1% | 0.1% | -0.6% | 0.0% |  |  |  |
| Overall | 0.1% | 0.2% | -0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
|  | 0.1% | 0.2% | -0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| Class F | 0.0% | 0.3% | -0.3% | 0.2% | -0.6% | -0.4% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |  |  |  |

### Simulation Results for the case of 4 bin parallelization

|  | All Intra HE |  |  | All Intra LC |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A (8bit) | 0.1% | 0.3% | 0.4% | 0.1% | 0.3% | 0.3% |  |  |  |
| Class B | 0.2% | 0.5% | 0.5% | 0.1% | 0.4% | 0.5% |  |  |  |
| Class C | 0.1% | 0.2% | 0.3% | 0.1% | 0.2% | 0.2% |  |  |  |
| Class D | 0.1% | 0.3% | 0.2% | 0.1% | 0.2% | 0.1% |  |  |  |
| Class E | 0.5% | 0.9% | 0.9% | 0.2% | 1.0% | 0.9% |  |  |  |
| Overall | 0.2% | 0.4% | 0.4% | 0.1% | 0.4% | 0.4% |  |  |  |
|  | 0.2% | 0.4% | 0.4% | 0.1% | 0.4% | 0.4% |  |  |  |
| Class F | 0.1% | 0.1% | 0.1% | 0.1% | 0.3% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.2% | 0.1% | 0.1% | 0.3% | 0.1% | 0.2% | 0.5% | 0.5% |
| Class B | 0.2% | 0.3% | 0.3% | 0.2% | 0.3% | 0.3% | 0.3% | 0.3% | 0.4% |
| Class C | 0.1% | 0.2% | 0.3% | 0.1% | 0.2% | 0.2% |  |  |  |
| Class D | 0.2% | 0.1% | 0.2% | 0.0% | 0.5% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.2% | 0.2% | 0.2% | 0.1% | 0.3% | 0.1% | 0.3% | 0.4% | 0.4% |
|  | 0.2% | 0.2% | 0.3% | 0.1% | 0.3% | 0.1% | 0.3% | 0.4% | 0.5% |
| Class F | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.3% | 0.3% | 0.4% | 0.1% | -0.2% | 0.4% |  |  |  |
| Class C | 0.2% | 0.0% | 0.3% | 0.1% | 0.1% | 0.2% |  |  |  |
| Class D | 0.2% | -0.2% | 0.3% | 0.0% | 0.5% | 0.5% |  |  |  |
| Class E | 0.3% | 0.9% | -0.6% | 0.0% | -1.5% | -1.2% |  |  |  |
| Overall | 0.2% | 0.2% | 0.2% | 0.1% | -0.2% | 0.1% |  |  |  |
|  | 0.2% | 0.2% | 0.2% | 0.1% | -0.2% | 0.1% |  |  |  |
| Class F | 0.1% | 0.0% | 0.0% | 0.1% | -0.7% | -0.3% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

# References

- V. Seregin, J. Sole, M. Karczewicz, “Non-CE11: Support modification for parallel calculation of significant map contexts”, JCTVC-H0427, 8th JCT-VC Meeting, San Jose, USA, Feb. 2012

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-G1200, 7th JCT-VC Meeting, Geneva, CH, Nov. 2011