| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H666 |
| --- | --- |

| Title: | Interlaced Indicators |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg, Aaron Wells, Paul Haskell 4300 N. First Street San Jose, CA 95134 | Tel: Email: | +1 408 542 2500 chadfogg@gmail.com |
| Source: | Ambarella Inc and Harmonic Inc. |  |  |

_____________________________

# Abstract

A minimalist syntax and semantic set is proposed for indicating how fields and frames from progressive or interlaced source are mapped into HEVC coded frames with just 4 orthogonal state bits. This metadata is intended for post-decode processing stages, such as display and editing, which need to understand the source nature of video independently from how it is encoded. “X” represents “don’t care” bits in the table below.

# Background

MPEG-2 (ISO/IEC 13818-2 | ITU-T H.262) has indicators for separately expressing the interlace/progressive format for both coding and source aspects of a video sequence. The MPEG-2 picture_header() element frame_pred_frame_dct served as the coding indicator, while prog_frame separately informed the output and post-processing stages whether the content contained within the coded field or frame was actually from a (captured) interlace or progressive source formatted signal.

Since the current HEVC interlace proposals are only capable of conveying all field or all frame sequences at a time, a system deploying HEVC would not possess the ability to switch between field and frames on a picture basis (as per the PAFF element pict_struct in both MPEG-2 | H.262 and AVC | H.264). It is therefore necessary to signal whether each field or frame was part of an interlace or progressive signal.

# Proposal

The proposed group interlace indicator source_format is a 4-bit fixed length field containing the following 1-bit sub-elements: prog_src, frame, field_parity, and duplicate. The indicator source_format should be sent with each coded frame, such as in an SEI, but it is recommended for better robustness, that it be sent in both in the PPS and slice headers.

Sub-elements:

prog_src informs whether the coded frame is originally from an interlace or progressive source.

frame expresses whether the coded HEVC frame conveys a field’s worth of lines (half a frame), or an entire frame’s worth of lines.

When applicable (frame==0), field_parity indicates whether the conveyed field is top or bottom parity.

Finally, duplicate, indicates whether the field or frame conveyed within the coded frame is a “dummy frame” resulting from a repetition process, such as 3:2 pulldown.

|  | prog _src | frame | field _parity | duplicate |
| --- | --- | --- | --- | --- |
| Encoded picture is an interlaced-source, top field | 0 | 0 | 0 | 0 |
| Encoded picture is a duplicated interlaced-source, top field that may be discarded for display | 0 | 0 | 0 | 1 |
| Encoded picture is an interlaced-source, bottom field | 0 | 0 | 1 | 0 |
| Encoded picture is a duplicated interlaced-source, bottom field that may be discarded for display | 0 | 0 | 1 | 1 |
| Source is interlaced, but coded as a frame | 0 | 1 | X | X |
| Encoded picture is a top field extracted from a progressive frame | 1 | 0 | 0 | 0 |
| Encoded picture is a duplicate top field extracted from a progressive frame and may be discarded for display | 1 | 0 | 0 | 1 |
| Encoded picture is a top field extracted from a progressive frame | 1 | 0 | 1 | 0 |
| Encoded picture is a duplicate bottom field extracted from a progressive frame and may be discarded for display | 1 | 0 | 1 | 1 |
| Encoded picture is a progressive frame | 1 | 1 | X | 0 |
| Encoded picture is a duplicate progressive frame that may be discarded for display | 1 | 1 | X | 1 |

# Patent rights declaration(s)

Ambarella and Harmonic Inc. do not have any current or pending patent rights relating to the technology described in this contribution.