| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0667 |
| --- | --- |

| Title: | Implementation issues with frequent slices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal for Profiles & Levels discussion |  |  |
| Author(s) or Contact(s): | Chad Fogg, Aaron Wells | Email: | chadfogg@gmail.com awells@ambarella.com |
| Source: | Ambarella |  |  |

_____________________________

# Abstract

Rather than limit the frequency of HEVC slices, a potentially viable scheme would be to restrict subsets of slice header elements that are nearly constant in applications yet have heavy impact upon implementation when changed. An analysis is included to identify those subsets. Based on the results thus far, it is recommended that Profiles & Levels limit deblocking control to no more than 4 alterations per frame. Finally, it is suggested to move slice header element sets identified here as fairly static into a slice parent entities (APS, PPS..) that have suitable frequency restrictions.

# Proposal

Slice headers serve many different purposes represented by disparate sets of syntax that change with rates in proportion to the local adaptively of dependent tools. The analysis below groups elements into sets based on their likelihood of change and implementation effect. For example, slice_qp_delta can be highly active in constant bitrate control scenarios or from visual optimization. Deblocking coefficients control are nearly constant throughout a picture for most applications. slice_qp_delta impacts the isolated inverse quantizer stage, while deblocking filter controls will trigger stalls and pipeline flushing.

The change rates of slice header sets are classified here either dynamically changing (by semantic definition, or by content) or fairly static. Implementation impact values range from light to heavy.

| Set | Hardware impact | Tele-conf | Broad-cast |
| --- | --- | --- | --- |
| Modes (slice_type, Qp) | Light | Dynamic | Dynamic |
| reference picture management (short_term_ref_pic_set()) | Medium | Dynamic | Static |
| reference picture list modification | Medium | Dynamic | Static |
| Slice identifiers | Light | Dynamic | Dynamic |
| Parameter set identifiers | Light | Static+ | Static |
| Deblocking loop filter control | Heavy | Static | Static |
| ALF loop filter control | Medium | Dynamic | Static |
| Motion prediction weights | Light | Static | Static |
| Wavefront substream pointers | Light |  | Dynamic |

Element-by-element breakdown:

| Set | Change characteristics | Hardware impact |
| --- | --- | --- |
| Coding modes slice_type slice_qp_delta | Dynamic (commonly modulated by rate control) | Light |
| Reference picture management pic_order_cnt_lsb short_term_ref_pic_set_pps_flag short_term_ref_pic_set() short_term_ref_pic_set_idx long_term_ref_pics_present_flag num_long_term_pics delta_poc_lsb_lt_minus1[] used_by_curr_pic_lt_flag[] collocated_from_l0_flag | Dynamic (Used in teleconferencing) | Medium |
| Slice identifiers first_slice_in_pic_flag slice_address | Dynamic (Guaranteed to change. Needed for synchronization) | Light |
| Parameter set identifiers entropy_slice_flag pic_parameter_set_id aps_id idr_pic_id no_output_of_prior_pics_flag | Static (needed for synchronization, coding, etc.) | Light |
| Reference picture list modifications num_ref_idx_active_override_flag num_ref_idx_l0_active_minus1 num_ref_idx_l1_active_minus1 ref_pic_list_modification() ref_pic_list_combination() | Static (long-term reference pictures list are not frequently changed) | Medium |
| Deblocking Loop filter controls inherit_dbl_params_from_APS_flag disable_deblocking_filter_flag beta_offset_div2 tc_offset_div2 | Static (deblocking usually tuned for large picture areas) | Heavy |
| ALF Loop filter controls alf_cu_control_param() | Dynamic | Medium |
| Motion prediction weighting pred_weight_table() | Static (usually designed to be the same weights throughout the frame) | Light |
| Wavefront substream pointers substream_length_mode substream_length[] | Dynamic | Light |

# Patent rights declaration(s)

Ambarella does not have any current or pending patent rights relating to the technology described in this contribution.