| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H668 |
| --- | --- |

| Title: | Profile and Level considerations |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Aaron Wells, Chad Fogg 2975 San Ysidro Way Santa Clara, CA 95051-0604 | Tel: Email: | +1 408 734 8888 awells@ambarella.com |
| Source: | Ambarella Inc. |  |  |

_____________________________

# Abstract

Profile and Levels definition for HEVC should permit bitstreams that utilize a single tile and single slice per frame at coded frame sizes up to 4K (3840x2160 consumer, 4096x2304 capture). This extends the common practice of coding a single slice per frame in AVC at 1080p levels.  Additionally, restrictions to the frequency of regular (non-lightweight ) slices should be limited to no more than 4 per coded frame. Limits to the number of motion vectors per window of coded blocks should also be updated from AVC.

# Proposal

Updating the place-holder Table A-9 (Annex A) in HEVC Working Draft 5:  16*8 (128) is the ratio of the AVC MaxMvsPer2Mb carried over to the 64x64 max LCU size (16 * 64*64/(2*16*16)).

| Level number | Max macroblock processing rate MaxMBPS (MB/s) | Max picture size MaxFS (MBs) | Max decoded picture buffer size MaxDpbMbs (MBs) | Max video bit rate MaxBR (1000 bits/s, 1200 bits/s, cpbBrVclFactor bits/s, or cpbBrNalFactor bits/s) | Max CPB size MaxCPB (1000 bits, 1200 bits, cpbBrVclFactor bits, or cpbBrNalFactor bits) | Vertical MV component range MaxVmvR (luma picture samples) | Min compression ratio MinCR | Max number of motion vectors per 4096 coded pixels |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 485 | 99 | 396 | 64 | 175 | [−64,+63.75] | 2 | - |
| … | … | … | … | … | … | … | … | … |
| 4 | 245 760 | 8 192 | 32 768 | 20 000 | 25 000 | [−512,+511.75] | 4 | 16*8 |
| 4.1 | 245 760 | 8 192 | 32 768 | 50 000 | 62 500 | [−512,+511.75] | 2 | 16*8 |
| 4.2 | 522 240 | 8 704 | 34 816 | 50 000 | 62 500 | [−512,+511.75] | 2 | 16*8 |
| 5 | 589 824 | 22 080 | 110 400 | 135 000 | 135 000 | [−512,+511.75] | 2 | 16*8 |
| 5.1 | 983 040 | 36 864 | 184 320 | 240 000 | 240 000 | [−512,+511.75] | 2 | 16*8 |

# Patent rights declaration(s)

Ambarella Inc. does not have any current or pending patent rights relating to the technology described in this contribution.