INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11 MPEG2011/m23618

February 2012, San José, USA

| Source | Sharp Corporation |
| --- | --- |
| Status | Proposal |
| Title | Sharp’s proposals for HEVC scalability Extension |
| Author | Hisao Kumai, Tomoyuki Yamamoto, Andrew Segall, Norio Ito |

# Introduction

A requirement of an HEVC scalability extension, where the base layer (BL) is compliant with one the existing MPEG video codices including MPEG-2 video, is proposed in former meetings such as m22768[2], and JCTVC-F290. But, H.264/AVC is taken it into account as a BL in the “Draft Call for Proposals on the Scalable Video Coding Extensions of HEVC” [3] because of increasing the computational or algorithmic complexity of scalable functionality and amount of work to develop the new standard in case of MPEG-2 video as BL.

The reception of digital TV is well established in many countries, and the MPEG-2 video coding standards have been widely implemented in digital TV. On the other hand, digital TV standard cannot change often but mechanism to introduce new functionality such as supporting UHDTV service to current digital TV is expected. Possible target to introduce such function is a hybrid TV which can both use broadcast and network simultaneously. However current scope of the HEVC scalability extension can only use H.264/AVC as BL, therefore most of the current digital TV standard cannot use this extension.

The main purpose of this proposal would be to show a mechanism to use not only H.264/AVC but also MPEG-2 video coding standard as BL while minimizing an increase of computational or algorithmic complexity.

# Proposals Details

## Use cases for hybrid TV

Figure 1 shows one of the use cases of 2 layer scalability for hybrid TV. In this case, the original input video is 4K2K (4K) video. Our purpose is to provide 4K video services using existing broadcast HD video as the BL. The BL input picture is generated by down sampling the input picture (4K =>HD), any down sampling method may be used to create the BL picture. For coding the enhancement layer (EL) picture, BL reconstructed pictures and its motion information are scaled and referred. The BL bitstream is transmitted to decoder via broadcast network, such as terrestrial, cable and satellite without changing existing broadcast system. On the other hand, the EL bitstream is transmitted via internet. Decoding process of EL, the method for referred BL is the almost same encoding process. In that context, hybrid TV can increase the coding efficiency of network environment compared to IPTV transmitting single HEVC bitstream.

Figure 1: Use cases for hybrid TV

## H.264/AVC and MPEG-2 video base layer compatibility mixed codec modes for scalability extension

This contribution proposes a scalability extension to the HEVC design, and its design concept is similar to the proposal in JCTVC-F290 which introduces extended NAL unit type which can encapsulate both AVC, MPEG-2 video and so on. Usually scalability coding performs using BL’s various information. And it contains different categories; some such as residual signal is coding dependent, and other such as reconstructed pixel data (Intra) is not coding depend. Therefore, our proposal is to add a new coding mode such that the coding of the enhancement layer can use only the coding standard independent information.

Figure 2 shows kinds of parameters used in the inter-layer prediction for H.264/SVC, in case of 2 layer scalability.

Figure 2: kinds of parameters used in the inter-layer prediction

The right side parameters, which are indicated by blue arrow, depend on the base layer coding, such as residual signal or transform coefficients which coding sequences are different depending on the video codec. The left side, which are indicated by red arrow, are loose coupled to the base layer coding, and may contain coded sequences compliant with any video codec which utilize motion vector and inter/intra coding mode.

In term of the dependency on the inter-layer coding, we define the mode that uses the prediction parameters: the left side parameters in Figure 2 (Intra, Motion, Modes), as “light mode”. And we also name the mode that uses all prediction parameters: adding right side parameters in Figure 2 to the “light mode” parameters, as “full mode”.

# Experiments

In this section, experimental results of “light mode” are presented for confirming a feasibility of the “light mode”.

## Implementation

HM4.0 was modified for demonstrating feasibility of the “light mode”. Only the reconstructed pixel data was used when encoding and decoding the enhancement layer. The modifications is to support 2 layer (one BL: MPEG-2 video and one EL: modified HEVC) to be able to swap the reference picture which is indicated in L0 for the MPEG-2 video reconstructed and scaled picture, in which case the P slice of the enhancement layer is encoded in order to simulate inter-layer intra prediction. I/B slice prediction functions are left with no change.

## Experimental and evaluation Conditions

FFmpeg version 0.6.1 was used for the MPEG-2 video encoding and decoding, JSVM 9.19.13 down sampling filter for 2x and 1.5 factors were used to create the base layers. The base layers were MPEG-2 video encoded and decoded using the FFmpeg, and down-sampled and up-sampled using the JSVM Filter, with spatial scalable HEVC enhancement layers.

Full experimental results can be found in the Excel spreadsheets. The BD-bitrate calculation is used for comparisons with simulcast high resolution layer and the scalable enhancement layer. To simplify the assessment, we select this combination of the target for comparison in order to evaluate the coding efficiency using existing broadcast encoded video data as base layer compared to single layer coding by HEVC.

## 2x Spatial Scalability

In this experiments, we select a test condition to simulate the use case which is represented in Figure 1. The JSVM down sampling filter for 2x factors are used to create the base layers. The enhancement layer QP values match the single layer coding QP values, 22, 27, 32, 37. The QP value of the base layer is selected to achieve the target PSNR:35db. The selected QP values of the base layer per sequence can be found in the Excel spreadsheets. Only Class A through C sequences are used, and only the Random access(RA) test conditions with few modifications. Table 1 show a key parameters of 2x Spatial Scalability test conditions. In Table 1, “HEVC” represents simulcast high resolution layer conditions, and “HEVC (EL)” represents scalable enhancement layer conditions with MPEG-2 video base layer. I/P/B/b slice represents a number of each slice included in the GOP structure according to the intra period. The column parameters are referred to HM4.0 configuration (encoder_randamaccess.cfg). We assume that P-slices in “HEVC (EL)” simulate as inter-layer intra pictures for demonstrating feasibility.

Table 1. 2x test conditions

|  | 2x |  |
| --- | --- | --- |
|  | HEVC | HEVC(EL) |
| IntraPeriod | 8 | only first |
| DecodingRefreshType | CDR | CDR |
| GOPSize | 8 | 8 |
| I slice | 1 | only first |
| P slice | 0 | 1(inter-layer intra) |
| B slice | 3 | 3 |
| b slice | 4 | 4 |
| InternalBitDepth | 8 | 8 |

Comparisons are made with the simulcast high resolution layer and the scalable enhancement layer, excluding MPEG-2 video bitrates, in Table 2. We assume that this is a feasibility study for a use case for hybrid TV which we proposed at clause 2.1. Using the scalable enhancement layer provides average bitrates reductions in the range of 32% to 38% for the RA mode, for the Class A through C sequences.

Table 2. 2x Experimental Results

| High Resolution vs. Enhancement Layer |  |  |  |
| --- | --- | --- | --- |
|  | Random access(Intra Period 8) |  |  |
|  | Y BD-rate[%] | U BD-rate[%] | V BD-rate[%] |
| Class A | -37.9 | -27.3 | -32.9 |
| Class B | -37.5 | -26.1 | -30.2 |
| Class C | -32.7 | -21.3 | -22.1 |
| All | -36.0 | -24.9 | -28.4 |

## 1.5x Spatial Scalability

In this experiments, we select from among several test conditions which is discussed in mpeg-hevc-ext reflector. The JSVM down sampling filter for 1.5x factors are used to create the base layers. The enhancement layer QP values match the single layer coding QP values, 22, 27, 32, 37, and we use the specific value of the base layer QP value 10 below the enhancement layer’s QP. Only Class B 1920x1080 sequences are used, with 1280x720 base layer, and only the RA operating modes with few modifications. Table 3 show a key parameters of 1.5x Spatial Scalability test conditions, and only the IntraPeriod parameter of “HEVC “is differ from Table 1.

Table 3. 1.5x test conditions

|  | 1.5x |  |
| --- | --- | --- |
|  | HEVC | HEVC(EL) |
| IntraPeriod | 32 | only first |
| DecodingRefreshType | CDR | CDR |
| GOPSize | 8 | 8 |
| I slice | 1 | only first |
| P slice | 0 | 1(inter-layer intra) |
| B slice | 15 | 3 |
| b slice | 16 | 4 |
| InternalBitDepth | 8 | 8 |

Comparisons are made with the simulcast high resolution layer and the scalable enhancement layer, excluding MPEG-2 video bitrates, in Table 4. Using the scalable enhancement layer provides average bitrates reductions about 13% for the RA modes, for the Class B sequences.

Table 4. 1.5x Experimental Results

| High Resolution vs. Enhancement Layer |  |  |  |
| --- | --- | --- | --- |
|  |  | Random access |  |
|  | Y BD-rate[%] | U BD-rate[%] | V BD-rate[%] |
| Class B | -13.0 | -7.2 | -9.9 |

# Conclusion

In this contribution, we propose to introduce “light mode’ for HEVC extension of a standard scalability. The mode uses minimum information given by BL and expected to give less computational or algorithmic complexity. According to the experimental results, the “light mode” shows significant coding advantages vs. single layer coding.

In accordance with these results of the study, Sharp suggests to add the “light mode” to the “Draft Call for Proposals on the Scalable Video Coding Extensions of HEVC”.

# References

[1] D. Hong, W. Jang, J. Boyce, A. Abbas, “Scalability Support in HEVC,” JCTVC-F290, Torino, IT, July, 2011.

[2] K. McCann, C. Kim, B. Choi and J. Park, “Samsung’s comments on HEVC Extension”, m22768, Geneva, CH, November, 2011.

[3] A. Luthra, J. Ohm, J. Ostermann “Draft Call for Proposals on the Scalable Video Coding Extensions of HEVC”, N12398, Geneva, CH, November, 2011.

# Patent rights declaration(s)

Sharp corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).