| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0674 WG11 Number: m23616 |
| --- | --- |

| Title: | Non-CE8: Cross-check of JCTVC-H0409 Harmonization of number of ALF classes between RA and BA modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes | Tel: Email: | wlai@sta.samsung.com +1 972 761 7456 ffernandes@sta.samsung.com +1 972 761 7427 |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This report presents Samsung’s cross-check results for JCTVC-H0409 Harmonization of number of ALF classes between RA and BA modes. Rate-PSNR numbers match exactly to the proponent’s results. The proposed change in ALF BA class label mapping is performed using two-step label mapping. The cross-checker recognized it can be performed by a “one-step hybrid label mapping” and present the method.

# Cross-check results

The simulations were run in the 64-bit Linux cluster environment.

|  | AI-HE | RA-HE | RAHE10 | LB-HE | LP-HE |
| --- | --- | --- | --- | --- | --- |
| Y | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| U | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| V | 0.0% | 0.0% | 0.0% | 0.1% | -0.2% |
| Enc Time | 100% | 101% | 100% | 100% | 101% |
| Dec Time | 100% | 102% | 101% | 101% | 100% |

# Further study

## The two-step class label mapping process

The 16 BA class lable is being mapped from the features “direction” and “normalized variance” as below:

th[NO_VAR_BINS] = {0, 1, 2, 2, 3, 3, 3, 4, 4, 4, 4, 5, 5, 5, 5, 5};

avgVarTab[3][6] = {{0, 1, 2, 3, 4, 5},

{0, 6, 7, 8, 9,10},

{0,11,12,13,14,15}};

avgvar = th[avgvar];

avgvar = avgVarTab[direction][avgvar];

Thus it is a two-step mapping process with a memory of 16 + 3x6 = 34 scalar values.

## One-step hybrid label mapping

The cross-checker recognized the above mapping can be performed using one-step with a “hybrid label table” (“hybrid” means same label assigned on different rows as well as on different columns) as below:

avgVarTab[3][16] = {{0, 1, 2, 2, 3, 3, 3, 4, 4, 4, 4, 5, 5, 5, 5, 5},

{0, 6, 7, 7, 8, 8, 8, 9, 9, 9, 9,10,10,10,10,10},

{0,11,12,12,13,13,13,14,14,14,14,15,15,15,15,15}};

avgvar = avgVarTab[direction][avgvar];

Thus this one-step mapping process with a memory of 3x16 = 48 scalar values.

Corresponding syntax

- 8.6.3.3	Derivation process for filter index array for luma samples

- ...

- ...

- ...

- Otherwise (alf_region_flag is equal to 0), the following ordered steps apply.

- The variables varTempH[ x ][ y ], varTempV[ x ][ y ] and varTemp1[ x ][ y ] with x, y = 0, 2, 4,...(nS) is derived as

varTempH[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x-1, yC+y ] – s’’[ xC+x+1, yC+y ] |	(8-1)
	varTempV[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x, yC+y-1 ] – s’’[ xC+x, yC+y+1 ] |	(8-2)

- The variables varTempH1[ x, y ], varTempV1[ x, y ], and varTemp3[ x, y ] with x, y = 0..( (nS) – 1 )>>2 are derived as

varTempH1[ x ][ y ] = ij varTempH[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 1, 2	(8-3)
	varTempV1[ x ][ y ] = ij varTempV[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 1, 2	(8-4)
	varTemp3[ x ][ y ] = ( varTempH1[ x ][ y ] + varTempV1[ x ][ y ] ) >> 2	(8-5)

- The variable direction is derived as

- If varTempV1[ x >> 2 ][ y >> 2 ] is greater than varTempH1[ x >> 2 ][ y >> 2 ] << 1,

direction = 1

- Otherwise, if varTempH1[ x >> 2 ][ y >> 2 ] is greater than varTempV1[ x >> 2 ][ y >> 2 ] << 1,

direction = 2

- Otherwise,

direction = 0

- The variable avgvar is derived as

var	-
	avgVar = Clip3( 0, 15, (varTemp3[ x >> 2 ][ y >> 2 ] * 1024 ) >> (3 + BitDepthY) )	(8-7)

- The filter index fIdx[ x, y ] with x, y = 0..(nS)-1 is derived as

fIdx[ x ][ y ] = classTab[direction][avgVar]	(8-8)

Table 8-XX – classTab[direction][avgVar] to determine filter index fIdx

| avgVar | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| direction = 0 | 0 | 1 | 2 | 2 | 3 | 3 | 3 | 4 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | 5 |
| direction = 1 | 0 | 6 | 7 | 7 | 8 | 8 | 8 | 9 | 9 | 9 | 9 | 10 | 10 | 10 | 10 | 10 |
| direction = 2 | 0 | 11 | 12 | 12 | 13 | 13 | 13 | 14 | 14 | 14 | 14 | 15 | 15 | 15 | 15 | 15 |

# Patent rights declarations

Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).