| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H676 |
| --- | --- |

| Title: | Cross verification of crosscheck of AHG10(JCTVC-H0010) HM5 PME B |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jongbum Choi #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Email: | jb79.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports Cross-Verification results for parallel motion estimation (PME B) described in JCTVC-H0010[1]. PME B is one of the proposed architecture in JCTVC-H0010. The software is consistent with PME B algorithm and the test result matched the result from HM5 PME B of JCTVC-H0010

# Introduction and code-level check

In PME B, Encoder constructs partial MCL(merge/skip MVP list). It is for enable full parallel MCL derivation for all PUs/CUs inside a parallel ME region. The partial MCL consists of non-dependency candidate in a parallel ME region. Encoder derivates exact merge/skip list and merge_idx after ME.

In HM5 PME B of JCTVC-H0010, parallel merge level(log2ParallelMergeLevelMinus2) is configurable and is in PPS. It specifies a parallel ME region. The macro settings is as follows

#define HM_PARALLEL_MRG  1

#define LOG2_PARALLEL_MERGE_LEVEL_MINUS2  4  //< 0-> 4x4, 1-> 8x8, 2->16x16, 3->32x32, 4->64x64

#define FIXED_TMVP_REFIDX0        1

# Test Result

Tests were conducted on LOG2_PARALLEL_MERGE_LEVEL_MINUS2 = {1, 2, 3, 4}.

| PME 64x64 | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 5.9% | 6.5% | 6.4% | 6.7% | 7.3% | 7.3% |
| Class B | 5.8% | 5.5% | 5.3% | 6.8% | 6.5% | 6.2% |
| Class C | 5.6% | 6.3% | 6.8% | 6.4% | 7.3% | 7.7% |
| Class D | 5.6% | 6.2% | 6.4% | 6.3% | 7.4% | 7.4% |
| Overall | 5.7% | 6.0% | 6.1% | 6.5% | 7.1% | 7.1% |
| Class F | 4.3% | 4.8% | 4.8% | 4.8% | 5.4% | 5.4% |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 104% |  |  | 107% |  |  |

| PME 64x64 | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 6.5% | 4.8% | 4.6% | 9.1% | 7.2% | 7.2% |
| Class C | 6.1% | 5.8% | 6.3% | 7.6% | 7.7% | 7.9% |
| Class D | 5.9% | 5.0% | 5.9% | 7.0% | 7.4% | 7.4% |
| Class E | 7.2% | 3.9% | 5.2% | 10.7% | 7.9% | 9.2% |
| Overall | 6.4% | 4.9% | 5.4% | 8.5% | 7.5% | 7.8% |
| Class F | 5.5% | 5.5% | 5.4% | 6.3% | 6.3% | 6.1% |
| Enc Time[%] | 107% |  |  | 101% |  |  |
| Dec Time[%] | 105% |  |  | 105% |  |  |

| PME 32x32 | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 3.7% | 3.8% | 3.8% | 4.5% | 4.9% | 4.8% |
| Class B | 3.2% | 2.6% | 2.5% | 4.1% | 3.8% | 3.6% |
| Class C | 3.5% | 3.9% | 4.2% | 4.5% | 5.1% | 5.4% |
| Class D | 3.7% | 3.9% | 4.1% | 4.4% | 5.2% | 5.3% |
| Class E |  |  |  |  |  |  |
| Overall | 3.5% | 3.5% | 3.5% | 4.3% | 4.7% | 4.7% |
| Class F | 2.8% | 3.0% | 3.1% | 3.4% | 3.8% | 3.9% |
| Enc Time[%] | 109% |  |  | 108% |  |  |
| Dec Time[%] | 113% |  |  | 113% |  |  |

| PME 32x32 | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 3.8% | 2.3% | 2.5% | 5.4% | 3.9% | 3.9% |
| Class C | 4.1% | 3.9% | 4.1% | 5.5% | 5.6% | 5.9% |
| Class D | 4.2% | 3.4% | 4.2% | 5.1% | 5.6% | 5.5% |
| Class E | 3.8% | 1.8% | 2.6% | 6.0% | 4.6% | 5.2% |
| Overall | 4.0% | 2.9% | 3.3% | 5.5% | 4.9% | 5.0% |
| Class F | 3.6% | 3.6% | 3.1% | 4.6% | 4.0% | 4.7% |
| Enc Time[%] | 112% |  |  | 105% |  |  |
| Dec Time[%] | 114% |  |  | 107% |  |  |

| PME 16x16 | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 1.2% | 1.2% | 1.0% | 1.7% | 2.0% | 1.9% |
| Class B | 0.8% | 0.4% | 0.4% | 1.2% | 1.1% | 1.1% |
| Class C | 1.2% | 1.5% | 1.5% | 1.8% | 2.1% | 2.4% |
| Class D | 1.4% | 1.4% | 1.6% | 2.0% | 2.5% | 2.4% |
| Class E |  |  |  |  |  |  |
| Overall | 1.1% | 1.1% | 1.1% | 1.7% | 1.9% | 1.9% |
| Class F | 1.0% | 1.2% | 1.2% | 1.5% | 1.7% | 1.7% |
| Enc Time[%] | 104% |  |  | 109% |  |  |
| Dec Time[%] | 106% |  |  | 111% |  |  |

| PME 16x16 | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.0% | 0.4% | 1.0% | 1.6% | 1.0% | 1.3% |
| Class C | 1.6% | 1.3% | 1.8% | 2.4% | 2.6% | 2.8% |
| Class D | 1.8% | 1.8% | 1.9% | 2.6% | 2.9% | 3.3% |
| Class E | 1.0% | 0.9% | 0.5% | 1.9% | 1.8% | 2.0% |
| Overall | 1.3% | 1.1% | 1.4% | 2.1% | 2.0% | 2.3% |
| Class F | 1.3% | 1.3% | 1.1% | 2.0% | 0.7% | 1.9% |
| Enc Time[%] | 106% |  |  | 108% |  |  |
| Dec Time[%] | 107% |  |  | 110% |  |  |

| PME 8x8 | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 105% |  |  | 107% |  |  |

| PME 8x8 | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.6% | 0.8% | 0.0% | 0.3% | 0.5% |
| Class C | 0.1% | 0.1% | 0.2% | 0.0% | 0.3% | 0.2% |
| Class D | 0.2% | 0.5% | 1.1% | 0.1% | 1.0% | 0.8% |
| Class E | 0.2% | 0.6% | 0.4% | 0.1% | 0.4% | 0.7% |
| Overall | 0.1% | 0.4% | 0.7% | 0.1% | 0.5% | 0.6% |
| Class F | -0.2% | -0.1% | -0.5% | -0.1% | -0.7% | -0.4% |
| Enc Time[%] | 107% |  |  | 107% |  |  |
| Dec Time[%] | 110% |  |  | 109% |  |  |

# Conclusions

This contribution reports cross-verification of PME B algorithm in JCTVC-H0010. The simulation results matched the data from PME B of JCTVC-H0010.

# Reference

[1] JCT-VC, “JCT-VC AHG report: Parallel merge/skip (AHG 10)”, JCTVC-H0010, 8th JCT-VC Meeting, San José, USA, 1-10 Feb., 2012