| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0679 |
| --- | --- |

| Title: | AHG21: Cross-check of JCTVC-H0138 on Unification of reference picture list modification processes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Edouard FRANCOIS Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 2 99 87 68 83 edouard.francois@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

The purpose of this document is to report cross-checking of proposal JCTVC-H0138 related to the unification of the single- and combined-reference picture lists modification processes. It is reported that the software implementation matches with the proposal description. The results provided by the proponent are also reported to be confirmed.

The cross-checker reports that the proposal results in a simplification of the HM design, with no impact on the coding efficiency. It is suggested to consider the proposal for adoption in the HM.

- Code verification

Proposal H0138 aims at unifying the modification process of reference picture lists for single L0/L1 and combined list cases. The default list is generated by insert first the reference pictures from short-term list 0, then short-term list 1, then long-term list. In addition, instead of a modification mechanism consisting in signaling using two syntax elements (the position in the reference list array, and the corresponding reference index), the modification mechanism consists in explicitly and successively signaling the reference indexes for all elements of the list. The same process applies whatever the type of list (list0, list1, combined list).

The source code has been analyzed. It has been verified that the HM has been modified as described in JCTVC-H0138. The proposed modifications are enabled by the macros #define REF_PIC_MOD and #define LIST_0_1_REORDER_ENABLE set to 1.

The source code was compiled successfully using windows32, VC2008 and using Linux 2.6.18-194.el5,  gcc version 4.1.2 20080704 (Red Hat 4.1.2-48) 64 bits.

# Results

Tests were done according to the tests description provided in [1]. Tested configurations were RA-HE, RA-LC and RA-HE10.

The xls file is attached. The results perfectly match those generated by Inter Digital.

# Conclusion

The results presented by Inter Digital in JCTVC-H0138 are confirmed. It is confirmed that the proposed solution does not have any impact on coding efficiency, while unifying the modification process of reference picture lists and therefore simplifying the related HM design. The conformity between the document description and the software modifications is also confirmed.

The cross-checker recommends considering this proposal for adoption into the HM.

# References

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1200, 7th Meeting: Geneva, Switzerland, Nov, 2011.