| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0693 |
| --- | --- |

| Title: | A real-time ARM HEVC decoder implementation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Karthic Veera Rajdeep Ganguly Bo Zhou Nagendra Kamath Surendranath Chowdary Junchen Du In-Suk Chong Muhammed Coban 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-3937 ichong@qualcomm.com mcoban@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This documents describes a real-time software HEVC player running on a mobile ARM platform. HEVC decoder is compliant to HM4.0 HE configuration. The software is developed on Qualcomm’s Android multimedia player platform. The player can perform WVGA(832x480) 30 fps playback.

# Introduction

The software decoder contains all main tools in HM4.0 except for CAVLC as shown in Table 1.

| Tool | HM 4.0 Decoder | Real time decoder |
| --- | --- | --- |
| SAO |  |  |
| ALF |  |  |
| NSQT/AMP |  | X |
| CAVLC |  | X |
| 10-bit internal bit-depth |  |  |

Table 1: List of tools supported by the real time decoder.

- Profiling

- Profiling was conducted on target device for RA_HE and LB_HE configuration for WVGA RaceHorses clip coded with constant QP=32. Motion compensation, transform, SAO and ALF modules use NEON acceleration.

- Playback configuration

Qualcomm mobile device:

- Processor: 	Dual core 1.5GHz ARM (Snapdragon) processor.

- OS: 		Android

- Content:	WVGA 30fps content coded with fixed QP=32.

- Configuration:	Single core decoding with player/rendering running on second core.

# Conclusion

Qualcomm implemented a real-time HM4.0 HE configuration compliant decoder running on a Snapdragon (ARM) 1.5GHz based mobile platform. The profiling results are inline with previously reported implementations of HM4.0 compliant decoders.

- References

- Frank Bossen, “On software complexity”, JCTVC-G757, JCT-VC 7rd Meeting, Geneva, November, 2011.

- Ken McCann, et. al, “HEVC software player demonstration on mobile devices, JCTVC-G988,” JCT-VC 7th Meeting Geneva, November 2011.