| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0694 |
| --- | --- |

| Title: | Signaling of long-term pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring, Heiko Schwarz, Thomas Wiegand Einsteinufer 37 10587 Berlin Germany | Email: | karsten.suehring@hhi.fraunhofer.de heiko.schwarz@hhi.fraunhofer.de thomas.wiegand@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes a modified long-term picture coding using bit strings to identify long-term pictures in the decoded picture buffer. Although a small overhead is introduced for long-term pictures that are near to the current decoded picture, approx. 45% of the necessary bits can be saved in scenarios where the long-term picture has a large picture order count difference.

# Introduction

With the adopted AHG21 reference picture set (RPS) coding method ([2], [3]) each coded pictures contains a list of the pictures marked “used for reference” in the decoded picture buffer (DPB). The signaling distinguishes between “short-term” references which can be signaled very efficiently using templates in the picture parameter set and “long-term” references which need to be signaled in each slice header. The adopted long-term signaling uses the picture order count (POC) difference between the current picture and the first long term picture and the difference between the long-term pictures for the following long-term pictures. With increasing distance between the current picture and the first long-term picture or increasing distance between the long-term pictures, the coded POC differences increase and thus, also the number of bits required for signalling the long-term pictures increases.

# Bit string based signaling

The idea behind the RPS signaling is to provide a safe identification of the pictures used for reference by the current picture. With absolute (lsb/msb) signaling, POC can be used to safely identify pictures, even when losses occur.

The size of the DPB is restricted and thus only a limited number of pictures can be stored in it at any point in time. This means only a small subset of the full POC range is actually used by pictures residing in the DPB. Depending on the other POC values, this makes it possible to safely identify the picture with a much smaller identification value. In certain situations even a single bit can be sufficient to safely identify a picture (see Figure 1). In other situations a longer bit string will be necessary, but most likely it will be shorter than the whole POC range (see Figure 2).

| POC |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 |  | 0 | 0 | 1 | 0 | 0 | 0 | 0 |  | long-term |
| 32 |  | 0 | 1 | 0 | 0 | 0 | 0 | 0 |  |  |
| 36 |  | 0 | 1 | 0 | 0 | 1 | 0 | 0 |  |  |
| 40 |  | 0 | 1 | 0 | 1 | 0 | 0 | 0 |  |  |
| 44 |  | 0 | 1 | 0 | 1 | 1 | 0 | 0 |  |  |

Figure 1: Example POC values of pictures in the DPB. First line contains the picture to be identified. The red value in the first line is sufficient for identification (and should be coded).

| POC |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 |  | 0 | 0 | 1 | 0 | 0 | 0 | 0 |  | long-term |
| 18 |  | 0 | 0 | 1 | 0 | 0 | 1 | 0 |  |  |
| 20 |  | 0 | 0 | 1 | 0 | 1 | 0 | 0 |  |  |
| 22 |  | 0 | 0 | 1 | 0 | 1 | 1 | 0 |  |  |
| 24 |  | 0 | 0 | 1 | 1 | 0 | 0 | 0 |  |  |
| 28 |  | 0 | 0 | 1 | 1 | 1 | 0 | 0 |  |  |

Figure 2:  Example POC values of pictures in the DPB. First line contains the picture to be identified. The red values in the first line are sufficient for identification (and should be coded).

The signaled bit string only needs to distinguish the current long-term picture from other long-term reference pictures in the DPB and from pictures to be marked “unused for reference”.

It is not necessary to take pictures into account, which are contained in an RPS. These pictures already have been safely identified as short-term reference pictures.

# Syntax

To signal a bit string it is necessary to specify position, length and value.

Following is the changed section of the slice header:

| if( long_term_ref_pics_present_flag ) { |  |
| --- | --- |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| lt_bit_pos[ i ] | u(4) |
| lt_bit_len[ i ] | un(v) |
| if (lt_bit_len>1) { |  |
| lt_bit_value[ i ] | u(v) |
| } |  |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |

# Semantics

lt_bit_pos specifies the position of the lowest significant bit of the picture identification bit string of the associated long term reference picture.

lt_bit_len specifies length of the identification bit string of the associated long term reference picture.

If lt_bit_len equal to zero, lt_bit_value is inferred to be equal to zero and the variable ltBitLen is set equal to one.

Otherwise, if lt_bit_len equal to one, lt_bit_value is inferred to be equal to one and the variable ltBitLen is set equal to one.

Otherwise, the variable ltBitLen is set equal to lt_bit_len.

lt_bit_value specifies the value of the identification bit string of the associated long term reference picture. The syntax element is coded using lt_bit_len bits.

# Parsing of un(v)

un(v) describes a unary code as follows:

This process is specified as follows:

leadingZeroBits = −1
for( b = 0; !b; leadingZeroBits++ ) 
	b = read_bits( 1 )

The variable codeNum is then assigned as follows:

codeNum = leadingZeroBits

where the value returned from read_bits( leadingZeroBits ) is interpreted as a binary representation of an unsigned integer with most significant bit written first.

Example bit strings with assignment to codeNum (informative)

| Bit string form | Range of codeNum |
| --- | --- |
| 1 | 0 |
| 0 1 | 1 |
| 0 0 1 | 2 |
| 0 0 0 1 | 3 |
| 0 0 0 0 1 | 4 |
| … | … |

# Decoding Process (for each reference picture)

The variable bitMask is generated using the following pseudo-code:

bitMask = 0

for (i=0; i< ltBitLen; i++){

bitMask = (bitMask<<1)  | 1

}

For each picture picX in the decoded picture buffer that is not marked “used for short-term reference” in the decoding process of the current picture, the following steps are performed

- the variable maskedPoc is derived as
maskedPoc = (PictureOrderCnt(picX) >> lt_bit_pos) & bitMask

- if maskedPoc is equal to lt_bit_value, the picture picX is marked “used for long-term reference”

# Temporal scalability and error resilience

Without temporal scalability or picture loss, the decoded picture buffers at encoder and decoder are always synchronized. This makes it easy to find a unique identification string for each long-term picture.

If temporal scalability is used and long-term pictures are marked “unused for reference”, the encoder needs to take into account, that a long-term picture may not have been removed in layers with higher temporal_id. This makes it necessary to take the picture marked “unused for reference” into account for creating the identifying bit string at least until the next picture of the temporal layer with the lowest temporal_id been encoded.

For error resilience the encoder can decide to consider additional pictures for the identification. For instance if the decoder can signal the reception status of a picture with a certain round trip time, the encoder could consider all pictures marked “unused for reference” in that interval.

Another way of adding additional information for error resilience would be to use additional bits or to select a different section of the bit string over time.

# Results

The following results were obtained by interpreting the Common conditions for reference picture marking and list construction proposals [4]. Problems with unclear description have been discussed on the JCT-VC email reflector. Only the long-term picture relevant clauses 2.6 and 3.3 have been tested. There should be no impact on short-term coding.

Only the bits for the actual long-term syntax in the slice header have been counted for the following reports.

For section 2.6 two whole scene sequences were coded, i.e. 10-30-10-30 and 20-60-20-60 seconds. To keep the test material simple, only a single low-resolution sequence was looped. This was executed for only 24 fps as the lowest and 60 fps as the highest frame rate.

| 60 fps |  |  |  | 24 fps |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 10/30 with 4800 frames | 20/60 with 9600 frames |  |  | 10/30 with 1920 frames | 20/60 with 3840 frames |
| HM 5.1 | 71827 | 144169 |  | HM 5.1 | 28661 | 57377 |
| proposal | 38255 | 10523 |  | proposal | 15215 | 30575 |
|  | -46,74% | -46,83% |  |  | -46,91% | -46,71% |

For simplifying section 3.3, 600 frames were encoded using only one slice per picture. This allowed coding of a very low-resolution sequence with only counting the necessary high level bits. The relative results should be similar when the slice header syntax is repeated once for each slice.

| 60 fps |  |  |  | 24 fps |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 200 ms RTT 600 frames | 1000 ms RTT 600 frames |  |  | 200 ms RTT 240 frames | 1000 ms RTT 240 frames |
| HM 5.1 | 10523 | 14375 |  | HM 5.1 | 3349 | 4787 |
| proposal | 9558 | 9231 |  | proposal | 3974 | 3629 |
|  | -9,17% | -35,78% |  |  | 18,66% | -24,19% |

# References

[1] 	JCTVC-G1103, WD5: Working Draft 5 of High-Efficiency Video Coding, Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan, Thomas Wiegand

[2]	JCTVC-G1002, JCT-VC break-out report: Reference picture buffering and list construction (AHG21), Jill Boyce, Rickard Sjöberg, Ye-Kui Wang

[3]	JCTVC-G788, Proposal on Decoded Picture Buffer Description Syntax Relating to AHG21 and JCTVC-F493, Gary Sullivan,Yongjun Wu, Jizheng Xu, Bin Li

[4]	JCTVC-G1036, Common conditions for reference picture marking and list construction proposals, Ye-Kui Wang, Miska M. Hannuksela, TK Tan, Rickard Sjöberg

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).