| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0699 |
| --- | --- |

| Title: | Non-CE11: Cross-check of proposal H0228 on transform coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vivienne Sze | Email: | sze@ti.com |
| Source: | Texas Instruments |  |  |

_____________________________

# Abstract

This contribution reports cross-check results from Texas Instruments for H0228.  The software was analyzed and no mismatch was found compared to the BD-rate results provided by HHI.

# Summary of Proposal Based on Analysis of Source Code

All TU sizes, luma/chroma use same context selection for significance map and coefficient levels (context depends on value of 5 neighbors).

# Experiment Results

The HM-5.0 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations

Table 1: Common Conditions

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.7% | -1.0% | -0.9% | -1.0% | -1.2% | -1.2% |  |  |  |
| Class B | -0.6% | -0.6% | -0.7% | -0.9% | -1.0% | -1.0% |  |  |  |
| Class C | -0.7% | -0.4% | -0.5% | -1.1% | -0.9% | -0.9% |  |  |  |
| Class D | -0.9% | -0.5% | -0.6% | -1.3% | -1.0% | -1.1% |  |  |  |
| Class E | -0.9% | -1.0% | -1.1% | -1.2% | -1.5% | -1.5% |  |  |  |
| Overall | -0.7% | -0.7% | -0.7% | -1.1% | -1.1% | -1.1% |  |  |  |
|  | -0.7% | -0.7% | -0.7% | -1.1% | -1.1% | -1.1% |  |  |  |
| Class F | -1.1% | -1.1% | -1.0% | -1.5% | -1.1% | -1.4% |  |  |  |
| Enc Time[%] | 96% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 94% |  |  | 96% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.3% | -0.3% | -0.4% | -0.6% | -0.4% | -0.8% | -0.1% | 0.7% | 1.0% |
| Class B | -0.3% | -0.2% | -0.1% | -0.5% | -0.4% | -0.5% | -0.2% | -0.3% | -0.3% |
| Class C | -0.6% | -0.1% | -0.1% | -0.8% | -0.4% | -0.5% |  |  |  |
| Class D | -0.7% | -0.3% | -0.2% | -1.0% | -0.1% | -0.6% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.5% | -0.2% | -0.2% | -0.7% | -0.3% | -0.6% | -0.2% | 0.2% | 0.3% |
|  | -0.5% | -0.2% | -0.1% | -0.7% | -0.3% | -0.6% | 0.5% | 0.4% | 0.4% |
| Class F | -1.0% | -0.8% | -0.8% | -1.3% | -1.1% | -1.3% |  |  |  |
| Enc Time[%] | 98% |  |  | 100% |  |  | 103% |  |  |
| Dec Time[%] | 96% |  |  | 102% |  |  | 104% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.4% | 0.5% | -0.3% | -0.6% | -0.3% |  |  |  |
| Class C | -0.3% | 0.4% | 0.4% | -0.7% | -0.3% | 0.0% |  |  |  |
| Class D | -0.4% | -0.1% | 0.2% | -0.7% | 0.7% | 0.1% |  |  |  |
| Class E | 0.0% | 0.7% | -1.4% | -0.1% | 0.0% | 0.1% |  |  |  |
| Overall | -0.2% | 0.3% | 0.0% | -0.5% | -0.1% | -0.1% |  |  |  |
|  | -0.2% | 0.3% | 0.0% | -0.5% | -0.2% | -0.1% |  |  |  |
| Class F | -0.9% | -0.6% | -0.8% | -1.1% | -2.0% | -1.1% |  |  |  |
| Enc Time[%] | 102% |  |  | 97% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 92% |  |  |  |  |  |

Note: Average coding efficiency of proposal H0228 relative to HM-5.0 anchor. Positive numbers in the table means coding loss. Due to variation in the simulation environment, the runtime may be heavily inaccurate and should not be used to infer the complexity of the proposed method.