| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0702 |
| --- | --- |

| Title: | Cross-verification for RIM's (JCTVC-H0411) by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Elena Alshina, Samsung #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | Elena Alshina +82-10-3026-1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Co. Ltd.. |  |  |

_____________________________

# Abstract

Test results for so-called variant B provided by RIM in JCTVC-H0411 are verified here. JCTVC-H0411-B leads to 0.2% Luma and 0.3% Chroma DB-rate drop but reduces memory access for significant coefficient fag context derivation.

# Introduction

After analyzing the code we can conclude that implementation matches with description in JCTVC-H0411 (variant B). Number of conditions to check in proposed modification remains the same as in HM5.0.

# Simulation results

Proponent provided s/w before in time enough for analysis and testing. The summary of our verification tests is presented in Table 1.

Table 1. Summary of verified test (JCTVC-H0411).

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0,4% | 0,3% | 0,4% | 0,4% | 0,8% | 0,9% |  |  |  |
| Class B | 0,4% | 0,5% | 0,5% | 0,3% | 0,9% | 1,0% |  |  |  |
| Class C | 0,2% | 0,2% | 0,2% | 0,1% | 0,4% | 0,5% |  |  |  |
| Class D | 0,1% | 0,3% | 0,1% | 0,1% | 0,3% | 0,2% |  |  |  |
| Class E | 0,6% | 0,7% | 0,6% | 0,5% | 1,4% | 1,4% |  |  |  |
| Overall | 0,3% | 0,4% | 0,4% | 0,3% | 0,7% | 0,8% |  |  |  |
| Class F | 0,1% | 0,1% | 0,2% | 0,1% | 0,4% | 0,1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| Class A (8bit) | 0,2% | 0,3% | 0,1% | 0,2% | 0,5% | 0,3% | 0,4% | 0,5% | 0,4% |
| Class B | 0,3% | 0,4% | 0,4% | 0,2% | 0,6% | 0,6% | 0,3% | 0,3% | 0,3% |
| Class C | 0,1% | 0,1% | 0,2% | 0,1% | 0,1% | 0,2% |  |  |  |
| Class D | 0,1% | 0,0% | 0,1% | 0,0% | 0,4% | -0,1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0,2% | 0,2% | 0,2% | 0,1% | 0,4% | 0,3% | 0,4% | 0,4% | 0,4% |
| Class F | 0,1% | 0,1% | 0,0% | 0,0% | 0,0% | 0,1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0,2% | 0,0% | 0,1% | 0,2% | 0,1% | 0,6% |  |  |  |
| Class C | 0,1% | 0,0% | 0,2% | 0,1% | 0,1% | 0,1% |  |  |  |
| Class D | 0,1% | 0,0% | -0,1% | 0,0% | 0,5% | 0,2% |  |  |  |
| Class E | 0,2% | -0,2% | -1,5% | 0,0% | -1,2% | 0,8% |  |  |  |
| Overall | 0,1% | 0,0% | -0,2% | 0,1% | 0,0% | 0,4% |  |  |  |
| Class F | 0,0% | -0,3% | -0,6% | 0,2% | 0,0% | 0,1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |  |  |  |

In average Luma BD-rate drop is 0.2% and 0.3% in average for Chroma. Variations for encoding and decoding time is under noise level.

More drop is observed for higher resolution because proposed in JCTVC-H0411 change is applied to TUs large than 8x8. No memory access outside of 4x4 coefficients sub-block is needed during significant coefficient fag context derivation.

Results are matched with provided by proponent.

.

# Conclusions

Samsung confirms performance reported by RIM in JCTVC-H0411.