| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0710r1 |
| --- | --- |

| Title: | BoG Report on Quantization Matrices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki, Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Email: | teruhikos@jp.sony.com Kazushi.Sato@jp.sony.com |
| Source: | CE4 subtest2 participants |  |  |

_____________________________

# Abstract

This document reports result of discussion on Quantization Matrices subjective viewing and complexity analysis.

# Introduction

As result of discussion on CE4 report it was agreed that subjective quality comparison be conducted to see

- Difference between flat matrices and lossless matrices

- Difference between lossless and lossy matrices

and then after the viewing session complexity of each proposal be assessed.

# Subjective Viewing

## Assignment of Bitstream Generation

|  | Bitstream generator |
| --- | --- |
| Lossless coding | Qualcomm |
| Lossy coding (2.1) | Sony |
| Lossy coding (2.2) | Media Tek |
| Lossy coding (2.4) | HiSilicon |
| Flat matrix | (HM-5.0) |

## Procedure for Bitstream Generation

Each proponent will generate bitstream with adjusting QP so that the bit-rate be closer to HM-5.0 (flat matrix), which is the anchor of common test conditions, by less than 5% from the bitrate with HM-5.0 with QP=32.

The following table shall be filled.

| Sequence | Class | Coding config | Cond | QP | Bitrate | Percentage Difference (%) |
| --- | --- | --- | --- | --- | --- | --- |
| Kimono | B | IntraLC | HM-5.0 (flat) | 32 | 7455.85 | 0.0 % |
|  |  |  | 2.1 |  |  |  |
|  |  |  | 2.2 |  |  |  |
|  |  |  | 2.3 |  |  |  |
|  |  |  | HM-5.1 (lossless) |  |  |  |
| BQMall | C | IntraLC | HM-5.0 (flat) | 32 | 8820.24 |  |
|  |  |  | 2.1 |  |  |  |
|  |  |  | 2.2 |  |  |  |
|  |  |  | 2.3 |  |  |  |
|  |  |  | HM-5.1 (lossless) |  |  |  |
|  |  | RALC | HM-5.0 (flat) | 32 | 857.07 |  |
|  |  |  | 2.1 |  |  |  |
|  |  |  | 2.2 |  |  |  |
|  |  |  | 2.3 |  |  |  |
|  |  |  | HM-5.1 (lossless) |  |  |  |

## Matrices to be Tested

3 Matrices are tested:

- Symmetry 1

- Symmetry 7

- Asymmetry 7

“Symmetry 1” has been distributed as part of CE4 description.

“Symmetry 7” and “Asymmetry 7” will be generated and provided by Rajan Joshi@Qualcomm. On these matrices property of 2Nx2N Transform for Luma and NxN Transform for Chroma is similar.

## Hardware and Software Settings

Hardware for subjective testing:

| System | Dell Studio XPS Desktop 435T/9000 |
| --- | --- |
| CPU | Intel Core i7 975 @3.33GHz |
| RAM | 12.0 GB |
| OS | Windows 7 Ultimate 64 bit |
| Video Disk | OCZ Tech REVODRIVE 3 X2 SERIES SSD 240GB (Blackmagic Design Video Disk Speed Read Test > 600 MB/s) (CrystalDiskMark Sequential read speed > 600 MB/s) |
| Graphic Card | EVGA GEFORCE GTX590 3GB GDDR5 |
| Monitor | Sony professional calibrated 2D monitor ( at least 42 in) Final monitor model is to be determined later. |

Software for subjective testing:

| Video Player | David Flynn’s video player will be used. |
| --- | --- |
| Scripts for displaying test videos | David Flynn’s scripts |

## Test Method

BASIC TEST CELL

The Basic Test Cell (BTC) will be done of:

- The message “ Reference” on the screen for 1 second (white letter on black background or, better, black letters on mid gray background);

- Play the uncompressed reference video clip;

- The message “A” on the screen for 1 second (white letter on black background or, better, black letters on mid gray background);

- Play the Anchor video clip for a given QP and coding condition;

- The message “B” on the screen for 1 second (white letter on black background or, better, black letters on mid gray background);

- Play the Test video clip for the same QP and coding condition;

- The message “VOTE” on the screen for 5 second (white letter on black background or, better, black letters on mid gray background);

IMPORTANT RULES:

1) Change in a random way (and hidden way to the viewing experts) the order of presentation of the Anchor and of the Test video clips at any BTC.

[E.g. “A” Anchor / “B” Test; “A” Test / “B” Anchor; “A” Test / “B” Anchor; “A” Anchor / “B” Test; …]

2) Run the above “basic test cell” (BTC) for all the test conditions and for a given video source.

3) Repeat all the test conditions for all the video sources.

4) Viewing experts must ignore the order of presentation of the “A” and “B” cases

5) Select three BTCs of a session (with very low, mid and very good quality) and show them also at the beginning of the viewing session (results of these BTCs will be discarded).

VIEWING CONDITIONS

This “expert viewing” protocol requires the following viewing conditions to be satisfied:

- Quite office with low ambient light (no light directly on the screen, no external windows light))

- two / three expert seated at the same desk;

- 32” or bigger HD display;

- Viewing distance 2H (H is the height of the active part of the screen)

- Paper sheet vote collection

- Voting scale from 0 [worst quality] to 10 [best quality]

- One voting sale for each BTC

## Total Time for Viewing Session

3 (training) + (3(sequences) x 4 (test mode) x 3 (test matrices) x 1 (QP) ) x [ 1s(Reference message) + 10s(uncompressed video) + 1s(A message) + 10s(anchor sequence) +  1s(B message) + 10s(test video) + 5s(VOTE message)

= (3 + 1 x 24) x 38s = 1539s ~ 26 min

# Complexity Assessment

After viewing session, each proponent will provide documents on complexity. Sample is shown in Appendix A of this document but each proponent can freely describe.

# Timeline

T1: Proponent should provide bitstream and decoder binary (for windows) to the coordinator by 10:00a.m, Feb 05, 2012

T2: Preparation of Viewing: Feb 05, 2012

T3: Viewing: Feb 06, 2012

T4: Complexity Assessment: Feb 07 2012

Appendix: Sample of Complexity Assessment

# Introduction

This document describes a complexity analysis of QM coding in CE4.2..

# Tools in the proposal

Prediction

Scan

Residual coding

Entropy coding

Post process

|  | Prediction | Scan | Residual coding | Entropy coding | Post processing |
| --- | --- | --- | --- | --- | --- |
| Proposal 2.1 |  |  |  |  |  |
| Proposal 2.2 |  |  |  |  |  |

# Memory requirements

| Matrix size | Num of values | PU Type | Component | Memory Size (byte) |
| --- | --- | --- | --- | --- |
| 4x4 | 16 | Intra/Inter | Y/Cb/Cr | 96 |
| 8x8 | 64 | Intra/Inter | Y | 128 |
| Total |  |  |  | 224 |

Table 1: Memory size of AVC

| Matrix size | Num of values | PU Type | Component | Memory Size (byte) |
| --- | --- | --- | --- | --- |
| 4x4 | 16 | Intra/Inter | Y/Cb/Cr | 96 |
| 8x8 | 64 | Intra/Inter | Y/Cb/Cr | 384 |
| 16x16 | 256 | Intra/Inter | Y/Cb/Cr | 1536 |
| 32x32 | 1024 | Intra/Inter | Y | 2048 |
| Total |  |  |  | 4064 (181.4％) |

Table 2: Memory size of HEVC

| Matrix size | Num of values | PU Type | Component | Memory Size (byte) |
| --- | --- | --- | --- | --- |
| 4x4 | 16 | Intra/Inter | Y/Cb/Cr | 96 |
| 8x8 | 64 | Intra/Inter | Y/Cb/Cr | 384 |
| 16x16 | 256 | Intra/Inter | Y/Cb/Cr | 0 |
| 32x32 | 1024 | Intra/Inter | Y | 0 |
| Total |  |  |  | 480 |

Table 3: Memory size of option 1 (2.1)

| Matrix size | Num of values | PU Type | Component | Memory Size (byte) |
| --- | --- | --- | --- | --- |
| 4x4 | 16 | Intra/Inter | Y/Cb/Cr | 96 |
| 8x8 | 64 | Intra/Inter | Y/Cb/Cr | 384 |
| 16x16 | 256 | Intra/Inter | Y/Cb/Cr | 384 |
| 32x32 | 1024 | Intra/Inter | Y | 128 |
| Total |  |  |  | 992 |

Table 4: Memory size of option 2b

# Computational complexity

Encoder

Decoder