| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0711 M23919 |
| --- | --- |

| Title: | Cross-verification of Non-CE8.a.2: SAO with LCU-based syntax (JCTVC-H0067) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim Minhua Zhou 12500 TI Blvd MS 8649 Dallas TX USA 75243 | Tel: Email: | 1-214-480-1562 wskim@ti.com zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution provides cross-verification results of Cisco Systems’ proposal in JCTVC-H0067 [1], which proposes LCU based SAO processing. In JCTVC-H0067, SAO parameters are signaled for each LCU as part of slice data. To efficiently code SAO parameters, it is proposed to share the parameter between luma and chroma, and only edge offset is used with fixed size of 1 or -1 without signaling offset value. Only “non-recursive SAO processing” option is cross-checked, where non-filtered samples of the neighboring LCUs are used for SAO processing of the current LCU. The experimental results perfectly match with the one provided by Cisco Systems in terms of rate-distortion performance, where there are 0.4% and 2.6% losses for Luma and Chroma, respectively, on average of all the high efficiency test configurations.

# Introduction

This contribution provides cross-verification results of Cisco Systems’ proposal in JCTVC-H0067 [1], which proposes LCU based SAO processing. In JCTVC-H0067, SAO parameters are signaled for each LCU as part of slice data. To efficiently code SAO parameters, it is proposed to share the parameter between luma and chroma, and only edge offset is used with fixed size of 1 or -1 without signaling offset value. Only “non-recursive SAO processing” option is cross-checked, where non-filtered samples of the neighboring LCUs are used for SAO processing of the current LCU.

# Test Settings and Conditions

The simulations of this document have used HM-5.0 software. The simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines. The common test conditions and reference configurations specified in [2] are followed. Simulation is carried out for high efficiency configurations only.

# Experimental results

Table 1 shows the summary of BD-rate results of JCTVC-H0067 compared to HM-5.0. There are 0.4% and 2.6% losses for Luma and Chroma, respectively, on average of all the high efficiency test configurations.

Table 1 Test-1: BD-rate results of the proposal method compared to HM-5.0.

|  | All Intra HE |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.2% | 0.2% | 0.3% |  |  |  |
| Class B | -0.1% | 1.2% | 1.9% |  |  |  |
| Class C | -0.3% | 1.1% | 1.7% |  |  |  |
| Class D | 0.0% | 0.1% | 0.3% |  |  |  |
| Class E | 0.0% | 1.6% | 1.3% |  |  |  |
| Overall | -0.1% | 0.9% | 1.2% |  |  |  |
|  | -0.1% | 0.9% | 1.3% |  |  |  |
| Class F | 1.1% | 2.5% | 4.0% |  |  |  |
| Enc Time[%] | 106% |  |  |  |  |  |
| Dec Time[%] | 108% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.4% | 0.3% | 0.4% | 1.1% | 6.5% | 8.0% |
| Class B | 0.4% | 2.2% | 2.6% | 1.1% | 2.5% | 3.0% |
| Class C | 0.0% | 1.6% | 2.2% |  |  |  |
| Class D | 0.4% | 0.8% | 0.8% |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall | 0.3% | 1.4% | 1.7% | 1.1% | 4.2% | 5.2% |
|  | 0.3% | 1.4% | 1.8% | 1.1% | 4.2% | 5.2% |
| Class F | 2.1% | 3.5% | 6.3% |  |  |  |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  |
|  | Low delay B HE |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.0% | 4.8% | 6.0% |  |  |  |
| Class C | 0.3% | 3.6% | 6.3% |  |  |  |
| Class D | 0.3% | 2.3% | 3.3% |  |  |  |
| Class E | 1.7% | 4.1% | 5.6% |  |  |  |
| Overall | 0.8% | 3.7% | 5.3% |  |  |  |
|  | 0.8% | 3.8% | 5.3% |  |  |  |
| Class F | 2.9% | 7.0% | 10.8% |  |  |  |
| Enc Time[%] | 91% |  |  |  |  |  |
| Dec Time[%] | 89% |  |  |  |  |  |
|  | Low delay P HE |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.9% | 4.5% | 5.2% |  |  |  |
| Class C | 0.3% | 3.5% | 5.3% |  |  |  |
| Class D | 0.4% | 2.5% | 2.5% |  |  |  |
| Class E | 1.5% | 3.9% | 4.4% |  |  |  |
| Overall | 0.7% | 3.6% | 4.4% |  |  |  |
|  | 0.7% | 3.7% | 4.5% |  |  |  |
| Class F | 3.1% | 6.7% | 10.4% |  |  |  |
| Enc Time[%] | 102% |  |  |  |  |  |
| Dec Time[%] | 111% |  |  |  |  |  |

# Conclusion

The cross-checker’s results completely match with those of proponents.

# Reference

- A. Fuldseth and G. Bjøntegaard “Non-CE8.a.2: SAO with LCU-based syntax,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H0067, Feb. 2012.

- F. Bossen, “Common test conditions and software reference configurations,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-G1200, Nov. 2011.