| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0713_r1 WG11 Number: m23941 |
| --- | --- |

| Title: | BoG report on subjective viewing test for CE10 deblocking filter proposals |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrey Norkin Kenneth Andersson Matthias Narroschke Xun Guo Geert van der Auwera | Email: | andrey.norkin@ericsson.com kenneth.r.andersson@ericsson.com matthias.narroschke@eu.panasonic.com xun.guo@mediatek.com geertv@qualcomm.com |
| Source: | BoG on subjective viewing |  |  |

_____________________________

This contribution is a report on informal subjective viewing for the deblocking filtering that was held during the San Jose meeting on February 2, 2012. The subjective viewing was conducted according to the mandate of Core Experiment 10 on deblocking filtering JCTVC-G1210. The goal of informal subjective viewing test was to determine how the deblocking filter proposals affect the subjective quality. In total, 15 proposals from CE10 were evaluated in the subjective test.

# Test procedure

Fifteen proposals were tested. The proposals in Subtests 10.1-10.3 were based on HM5.0 and the proposals in Subtest 10.4 were based on the software distributed in CE10 by Toshiba on January 9, 2012. In this software, signaling of deblocking filter parameters from G1035 was implemented. Therefore, two tests were performed. Test 1 included the proposals from Subtests 1-3 and HM5.0. Test 2 included proposals from Subtest 4 and CE10.4 anchor.

The proposals in each test session were evaluated by a group of 5-6 experts. In total, four viewing sessions have been held. The identities of the proposals and the anchor were hidden (the test subjects were shown labels from A to M instead).

Each session consisted of two tests, Test 1 and Test 2.

The following six sequences have been used in Test 1.

Sequence 1: BQMall, random-access high-efficiency, QP 37 
Sequence 2: BQMall, low-delay low-complexity, QP 32
Sequence 3: Vidyo3, low-delay high-efficiency, QP 37 
Sequence 4: Vidyo3, low-delay low-complexity, QP 32
Sequence 5: Kimono, random-access high-efficiency, QP 37 
Sequence 6: Kimono, low-delay low-complexity, QP 32

The sequences were encoded according to common test conditions G1200 [3].

Test 2 (for proposals from CE10.4) included additional four high-QP sequences. The following additional sequences have been used in Test 2:

Sequence 7: Cactus, low-delay high-efficiency, QP37
Sequence 8: ParkScene, all-intra high-efficiency QP37
Sequence 9: RaceHorses_832x480, low-delay high-efficiency, QP37
Sequence 10: BasketballDrill, random-access low-complexity, QP 37

Test subjects were given score sheets and asked to grade each proposal with a grade from 1 to 10. The test procedure was as follows. First, the reference (a compressed sequence with similar quality to the proposals) was shown. The proponents were told that the reference has a score of 5.  Then, the proposals were shown in a randomized order one after another with a five-second break for voting after each test sequence. The order of the sequences was randomized for each sequence, each test, and each viewing session.

The length of one session was about 38 minutes. In total, 22 test subjects participated in scoring. The test sessions were conducted by Kenneth Andersson, Andrey Norkin and Xun Guo, and processing of score sheets was done by Geert van der Auwera, Matthias Narroschke, Kenneth Andersson, and Andrey Norkin.

# Test equipment

The following equipment provided by Huawei was been used in the test.

| TV Monitor | 42" Sony LMD-4251ED monitors |
| --- | --- |
| System | Dell Studio XPS 9100 |
| CPU | Intel Core i7 @3.47GHz |
| RAM | DDR3 1333 18GB |
| OS | Windows 7 Ultimate 64 bit |
| Video Disk | 2TB HDD |
| Graphic Card | AMD Radeon HD 6700 video card |

BBC’s video player provided by David Flynn has been used to conduct the test.

# Proposals

The following CE10 proposals have been tested in subjective Test 1.

- [JCTVC-H0310, MediaTek] CE10 Subtest1: Chroma Deblocking Filter

- [JCTVC-H0311, MediaTek] CE10 Subtest1: Simplified Chroma Deblocking Filter in JCTVC-H0310

- [JCTVC-H0147, Panasonic] CE10.1: Symmetric rounding in the deblocking filter

- [JCTVC-H0084, TI] CE10 Subtest 1: Test results on unification of luma and chroma filtering

- [JCTVC-H0280, MediaTek, Sony] CE10.2.1: Reducing one pixel line buffer by modified deblocking filter for horizontal LCU boundaries

- [JCTVC-H0472, Ericsson] CE10.2: Level restrictions on tiles for line buffer reduction

- [JCTVC-H0146, Panasonic] CE10.3: Simplified delta calculation in the deblocking filter

- [JCTVC-H0108, LG] CE10 Subtest3: Simplified BS calculation process in deblocking filter

- [JCTVC-H0473, Ericsson] CE10.3: Deblocking filter simplifications: Bs computation and strong filtering decision

- [JCTVC-H0396, Qualcomm] CE10 Subtest 3: Deblocking Filter Simplifications

- [JCTVC-H0155, SKT/SKKU] CE10.3: Simplification of First Delta Calculation in Deblocking by SKT/SKKU

- [JCTVC-H0156, SKT/SKKU] CE10.3: Simplification of Second Delta Calculation in Deblocking by SKT/SKKU

The following CE10 proposals have been tested in subjective Test 2.

- [JCTVC-H0148, Panasonic] CE10.4: Deblocking filter using adaptive weighting factors

- [JCTVC-H0574, Ericsson] CE10.4: deblocking parameters signaling

- [JCTVC-H0398, Qualcomm] CE10 Subtest 4: Transform Dependent Deblocking Filter Parameter Adjustment

# Test results

The graphs below show the test results. MOS values with overlapping CI values cannot be rated as distinguishable.

## Test 1

HM	     174    229       287       383     409     466    574obj   574subj   590

Test 2.  Subtest 4

# Conclusions

The confidence intervals of the proposals overlap with each other and the anchors. Therefore, the test did not show significant differences between the tested proposals (and the anchors).

# References

[1]	A. Norkin, X. Guo, B. Jeon, M. Narroschke, “Description of CE10: deblocking filtering,” JCTVC-G1210, Geneva, November 2011.

[2]	A. Norkin, X. Guo, B. Jeon, M. Narroschke, “CE10: Summary report of Core Experiment on deblocking filtering,” JCTVC-H0040, San Jose, February 2012.

[3]	F. Bossen, “Common test conditions and software reference configurations,” JCTVC-G1200, Geneva, November 2011.